# Technical Specifications

Community Equipment

This technical specification document provides detailed descriptions, dimensions, capacities, materials and safety features for the full range of community equipment items used in health and social care settings to support mobility, transfers and daily living.

# Technical Specifications

Community Equipment

This technical specification document provides detailed descriptions, dimensions, capacities, materials and safety features for the full range of community equipment items used in health and social care settings to support mobility, transfers and daily living.

# Contents

Introduction and purpose of this document	2

Structure and organisation of the document	2

Relationship to pricing and contracts	2

Use of Close Technical Equivalents (CTEs)	3

Accuracy, currency, and website links	3

Final note to bidders	3

- Bathing	4

- Toileting and commodes	24

- Beds, mattresses and bed accessories	33

- Seating accessories	61

- Cushions and pressure relief	70

- Hoists, slings and manual handling	78

- Paediatric	106

- Respiratory therapy	132

- Mobility	142

- Retail	153

# Introduction and purpose of this document

This document has been produced to support bidders responding to this Invitation to Tender (ITT) for the provision of Community Equipment and associated services. It sets out Lancashire County Council’s preferred catalogue of equipment, specifying the equipment that is currently in use by the Councils and the ICB, alongside the minimum technical, safety and performance requirements expected.

The primary purpose of this catalogue is to help bidders understand:

- the scope and breadth of equipment currently required across adult and paediatric services;

- the clinical and functional intent behind each category of equipment;

- the level of specification, quality, and safety standards expected by the Councils and the ICB; and

- how the catalogue aligns to the pricing schedules and contractual mechanisms within the ITT.

This catalogue should be read alongside the ITT instructions, service specification, and pricing documentation.

# Structure and organisation of the document

The catalogue is structured by functional equipment groupings, such as beds, hoists, seating and mobility aids. Within each section:

- products are ordered by unique product code to support clarity and auditability;

- each item includes a technical specification describing key dimensions, capacities, materials, controls, and safety features; and

- web links are provided to supplier or manufacturer information to support transparency and ease of reference

The Council has taken care to rationalise and structure the catalogue to minimise duplication and ambiguity, while still reflecting the reality of community equipment provision across a diverse population and range of needs.

# Relationship to pricing and contracts

The equipment listed within this catalogue directly relates to the catalogue prices and mechanisms set out in the Costing Schedule. Bidders should assume that:

- the listed items represent the Council’s preferred or benchmark products, forming the reference point for evaluation and pricing;

- pricing schedules are expected to align with the functional equivalent of the specifications provided here; and

- bidders must be able to supply equipment that meets or exceeds these specifications for safety, durability, and clinical use.

The inclusion of a product within this catalogue does not mandate that a bidder must supply that exact make or model.

# Use of Technical Equivalents (TEs)

Bidders may propose alternative products to those listed, provided that:

- the alternative is clinically, functionally, and technically equivalent;

- it meets all relevant UK and EU safety standards, including CE or UKCA marking where applicable;

- it performs within a tolerance of ±5% of the key dimensional, capacity, and performance criteria set out in the specification, unless otherwise agreed; and

- it is suitable for use within community, residential, and home environments as appropriate

Where alternative products are proposed, bidders should clearly identify and provide a website link to the TE within their submission. All proposed alternatives may be subject to clarification or clinical review as part of the procurement process.

# Accuracy, currency, and website links

At the time of publication:

- all product descriptions and technical details reflect the best available information held by the Councils and the ICB;

- all included web links were live and accessible; and

- specifications were aligned to current usage and commissioning practice.

- information contained within this document is indicative and may be updated following clinical review

The partners recognises that product ranges may change over time. The Provider will be expected to manage product lifecycle changes in line with contract requirements, ensuring continuity of supply and safe transition where substitutions are required.

# Final note to bidders

This catalogue is intended as a practical, transparent reference tool, supporting a consistent understanding of expectations and reducing ambiguity during the tender process. It should not be read in isolation, nor as a constraint on innovation or safe service development.

# Bathing

## BAT216 – Bath Lift – Neptune

Features
Electronic bath lift with reclining backrest, hand control, and washable covers. Designed for safe and comfortable bathing.

Technical specification

- Height: 7-43cm

- Width: 35-73cm

- Depth: 73cm

- Seat Height: 7-43cm

- Seat Depth: 34cm

- Seat Width: 35cm

- Backrest Height: 67cm

- Backrest Width: 35cm

- Batteries Included?	Yes

- Battery Cell Type: Ni-Mh

- Number of Batteries: 1	1

- Product Weight: 9kg (20lb)

- User Weight Capacity: 160kg (25st)

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering function: Yes

- Alerts: Battery indicator

- Noise level: Low

- Cleaning instructions: Wipe clean with mild detergent

Website link: https://drivemedical.co.uk/community/neptune-bathlift

## BAT231 – Easibathe Half-Height Portable Carer Screen 1.4m wide

Features

Portable carer screens are used to provide a temporary visual barrier to support privacy during care interventions and clinical procedures. The screens are supplied in a 4-panel configuration and are designed for portable use.

Technical specification

- Configuration: 4-panel

- Standard height: 750 mm

- Dimensions: 1400 mm

Website link: EB - 4 Panel Portable Carer Screen 1400mm

## BAT240 – Orca Reclining Back Bathlift

Features

The Aquatec Orca is a powered bath lift designed to assist safe transfer into and out of the bath. It provides a stable seated surface with a low minimum seat height to facilitate immersion. The backrest reclines to support positioning, and side flaps enable lateral transfer. The unit separates into two sections for handling and storage.

The seat surface is textured to allow use without additional covers. A hand control with integrated battery operates lift and recline functions.

Technical specification

- Technical specification

- Seat width: 375 mm (without side flaps); 715 mm (with side flaps)

- Seat depth: 590 mm

- Seat height range: 60–420 mm

- Backrest width: 360 mm

- Backrest height: 665 mm

- Backrest recline: up to 40°

- Base area: 580 × 295 mm

- Total depth: 650 mm (upright) – 920 mm (reclined)

- Total height: 1075 mm

- Maximum user weight: 140 kg (XL: 170 kg)

- Product weight: 10.8 kg (excluding hand control)

- Cover thickness: 3 mm

Website link: Aquatec Orca NG Bathlift - Invacare UK

## BAT240S – Lap strap for Orca Bath Lifts

Features
Safety lap strap designed for use with Orca bath lifts to secure the user during bathing.

Technical specification

- Dimensions: Adjustable length

- Product weight: < 0.5 kg

- User weight capacity: Matches bath lift (up to 140 kg)

- Safe working load: As per bath lift

- Control: Manual buckle

- Mobility: Static

- Materials: Nylon, plastic buckle

- Warranty: 1 year

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Machine washable

Website link: https://www.invacare.co.uk/independent-living-equipment/bathing/aquatec-orca-ng-bathlift

## BAT241 – Orca Fixed Back Bath Lift

Features
Fixed back bath lift designed for safe transfers into the bath. Lightweight and easy to install.

Technical specification

- Dimensions: Seat width approx. 375 mm

- Product weight: Approx. 10 kg

- User weight capacity: 140 kg

- Reach parameters:

  - Vertical reach: Approx. 6–48 cm

- Safe working load: 140 kg

- Movement parameters:

  - Lift speed: Approx. 40 mm/sec

  - Rotation capability: Fixed

  - Tilt or recline angles: Fixed back

  - Transfer path: Vertical lift

  - Motorised

- Control: Handheld remote

- Mobility: Static

- Operation time: Approx. 6 lifts per charge

- Materials: Plastic, aluminium

- Warranty: 1 year

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering function: Yes

- Alerts: Battery indicator

- Noise level: Low

- Cleaning instructions: Wipe clean

Website link: https://www.invacare.co.uk/independent-living-equipment/bathing/aquatec-orca-ng-bathlift

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## BAT245A – Etac Swift Mobile Tilt in Space Shower Chair with Pan Holder

Features
Tilt-in-space shower chair with pan holder, adjustable seat height, and ergonomic design.

Technical specification

- Dimensions: Seat height 475 - 625

- Product weight: Approx. 20 kg

- User weight capacity: 160 kg

- Reach parameters:

  - Vertical reach: Tilt range 0º t/m +35º / -5º t/m +30º

- Safe working load: 160 kg

- Movement parameters:

  - Lift speed: Manual tilt

  - Rotation capability: Swivel castors

  - Tilt or recline angles: -5° to +35°

  - Transfer path: Straight-line push

  - Manual

- Control: Manual tilt lever

- Mobility: 4 castors with brakes

- Materials: Aluminium, polyurethane

- Warranty: 2 years

Safety features

- Safety standards and compliance: CE marked

- Noise level: Low

- Cleaning instructions: Wipe clean or disinfect

Website link: https://www.etac.com/products/bathroom-aids/mobile-shower-commodes/etac-swift-mobil-tilt-2/#variants-anchor

## BAT246 – ETAC Swift Soft Comfort Seat 6”

Features
Height-adjustable modular shower chair seat with soft comfort padding. Designed for use with Etac Swift chairs.

Technical specification

- Seat, opening width 15 cm (6”)

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or disinfect

Website link: https://www.etac.com/en-gb/uk/products/bathroom-aids/accessories/soft-comfort-seat-height-4-cm-a28fee79/

## BAT246C – ETAC Swift Soft Comfort Seat - Low Profile

Features
Low-profile soft seat for enhanced comfort. Compatible with Swift Mobil chairs.

Technical specification

- Seat, opening width 18 cm

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or disinfect up to 85°C

Website link: https://www.etac.com/en-gb/uk/products/bathroom-aids/accessories/soft-comfort-seat-height-2-cm

## BAT246D – Super Soft Comfort Seat - Horseshoe - Low Profile 2cm

Features
Soft comfort seat with horseshoe cut-out for hygiene access. Low-profile design for Swift Mobil chairs.

Technical specification

- 2 cm, rear opening

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or disinfect up to 85°C

Website link: https://www.etac.com/en-gb/uk/products/bathroom-aids/accessories/soft-comfort-seat---2-cm-rear-opening/

## BAT310C – Extra Wide Perching Stool with Back and Arms

Features
Vinyl padded seat, side-mounted stress bars, large footprint for stability. Designed for users needing support while perching.

Technical specification

- Seat height 460-610mm

- Seat Width 510mm

- Seat Depth 330mm

- Product weight: Approx. 7kg

- User weight capacity: 280kg

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean with mild detergent

Website link: https://drivemedical.co.uk/community/bariatric-perching-stool

## BAT311 – Adjustable Width Swivel Bather

Features
Swivel bather with adjustable width arms for safe and easy transfers into the bath.

Technical specification

- Seat width 445mm

- Seat depth 380mm

- User weight capacity: 160 kg

- Reach parameters:

  - Vertical reach: Seat height approx. 50 mm above bath

  - Turning radius: Swivel seat 90° each direction

- Safe working load: 150 kg

- Movement parameters:

  - Rotation capability: 180° swivel

  - Tilt or recline angles: Fixed seat angle

  - Transfer path: Swivel seat for lateral transfer

  - Manual

- Control: Manual lever

- Mobility: Static (fits on bath)

- Materials: Aluminium, plastic seat

- Warranty: 1 year

Safety features

- Safety standards and compliance: CE marked

- Noise level: Silent

- Cleaning instructions: Wipe clean with disinfectant

Website link: https://www.smitcare.co.uk/myco-ultra-swivel-bather-musb/

## BAT465 – Bellavita Bathlift

Features

The Bellavita is a powered bath lift designed to assist with safe transfer into and out of the bath. It provides a low minimum seat height to support immersion and a reclining backrest to facilitate positioning during bathing. The lift is operated via a waterproof hand control and raises and lowers the user in a controlled manner. The backrest reclines up to 50° to support comfort and positioning. A front seat aperture allows improved access for personal hygiene. The unit can be used with or without removable, washable covers. The lift separates into two sections to support installation, removal, and storage. Four suction feet secure the unit within the bath. The hand control includes battery level indication and a safety function to prevent descent if there is insufficient charge to return to the top position. An emergency stop function is incorporated.

Technical specification

- Height: 6.5–48 cm (2.6–18.9")

- Width: 70 cm (27.6")

- Length: 83 cm (32.7")

- Seat height: 6.5–48 cm

- Seat depth: 42 cm

- Seat width: 35 cm

- Backrest height: 64 cm

- Backrest width: 32–36 cm

- Batteries included: Yes

- Battery cell type: Lithium-ion

- Number of batteries: 1

- Product weight: 9.7 kg

- Recline angle: 50°

- User weight capacity: 140 kg (22 st)

- Product warranty: 5 years

Website link: Bellavita Lightweight Battery-Powered Reclining Bath Lift

## BAT500LSS – Waist Strap Buckles – 17

Features

Waist strap designed for use with Freeway shower chairs. Intended to provide pelvic positioning support and reduce forward sliding in the seated position.

Technical specification

- Size: 17 inch

- Chair width compatibility: 440 mm (17")

- SKU: 1602WHABWSVAT relief

- Fastening: Buckle

- Product weight: Not specified

- User weight capacity: Dependent on host chair specification

- Safe working load: Dependent on host chair specification

- Warranty: Manufacturer standard

Safety features

- Assists with pelvic positioning in seated users

- Reduces risk of forward sliding

- Secured via buckle fastening system

- CE marked

- Wipe clean or machine washable

Website link: Freeway Buckle Waist Strap | Shower Chair | Replacement Spare Part Accessories | Buy Now Order Online | Near Me | Easy Care Systems

## BAT500LSM – Waist Strap Buckles – 19

Features
Waist strap designed for use with Freeway shower chairs. Intended to provide pelvic positioning support and reduce forward sliding in the seated position.

Technical specification

- Product: Waist strap

- Model: Freeway Buckle Waist Strap

- Size: 19 inch

- Chair width compatibility: 490 mm (19")

- Fastening: Buckle

- Product weight: Not specified

- User weight capacity: Dependent on host chair specification

- Safe working load: Dependent on host chair specification

- Warranty: Manufacturer standard

Safety features

- Assists with pelvic positioning in seated users

- Reduces risk of forward sliding

- Secured via buckle fastening system

- CE marked

- Wipe clean or machine washable

Website link: Freeway Buckle Waist Strap | Shower Chair | Replacement Spare Part Accessories | Buy Now Order Online | Near Me | Easy Care Systems

## BAT800 – Seats - 10635 - Medium App Seat – 19

Features
Medium aperture seat designed for use with compatible shower or commode frames.

Technical specification

- Dimensions: 19” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame (typically 160 kg)

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane or ABS plastic

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT801 – Seats - 10310 - Horseshoe Small – 19

Features
Small horseshoe-shaped seat for hygiene access, compatible with 19” frames.

Technical specification

- Dimensions: 19” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT804 – Seats - 10550 - Small App Seat – 17

Features
Small aperture seat for 17” frame, designed for use with commode or shower chairs.

Technical specification

- Dimensions: 17” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT808 – Frame T60B 19 Frame

Features
Shower or commode chair frame designed to accommodate 19” seat components. Compatible with various seating and support accessories.

Technical specification

- Dimensions: Frame width 19”

- User weight capacity: 200 kg

- Mobility: Static or wheeled depending on configuration

- Materials: Powder-coated steel or aluminium

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT809 – Seats - 10540 - Large App Seat – 17”

Features
Large aperture seat for 17” frame, designed for hygiene access and comfort.

Technical specification

- Dimensions: 17” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT810 – Seats - 10630 - Large App Seat - 19

Features
Large aperture seat for 19” frame, designed for use with commode or shower chairs.

Technical specification

- Dimensions: 19” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT814 – Frame - 10100 - T70B - 19 Frame

Features
Heavy-duty frame for 19” seating systems, compatible with a range of commode and shower chair seats.

Technical specification

- Dimensions: Frame width 19”

- User weight capacity: 200 kg

- Mobility: Static or wheeled depending on configuration

- Materials: Powder-coated steel or aluminium

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT815 – Seats - 10220 - Horseshoe Small - 17

Features
Small horseshoe-shaped seat for 17” frame, designed for hygiene access and user comfort.

Technical specification

- Dimensions: 17” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT817 – Seats - 10545 - Medium App Seat - 17

Features
Medium aperture seat designed for 17” frames, suitable for shower or commode use.

Technical specification

- Dimensions: 17” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT818 – Frame - 10105 - T70C - 17 Frame

Features
Commode or shower chair frame compatible with 17” seating systems.

Technical specification

- Dimensions: Frame width 17”

- User weight capacity: 200 kg

- Safe working load: 160 kg

- Mobility: Static or wheeled depending on configuration

- Materials: Powder-coated steel or aluminium

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT820 – Seats - 10210 - Horseshoe Large - 17

Features
Large horseshoe-shaped seat for 17” frame, designed for hygiene access.

Technical specification

- Dimensions: 17” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT821 – Seats - 10300 - Horseshoe Large - 19

Features
Large horseshoe-shaped seat for 19” frame, designed for hygiene access and comfort.

Technical specification

- Dimensions: 19” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- arranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT824 – Frame T60C 17 Frame

Features
Shower or commode chair frame designed to accommodate 17” seat components.

Technical specification

- Dimensions: Frame width 17”

- User weight capacity: 200 kg

- Reach parameters: N/A

- Safe working load: 160 kg

- Mobility: Static or wheeled depending on configuration

- Operation time: N/A

- Materials: Powder-coated steel or aluminium

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering function: N/A

- Alerts: N/A

- Noise level: Silent

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT825 – Seats - 10305 - Horseshoe Medium - 19

Features
Medium horseshoe-shaped seat for 19” frame, designed for hygiene access and user comfort.

Technical specification

- Dimensions: 19” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT827 – Frame - 10040 - T40B - 19 Frame

Features
Shower or commode chair frame designed to accommodate 19” seat components.

Technical specification

- Dimensions: Frame width 19”

- User weight capacity: 200 kg

- Mobility: Static or wheeled depending on configuration

- Materials: Powder-coated steel or aluminium

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT829 – Frame - 10045 - T40C - 17 Frame

Features
Shower or commode chair frame designed to accommodate 17” seat components.

Technical specification

- Dimensions: Frame width 17”

- User weight capacity: 200 kg

- Mobility: Static or wheeled depending on configuration

- Materials: Powder-coated steel or aluminium

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT830 – Seats - 10215 - Horseshoe Medium - 17

Features
Medium horseshoe-shaped seat for 17” frame, designed for hygiene access and user comfort.

Technical specification

- Dimensions: 17” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT832 – Armrests - 11105 - Straight Arm

Features
Straight armrests designed for use with compatible commode or shower chair frames, offering lateral support and comfort.

Technical specification

- Dimensions: Approx. 300 mm length

- Product weight: < 1 kg per armrest

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane or padded vinyl over metal

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT833 – Footrests - 1135STD - Footrest Standard - 195mm

Features
Standard footrest for use with compatible shower or commode chairs, providing lower limb support.

Technical specification

- Dimensions: 195 mm length

- Product weight: Approx. 1 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Control: Manual adjustment

- Mobility: Static

- Materials: Aluminium or coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT846 – Seats - 10640 - Small App Seat - 19

Features
Small aperture seat for 19” frame, designed for hygiene access and user comfort.

Technical specification

- Dimensions: 19” seat width

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT869 – Horizontal Handle Backrest - 17

Features
Backrest with horizontal handle for 17” frame, designed to provide upper body support and assist with transfers.

Technical specification

- Dimensions: Width 17”

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Powder-coated steel, polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT870 – Horizontal Handle Backrest - 19

Features
Backrest with horizontal handle for 19” frame, offering support and stability during seated use.

Technical specification

- Dimensions: Width 19”

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Powder-coated steel, polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT871 – Vertical Handle Backrest - 17

Features
Backrest with vertical handle for 17” frame, designed to provide upright support and assist with transfers.

Technical specification

- Dimensions: Width 17”

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Powder-coated steel, polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT872 – Vertical Handle Backrest - 19

Features
Backrest with vertical handle for 19” frame, offering upright support and stability during seated use.

Technical specification

- Dimensions: Width 19”

- Product weight: Approx. 2 kg

- User weight capacity: Matches host frame

- Safe working load: As per host frame

- Mobility: Static

- Materials: Powder-coated steel, polyurethane

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT873 – Waist Straps Velcro - 17

Features
Adjustable Velcro waist strap for 17” seating systems, designed to provide secure positioning.

Technical specification

- Dimensions: Adjustable length for 17” seat width

- Product weight: < 0.5 kg

- User weight capacity: Matches host equipment

- Safe working load: As per host equipment

- Control: Manual Velcro fastening

- Mobility: Static

- Materials: Nylon strap, Velcro

- Warranty: 1 year

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or machine washable

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT874 – Waist Straps Velcro - 19

Features
Adjustable Velcro waist strap for 19” seating systems, designed to provide secure positioning.

Technical specification

- Dimensions: Adjustable length for 19” seat width

- Product weight: < 0.5 kg

- User weight capacity: Matches host equipment

- Safe working load: As per host equipment

- Control: Manual Velcro fastening

- Mobility: Static

- Materials: Nylon strap, Velcro

- Warranty: 1 year

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or machine washable

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

## BAT881NC – Kanjo Eco Bathlift without covers

Features
The KANJO Eco provides perfect hygiene and safety when bathing. An ergonomically designed seat and back and a function that adjusts from a fixed to a reclining position at the push of a button allow the user to relax and bathe in comfort. Optional covers made of tear-resistant and in some cases, antibacterial materials, add even greater convenience.

Technical specification

- Bathlift Seat width without/with side flaps (cm): 37.5 / 71

- Seat depth (cm): 50

- Seat height (cm): 6.5–45.5

- Back rest W x H (cm): 35 x 68 Overall height (cm) 110.5

- Overall depth, back vertical (cm): 69

- Overall depth, back lowered (cm): 90

- Max. user weight (kg): 140

- Total weight incl. hand control unit (kg): 10.8

Website link: https://hmbathlifts.co.uk/product/kanjo-eco-bathlift/

- BAT881RC - Kanjo bathlift replacement covers

The Kanjo Bathlift Cover Set is a removable accessory designed to fit the seat and backrest of a Kanjo Bathlift. It provides a cushioned layer to support user comfort during bathing while protecting the underlying surfaces from wear. The covers attach using a clip-on method and can be removed for cleaning. Selected variants include an antibacterial treatment to assist with hygiene management.

Technical Specification:

- Material: Foam padding with outer cover

- Foam thickness: 6 mm (¼ inch)

- Compatibility: Kanjo Bathlift seat and backrest

- Fixing method: Clip/snap-on

- Cleaning: Removable and washable

- Surface: Tear-resistant

- Antibacterial treatment: Silver ion (selected variants)

Website link: Seat and backrest cover blue, with plugs - HandM Bathlifts

- BAT901A - Comfort Value Attendant Propelled Shower Chair 17" Seat width

This attendant propelled shower commode chair is designed for assisted showering and toileting. It features a powder-coated stainless steel frame and a 17-inch vinyl seat, available in full or aperture configurations. The chair can be used over most wash/dry toilets or with a commode pan.

Movable armrests and repositionable footrests with fold-up footplates support transfers and positioning. The unit is fitted with four braked castors for controlled movement and stability during use.

Safety Features:

- Four castors with individual brakes

- Stable, rigid frame construction

- Non-slip vinyl seating surface

- Secure armrests for support during transfers

- Footrests with fold-up plates for safe positioning

Technical Specification:

- Seat width: 17 inches (approx. 432 mm)

- Seat options: Full seat or aperture

- Seat height: 550 mm

- Overall dimensions: 965–985 mm (H) x 485 mm (W) x 635 mm (D)

- Frame material: Powder-coated stainless steel

- Seat material: Vinyl

- Maximum user weight: 180 kg

- Armrests: Movable

- Footrests: Repositionable with fold-up footplates

- Backrest: Fixed height with push handles

- Castors: 4 swivel castors with brakes

- Operation: Manual (attendant propelled)

- Compatibility: Suitable for most wash/dry toilets or commode pan use

- Colour: White/Grey

- Product weight: 23 kg

Website link: COMFORT VALUE® Shower Chair - Attendant | CareAndIndependence

- BAT901B - Comfort Value Attendant Propelled Shower Chair 19"

This attendant propelled shower commode chair is designed for assisted showering and toileting. It features a powder-coated stainless steel frame and a 19-inch vinyl seat, available in full or aperture configurations. The chair can be positioned over most wash/dry toilets or used with a commode pan.

Movable armrests and repositionable footrests with fold-up footplates support transfers and positioning. The unit is fitted with four braked castors for controlled movement and stability during use.

Safety Features:

- Four castors with individual brakes

- Stable, rigid frame construction

- Non-slip vinyl seating surface

- Secure armrests for support during transfers

- Footrests with fold-up plates for safe positioning

Technical Specification:

- Seat width: 19 inches (approx. 483 mm)

- Seat options: Full seat or aperture

- Seat height: 550 mm

- Frame material: Powder-coated stainless steel

- Seat material: Vinyl

- Maximum user weight: 180 kg

- Armrests: Movable

- Footrests: Repositionable with fold-up footplates

- Backrest: Fixed height with push handles

- Castors: 4 swivel castors with brakes

- Operation: Manual (attendant propelled)

- Compatibility: Suitable for most wash/dry toilets or commode pan use

- Colour: White/Grey

Website link: https://careandindependence.com/products/comfort-value/bathing-family/product/comfort-value-shower-chair-attendant

## PEABAT001 – Bealift – Standard

Features
Standard mobile hoist for safe patient transfers.

Technical specification

- Max user weight 63kg

- fit a standard bath size with an internal width of 50cm (19.5")

- Safety features

- CE marked

Website link: https://www.bealift.uk/bealifts

# Toileting and commodes

## BAT244 – Commode Pan Including Lid with Handle

Features
Standard commode pan with secure lid and handle for hygienic waste disposal.

Technical specification

- Dimensions: Approx. 400cl capacity

- Product weight: < 1 kg

- Materials: Polypropylene

- Warranty: 1 year

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wash with disinfectant

Website link: https://www.etac.com/en-gb/uk/products/bathroom-aids/accessories/pan-with-lid-and-handle/

## BAT875 – Oval Commode Pans

Features
Standard oval commode pan with lid and handle, designed for use with a variety of commode chairs.

Technical specification

- Product weight: < 1 kg

- Mobility: Portable

- Materials: Polypropylene

- Warranty: 1 year

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wash with disinfectant

Website link: https://prismmedical.co.uk/wp-content/uploads/2025/12/Freeway-Shower-Chairs-Brochure-web.pdf

- PEATOI003 - Combi Potty Chair Size 3

The Combi Potty Chair Size 3 is a paediatric toileting and commode chair designed for children requiring seated support during toilet training. It can be used as either a potty chair or a commode depending on the child’s stage of toileting. The product includes a removable potty insert which allows for easy emptying and cleaning. The chair can also be used with a commode seat when required. This version is supplied with adjustable arms, allowing easier lateral transfers and improved access for carers. The chair is intended to provide basic seated postural support during supervised toileting activities in home, school, or care environments.

All surfaces are designed to support routine cleaning and infection control procedures.

Technical Specification (Size 3)

- Seat height: 300–400 mm

- Seat width: 365 mm

- Seat depth: 310–385 mm

- Back height: 400 mm

- Maximum user weight: 55 kg

- Product weight (shipping weight): 14 kg

- Arm type: Adjustable arms

- Use type: Indoor supervised use

- Function: Toileting / potty training / commode use

- Removable components: Potty insert and commode seat

Safety Information

- Must be used under constant supervision at all times.

- Ensure the chair is positioned on a stable and level surface before use.

- Do not exceed the maximum user weight of 55 kg.

Website Link: toileting and bathing :: Combi Toileting Chair - special needs furniture, disabled furniture, therapeutic equipment, smirthwaite

- PEATOI150 – Flamingo Potty Size 1 with Lap Belt

Features
Paediatric potty chair with lap belt for safety and postural support.

Technical specification

- Dimensions: Size 1 seat

- User weight capacity: 35KG

- Safe working load: 48KG

- Control: Manual adjustment

- Mobility: Static

- Materials: Plastic frame, padded seat, safety belt

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/toilet--bath/r82-flamingo/

## PEATOI151 – Flamingo Potty Size 2 With Lap Belt

Features
Paediatric potty chair with adjustable seat height and lap belt for safety and support.

Technical specification

- Dimensions: Size 2 seat

- User weight capacity: 50KG

- Safe working load: 65KG

- Control: Manual adjustment

- Mobility: Static

- Materials: Plastic frame, padded seat, safety belt

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/toilet--bath/r82-flamingo/

## TOI110 – Mediatric Commode Det. Arm – Adj. Height

Features
Commode with detachable arms and adjustable height for ease of transfer and comfort.

Technical specification

- Seat Height 450 – 600mm

- Seat Width 580mm

- Seat Depth 460mm

- User weight capacity: 285kg

- Control: Manual adjustment

- Mobility: Static

- Materials: Steel frame, plastic seat and pan

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.cefndy.com/product/m222-mediatricadjustable-height-detachable-arms-commode/

## TOI705HH – EZY-FIT 2" toilet seat without lid

Features

The EZY-FIT® height-adjustable raised toilet seat is a modular toileting aid intended to assist individuals who require a higher seating position. It reduces the degree of hip and knee flexion needed when sitting and standing, which may support users with reduced mobility or those recovering from surgery.The unit is constructed from polypropylene and uses a system of interchangeable seat bases and spacers to provide multiple height options (50 mm, 100 mm, or 150 mm). It is secured to the toilet using suction feet and is compatible with a range of toilet bowl shapes, including round, oval, and square designs. The surface is smooth to facilitate cleaning, and an optional lid may be fitted depending on configuration.

Technical specification

- Overall height: 190 mm

- Seat height: 50mm (2")

- Length: 380 mm

- Width: 400 mm

- Maximum user weight: 200 kg (31 st)

- Material: Polypropylene (plastic)

- Hygiene: Washable with non-abrasive cleaners; autoclavable; store out of direct sunlight

- Disposal: Recyclable

- Warranty: 36 months

Website link: EZY-FIT 6" Raised Toilet Seat – Helping Hand

## TOI706HH– EZY-FIT 4" toilet seat without lid

Features

The EZY-FIT® raised toilet seat (100 mm / 4") is a toileting aid designed to increase seat height and reduce the degree of hip and knee flexion required during sitting and standing. It is intended for users with reduced mobility or those requiring additional support during toileting.

The seat is manufactured from polypropylene and attaches to the toilet using suction feet, providing a secure fit without the use of tools. It is compatible with a range of toilet bowl shapes, including round, oval, and square designs. The surface is smooth to support routine cleaning and infection control. An optional lid is available depending on configuration.

Technical Specifications

- Seat height: 100 mm (4")

- Overall height: 190 mm

- Length: 380 mm

- Width: 400 mm

- Maximum user weight: 200 kg (31 st)

- Material: Polypropylene (plastic)

- Hygiene: Washable with non-abrasive cleaners; autoclavable; store out of direct sunlight

- Disposal: Recyclable

- Warranty: 36 months

Website link: EZY-FIT 4" Raised Toilet Seat – Helping Hand

## TOI707HH– EZY-FIT 6" toilet seat without lid

Features

The EZY-FIT® height-adjustable raised toilet seat is a modular toileting aid intended to assist individuals who require a higher seating position. It reduces the degree of hip and knee flexion needed when sitting and standing, which may support users with reduced mobility or those recovering from surgery.The unit is constructed from polypropylene and uses a system of interchangeable seat bases and spacers to provide multiple height options (50 mm, 100 mm, or 150 mm). It is secured to the toilet using suction feet and is compatible with a range of toilet bowl shapes, including round, oval, and square designs. The surface is smooth to facilitate cleaning, and an optional lid may be fitted depending on configuration.

Technical specification

- Overall height: 190 mm

- Seat height: 150mm (6")

- Length: 380 mm

- Width: 400 mm

- Maximum user weight: 200 kg (31 st)

- Material: Polypropylene (plastic)

- Hygiene: Washable with non-abrasive cleaners; autoclavable; store out of direct sunlight

- Disposal: Recyclable

- Warranty: 36 months

Website link: EZY-FIT 6" Raised Toilet Seat – Helping Hand

## TOI722 – Extra Wide Toilet Frame

Features
Toilet surround frame designed to provide additional support and stability during transfers. Height adjustable to suit a range of user requirements. Arm supports included for improved grip and user confidence. Suitable for industrial cleaning environments.

Technical specification

- Width between arms: 550 mm

- Frame depth: 470 mm

- Footprint (W x D): 615 x 490 mm

- Height to top of arms: 650 – 800 mm (adjustable)

- User weight capacity: Up to 254 kg (40 stone)

- Safe working load: 254 kg

- Control: Manual height adjustment

- Mobility: Static

- Materials: Not specified (industrial clean suitable finish)

- GTIN: 5060432982737

Safety features

- CE marked

- Arm supports for stability

- Suitable for industrial cleaning

- Wipe clean design

Website link: https://www.cefndy.com/product/x154-toilet-surround/

## TOI729 – Adjustable Height Commode

Features
Standard commode with adjustable seat height and removable pan.

Technical specification

- Seat Height 45-60cm

- Seat Width 36cm

- Seat Depth 36cm

- User weight capacity: Up to 160 kg

- Control: Manual height adjustment

- Mobility: Static

- Materials: Powder-coated steel, plastic seat and pan

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/stacking-commode

## TOI731 – Glideabout Commode

Features
Mobile commode chair with four castors, removable pan, and adjustable height.

Technical specification

- Seat Size 435 x 460mm

- Seat Height 500mm

- Product weight: Approx. 10–12 kg

- User weight capacity: 190kg

- Control: Manual height adjustment

- Mobility: 4 castors with brakes

- Materials: Powder-coated steel, plastic seat and pan

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.smitcare.co.uk/apex-mobile-commode-sac360/

- TOI731A - height adjustable wheeled commode

The Lenham Height Adjustable Mobile Commode is a mobile toileting aid designed for users requiring assistance with personal care. It is height adjustable to allow positioning over standard toilets, including raised toilet seats, and to accommodate different user requirements. The commode is fitted with four braked castors to allow movement and secure positioning when stationary. Detachable arms are included to support lateral transfers. A removable commode pan is provided for waste collection and cleaning. The frame is finished with a protective coating, and the seat is made from moulded plastic for ease of cleaning and infection control.

Technical Specification

- Max User Weight: 165 kg

- Overall Height: 940–1060 mm

- Overall Width: 515 mm

- Overall Depth: 540 mm

- Seat Height: 525–665 mm

- Seat Width: 430 mm

- Seat Depth: 450 mm

- Width Between Arms: 400 mm

- Arm Height (from seat): 210 mm

- Floor to Arm Height: 735–860 mm

- Backrest Height: 330 mm

- Backrest Width: 480 mm

- Product Weight: 13.5 kg

- Commode Capacity: 7.5 litres

- Castors: 4 x braked castors

- Frame Finish: Chrome plated steel

- Seat Material: Moulded plastic

Safety Information

- For indoor use only

- Castor brakes must be applied when the product is stationary

- Ensure arms are correctly secured before use

- Do not exceed maximum user weight of 165 kg

- Check stability before each use

Website Link: Lenham Height Adjustable Mobile Commode | Aidapt

- TOI731C - Medecure Fixed Height Commode

Mobile commode features removable arms and leg rests.
Technical Specifications

- Overall Depth: 92.2 cm

- Overall Width: 53 cm

- Overall Height: 96.2 cm

- Seat Height: 52.8 cm

- Under Seat Clearance: 29 cm

- Width Between Arms: 42.5 cm

- Castor Size: 12.5 cm

- Max User Weight: 190 kg

Website link: https://products.medequip-uk.com/toi731.html

## 65. TOI741 – HD Toilet Seat and Frame Extra Wide

Features

The Grand Stamford Combi Scandia is a height-adjustable raised toilet seat designed for stability and ease of use. It consists of a metal support frame and a one-piece moulded seat that can be clipped on and removed as needed. The seat is contoured and includes front and rear cut-away sections to allow access for personal hygiene. A slight forward slope is incorporated into the seat surface to assist with sit-to-stand movement. The frame is constructed from steel and coated with a thermoplastic finish to resist wear and corrosion. Non-slip, non-marking feet provide grip on floor surfaces. The unit is stackable for storage. A bariatric variant (166HD) includes reinforced support struts on both sides of the frame to increase structural strength and accommodate higher user weight.

Technical specification

- Overall height: 66–81 cm

- Overall width: 74 cm

- Overall depth: 47 cm

- Seat height: 44.5–60 cm

- Seat depth: 43 cm

- Seat width: 53.5 cm

- Width between arms: 65 cm

- Product weight: 6.6 kg

- Maximum user weight: 280 kg (44 st)

- Warranty: 5 years

Website link: Grand Stamford Combi Scandia

- TOI756 - BIRSTALL BARIATRIC COMMODE

The Birstall Bariatric Mobile Commode is a mobile toileting chair designed for users requiring bariatric support. It provides a wider seating area and a reinforced frame to support higher user weights. The commode is fitted with four castors to allow movement and positioning by a caregiver. Each castor includes a brake to secure the unit when stationary. The arms are detachable to allow lateral transfers. The seat is removable for cleaning. A 14 litre commode bucket is included and can be removed for emptying. A lid is provided to reduce spill risk and contain odours.

Technical Specification

- Overall Height: 1070 mm

- Overall Width: 630 mm

- Overall Depth: 540 mm

- Seat Height: 500 mm

- Seat Width: 610 mm

- Seat Depth: 430 mm

- Backrest Height: 220 mm

- Backrest Width: 520 mm

- Width Between Arms: 610 mm

- Product Weight: 13 kg

- Max User Weight: 203 kg

- Castors: 4 x 130 mm with brakes

- Bucket Capacity: 14 litres

- Frame Material: Steel frame with protective coating

Safety Information

- Intended for indoor use only

- Castor brakes must be applied when the product is stationary

- Arms must be secured correctly before use to prevent instability

- Check castors and frame condition before each use

- Do not exceed maximum user weight of 203 kg

- Ensure bucket is correctly positioned before use

Website Link: Birstall Bariatric Mobile Commode

## TOI781 – Adjustable Height Bariatric Commode

Features
Heavy-duty commode with adjustable height, suitable for bariatric users.

Technical specification

- Seat Height 43-58cm

- Seat Width 61cm

- Seat Depth 43cm

- User weight capacity: 280kg

- Control: Manual height adjustment

- Mobility: Static

- Materials: Powder-coated steel, plastic seat

- Warranty: 1 year (reissue)

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

# Beds, mattresses and bed accessories

## BED112 – High Risk Foam Double Mattress

Features
Foam mattress designed for users at high risk of pressure ulcers, suitable for double beds.

Technical specification

- Dimensions: 198 x 136 x 15 cm

- User weight capacity: 248kg

- Mobility: Static

- Materials: High-density castellated foam

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, waterproof cover

Website link: https://www.winncare.uk/products/somerset/

- BED112INT - Sphynx Very High Risk Double Foam Mattress

The Integrity Sphynx™ Very High Risk mattress is a high-specification castellated foam mattress designed to provide effective pressure redistribution for individuals at very high risk of pressure injury. It features U-core side formers for enhanced stability and a vapour-permeable cover to support comfort, durability and infection control.

Specification

- Dimensions: 	198cm L x 88cm W x 15cm D

- Max User weight 254kg

- Risk category Very High Risk

Website link: https://www.integrity-healthcare.co.uk/products/pressure-area-care/mattresses/integrity-sphynx/

## BED113 – V. High Risk Static Foam Mattress

Features
Static foam mattress for very high-risk pressure care, suitable for profiling beds.

Technical specification

- Dimensions: 198 X 88 X 15cm

- User weight capacity: 254kg

- Safe working load: 220 kg

- Mobility: Static

- Materials: Multi-layered foam, waterproof cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.winncare.uk/products/avon/

## BED113DIV – For Divan Bed Only – Very High Risk Static Mattress

Features
Static pressure-relieving mattress for very high-risk users, suitable for divan beds.

Technical specification

- Dimensions: 198 x 88 x 15 cm

- User weight capacity: Up to 254 kg

- Mobility: Static

- Materials: High-density foam, multi-layered

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean cover

Website link: https://www.winncare.uk/products/avon/

## BED113INT - Sphynx Very High Risk Foam Mattress (30)

The Integrity Sphynx mattress is a static foam mattress designed for users at very high risk of pressure damage. It consists of a foam core with shaped side supports to assist positioning and transfers.

The surface is castellated to help distribute pressure. The outer cover is stretchable and waterproof, allowing cleaning and helping to maintain hygiene. The mattress is suitable for use in care environments, including hospitals and home care.

Safety Features:

- Pressure redistributing foam surface

- Side supports to assist positioning and reduce movement

- Waterproof cover to prevent fluid ingress

- Welded seams to reduce contamination risk

- Fire safety: BS7177 Crib 5 compliant

Technical Specification:

- Maximum user weight: 254 kg (40 stone)

- Dimensions: 1980 mm (L) x 880 mm (W) x 150 mm (D)

- Cover: Multi-stretch, waterproof

- Cleaning: Wipe clean or machine washable up to 95°C

- Drying: Air dry or low tumble dry

- Seams: Welded

Website link: Integrity Sphynx | Integrity Mobility Solutions

- BED113DIVINT - Very High Risk Mattress for a Divan Bed – Integrity

Very High Risk foam mattress for use with Divan size bed.

Specification
Depth = 150mm

Length = 1880mm

Max User Weight = 247kg

Width = 915mm

Website link: https://www.integrity-healthcare.co.uk/products/pressure-area-care/mattresses/integrity-sphynx/

## BED114 – High Risk Static Mattress

Features
Static foam mattress for high-risk pressure care, suitable for single profiling beds.

Technical specification

- Dimensions: 198 x 88 x 12.5cm

- User weight capacity: 190kg

- Mobility: Static

- Materials: Multi-layered foam, waterproof cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.winncare.uk/products/ola-5/

## BED114INT - Integrity Angora Eco - High Risk Foam Mattress (30)

The Integrity Angora mattress is a static foam mattress for users at high risk of pressure damage. It consists of a single-piece foam base with a castellated surface to assist with pressure distribution. The mattress includes a stretch, waterproof cover to support cleaning, and hygiene. It is suitable for use in clinical and home care environments.

Safety Features:

- Pressure redistributing foam surface

- Waterproof cover to reduce fluid ingress

- Sealed zip cover

- Fire tested to BS7177 Crib 5

Technical Specification:

- Product code: 1001-15060

- Risk category: High risk

- Maximum user weight: 254 kg (40 stone)

- Dimensions: 1980 mm (L) x 880 mm (W) x 150 mm (D)

- Foam type: Single-piece castellated foam

- Foam category: Severe to very severe

- Cover: Multi-stretch, waterproof

- Cleaning: Wipe clean or machine washable up to 95°C

- Drying: Air dry or low tumble dry

- Zip: Sealed

- Fire safety: BS7177 Crib 5 compliant

The product is intended for pressure care use in clinical or home settings.

Website Link: Integrity Angora | Integrity Mobility Solutions

## BED117 - Herida Lothian Active Mattress

Features
Dynamic mattress system for very high-risk users, re-issued model.

Technical specification

- Dimensions: 200cm x 88cm x 16cm

- User weight capacity: 180KG

- Mobility: Static

- Materials: Nylon PU cover, alternating air cells

- Warranty: 1 year (re-issue)

Safety features

- Safety standards and compliance: CE marked

- Alerts: Low pressure alarm

- Cleaning instructions: Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## BED129 – Medley Ergo Side Rail Bumpers

Features
Foam bumpers for Medley Ergo bed rails to reduce injury risk.

Technical specification

- Dimensions: Fits standard Medley Ergo rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Mobility: Static

- Materials: Foam core with wipe-clean cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or disinfect

Website link: https://www.thorpemill.co.uk/product/two-rail-bumper-with-net-window-trcsb/

## BED137 – Herida Cleveland Bariatric 11 ECO, Dynamic Mattress

Features
Dynamic bariatric mattress system with alternating pressure for very high-risk users.

Technical specification

- Dimensions: Bariatric size

- Product weight: Approx. 15–20 kg

- User weight capacity: Up to 318 kg

- Safe working load: 350 kg

- Mobility: Static

- Materials: Nylon PU cover, deep air cells

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Alerts: Low pressure and power failure alarms

- Cleaning instructions: Wipe clean

Website link: https://www.heridamedical.co.uk/product/cleveland-bariatric-11-eco/

## BED138 – Herida Argyll II Deep Cell Mattress

Features
Alternating pressure mattress with deep cell construction for high-risk users.

Technical specification

- Dimensions: 1980 x 880 x 200 mm

- Safe working load: 230kg

- Mobility: Static

- Materials: Nylon PU cover, deep air cells

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Alerts: Low pressure alarm

- Cleaning instructions: Wipe clean

Website link: https://www.winncare.uk/products/argyll-with-clp/

## BED141– Select Medical Pure Air 8 with Constant Low-Pressure Mattress

Features

The Pure Air 8 is a dynamic alternating pressure mattress designed to support pressure redistribution through immersion and offloading. It utilises a multi-cell air system with controlled pressure cycling to reduce interface pressure and support tissue viability. The system incorporates a figure-of-8 air cell configuration with alternating inflation to provide a stable support surface during therapy. Cyclical cell inflation supports pressure variation while maintaining patient positioning. The control unit regulates internal cell pressures and provides audible alerts for low pressure.

The surface is constructed from materials suitable for routine cleaning and decontamination. A CPR release function allows rapid deflation in emergency situations.

Technical specification

- Dimensions: 200 × 90 × 20 cm

- Number of cells: 20 (fully alternating)

- Inflated cell height: 20 cm (8")

- Alternation cycle: 1 in 2

- Cell cycle time: 10 or 15 minutes

- Initial inflation time: 30–45 minutes

- Cell material: TPU

- Internal cell pressure range: 20–45 mmHg

- Top cover: Multi-stretch polyurethane

- Seams: High-frequency welded

- Safe working load (SWL): >222 kg

- CPR function: Dial release, deflates within 30 seconds

- Cleaning: Cover machine washable up to 95°C

- Transport mode: Up to 24 hours via transport cap

Control unit:

- Dimensions: 28 × 11 × 20 cm

- Weight: 2.6 kg

- Pressure adjustment: Semi-automatic

- Max inflate: Auto-return to therapy after 20 minutes

- Noise level: <40 dB(A)

- Alerts: Audible low-pressure alert with mute function

- Control panel lock: Yes

Website link: PURE AIR 8 | Winncare

- BED141BWD - Select Medical Pure Air 8 with CLP
As Above

## BED142 – Herida – Ayrshire Deep Cell, Low Air Loss Mattress

Features
Dynamic mattress with deep cell design and low air loss for high-risk pressure care.

Technical specification

- Dimensions: 2000 x 880 x 200mm

- User weight capacity: 180kg

- Materials: Nylon PU cover, deep air cells

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Alerts: Low pressure alarm

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://www.heridamedical.co.uk/product/ayrshire-low-air-loss-mattress/

## BED143 – Herida – Moray – Combo Hybrid Mattress – Foam (Cell and Foam Split)

Features
Hybrid pressure-relieving mattress combining foam and air cell technology.

Technical specification

- Dimensions: 198 cm x 88 cm x 15 cm

- Product weight: Approx. 12–14 kg

- User weight capacity: 230kg

- Mobility: Static

- Materials: Foam and air cell combination, waterproof cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

- Optional pump for dynamic mode

Website link: https://www.winncare.uk/products/moray-combo/

## BED150 – 4 Section Profiling Bed – Standard

Features
Electric profiling bed with four adjustable sections for enhanced comfort and care.

Technical specification

- Dimensions: 1020 x 2240mm

- Height adjustment 330-730mm

- Product weight: Approx. 75–80 kg

- Safe working load: 180kg

- Movement: 4-section profiling

- Control: Handset

- Mobility: Lockable castors

- Materials: Steel frame, wooden panels

- Warranty: 2 years

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering: Yes

- Cleaning instructions: Wipe clean

Website link: https://www.invacare.co.uk/beds/beds/invacare-medley-ergo-medical-bed

## BED151 – Invacare Cotsides

Features
The Bella Wood Side Rail Kit is a set of four wooden side rails designed for use on compatible profile care beds. The rails are used to provide a physical boundary along the bed edge and are intended to reduce the risk of accidental falls during rest or sleep. The kit is supplied as rails only; mounting sliders are required and supplied separately. The system is designed for use with compatible bed frames including Invacare Accent and Medley models.

Technical Specification

- Components: Set of 4 side rails

- Material: Wood

- Configuration: Bed side rails only (sliders not included)

- Compatibility: Invacare Accent, Invacare Medley, Invacare Medley Ergo

- Application: Profile care bed systems

- Installation: Requires compatible slider fittings

Safety

- Must only be used with compatible bed systems and approved fittings

- Installation must be completed according to manufacturer instructions

- Rails must be checked regularly for secure attachment and stability

- Do not use if any component is damaged or incomplete

- Ensure correct fitting to reduce risk of entrapment or injury

- Rails should be used as part of a full bed safety assessment

- CE marked

- Wipe clean

Website link: Invacare Bella Wood Side Rail Kit for Profile Bed – Medical Spares

- BED151HH – Harvest Woburn Side Rails

Features
Side rails for Harvest Woburn beds to prevent falls and enhance user safety.

Technical specification

- Dimensions: Full-length rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://harvesthealthcare.co.uk/product/woburn-side-rails/

## BED151KAR – Karomed Supreme Bed Rails

Features
Side rails for Karomed Supreme beds, designed for safety and fall prevention.

Technical specification

- Dimensions: Full-length rails

- Product weight: Approx. 6 kg per pair

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## BED152 – Longer Full Length Side Rails – for Extended Bed

Features
Extended side rails for use with longer profiling beds to ensure full coverage.

Technical specification

- Dimensions: Extended length

- Product weight: Approx. 7 kg per pair

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.invacare.co.uk/beds/beds/invacare-medley-ergo-medical-bed

- BED152HH – Woburn Extended Side Rails

Features
Extended side rails for Woburn beds to provide additional safety for taller users.

Technical specification

- Dimensions: Extended Rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: https://harvesthealthcare.co.uk/product/bed-length-extension/

## BED153 – Bed Extension

Features
Extension piece for profiling beds to accommodate taller users.

Technical specification

- Dimensions: Adds approx. 200 mm to bed length

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual fitting

- Mobility: Static

- Materials: Steel, coated finish

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.invacare.co.uk/beds/beds/invacare-medley-ergo-medical-bed

## BED153HH – Harvest Bed Extensions

Features
Extension kit for Harvest profiling beds to accommodate taller users.

Technical specification

- Dimensions: Adds approx. 200 mm to bed length

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual fitting

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://harvesthealthcare.co.uk/product/bed-extension-kit/

## BED156 – Mattress Infill

Features
Foam infill piece for Invacare profiling beds to fill gaps and improve comfort.

Technical specification

- Dimensions: Compatible with Invacare bed models

- Product weight: Approx. 2 kg

- User weight capacity: Matches host mattress

- Safe working load: As per host bed

- Mobility: Static

- Materials: Foam with waterproof cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.thorpemill.co.uk/product/mattress-infillextension-with-straps-if881815-s/

## BED158 – Grab Rail for Invacare Medley Beds

Features
Grab rail designed for use with Invacare Medley profiling beds.

Technical specification

- Dimensions: 400 x 500mm

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual fitting

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.invacare.co.uk/medley-ergo-family

## BED175 – Harvest Bariatric Profiling Bed

Features
Electric profiling bed designed for bariatric users, with reinforced frame and wide platform.

Technical specification

- Dimensions: 2195 x 1340mm

- Mattress platform: 2000 x 1200mm

- Height 275 – 690mm

- Product weight: Approx. 120 kg

- User weight capacity: 317 kg

- Safe working load: 381 kg

- Control: Handset

- Mobility: Lockable castors

- Materials: Reinforced steel frame, wooden panels

- Warranty: 2 years

Safety features

- CE marked

- Emergency lowering

- Wipe clean

Website link: https://harvesthealthcare.co.uk/product/woburn-ultimate-bariatric-bed/

## BED175BEX – Bed Extension for BED175 Woburn Ultimate Bariatric Bed

Features 
Extension kit for Woburn Ultimate Bariatric Bed to increase bed length.

Technical specification

- Dimensions: Adds approx. 200 mm

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual fitting

- Mobility: Static

- Materials: Steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://harvesthealthcare.co.uk/product/woburn-ultimate-bariatric-bed/

- BED175CS – Side Rails for Woburn Ultimate Bariatric Bed

Features
Side rails for Woburn Ultimate bariatric bed to prevent falls and enhance safety.

Technical specification

- Dimensions: Full-length rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Reinforced steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://harvesthealthcare.co.uk/product/woburn-ultimate-bariatric-bed/

## BED175EX – Harvest Woburn Ultimate, Bed Rail Extension Pieces

Features
Extension pieces for bed rails on the Woburn Ultimate bariatric bed.

Technical specification

- Dimensions: Adds length to standard rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual fitting

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://harvesthealthcare.co.uk/product/woburn-ultimate-bariatric-bed/

## BED175INF– Harvest Bariatric Ultimate Mattress Infill

Features

Static mattress length extension infills are designed for use with compatible static mattress systems to extend mattress length and reduce gaps between the mattress and bed frame. They are used in conjunction with bed extension platforms.

The infill supports continuous surface coverage and maintains pressure redistribution in distal areas, including the heels.

Technical specification

- Dimensions: 1200 × 200 × 160 mm

- Extension length provided: 200 mm

- Construction: Foam insert

Website link: Static Mattress Length Extension Infill › Harvest Healthcare

## BED176 – Harvest Bariatric Bed Lever

Features
Heavy-duty bed lever for use with Harvest bariatric beds, providing support for transfers.

Technical specification

- Dimensions: 610 x 240mm

- Product weight: 3.7kg

- User weight capacity: 120kg

- Control: Manual fitting

- Mobility: Static

- Materials: Reinforced steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://harvesthealthcare.co.uk/product/grab-handle/

## BED206ACS – Dali Bed – Standard Bed Rails

Features
Side rails for Dali profiling beds to prevent falls and enhance safety.

Technical specification

- Dimensions: Full-length rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Steel with coated finish

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.lisclare.com/product/dali-eco/

## BED212A – Mattress Variator For Divan Beds

Features
Mattress variator to elevate head or foot sections of divan beds.

Technical specification

- Width 95-142cm

- Product weight: Approx. 6 kg

- User weight capacity: 255KG

- Control: Electric

- Mobility: Static

- Materials: Steel frame, foam platform

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://romamedical.co.uk/product/5421-rest-easy-mattress-elevator/

## BED225B – Double Loop Bedstick

Features
Dual-handled bedstick for assistance with repositioning and transfers.

Technical specification

- Length 960-1580

- Product weight: Approx. 3.8 kg

- User weight capacity: 127kg

- Control: Manual placement

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.aidapt.co.uk/public/template-product-details.php?product_id=VY429

## BED300 – Low to Medium Risk Castellated Foam Overlay

Features
Foam overlay mattress for users at low to medium risk of pressure ulcers.

Technical specification

- Risk category: At risk

- Maximum user weight: 127 kg (20 stone)

- Warranty: 2 years cover, 4 years foam

- Dimensions:

- 43 cm (L) × 43 cm (W) × 10 cm (H)

- 17” × 17” × 4”

- Other sizes available on request

- Foam properties:

- Density: 34–36 kg/m³

- Nominal hardness: 125 N

- Fatigue classification: Very severe use

- Fire performance: BS 7176

- Product weight: 1 kg

- Colour: Light blue

- Cleaning compatibility: Suitable for use with 1,000 ppm (0.1%) hypochlorite solution, or in accordance with local infection control protocols.

Website link: https://www.directhealthcaregroup.com/products/dyna-tek-pad-cushion/

## BED314 – Herida Yorkshire Mattress (Crib 7 soft cover) Size 198 x 88 x 15cm

Features
Static foam mattress with Crib 7 fire-retardant cover for high-risk users.

Technical specification

- Dimensions: 198 × 88 × 15 cm

- User weight capacity: 248kg

- Mobility: Static

- Materials: High-density foam, Crib 7 cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked, Crib 7 fire rating

- Cleaning instructions: Wipe clean, disinfectant safe

Website link: https://www.winncare.uk/products/yorkshire/

## BED334 – Mattress Stopper

Features
Accessory to prevent mattress movement on profiling beds.

Technical specification

- Dimensions: 380x450x310mm

- Fits standard bed frame

- Control: Manual fitting

- Mobility: Static

- Materials: Metal or plastic

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.aidapt.co.uk/public/template-product-details.php?product_id=VG813

- BED337HAR – Harvest Bumpers

Features
Foam bumpers for Harvest bed rails to reduce injury risk.

Technical specification

- Dimensions: Fits standard Harvest bed rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Mobility: Static

- Materials: Foam core with wipe-clean cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or disinfect

Website link: N/A

## BED337MB – Mesh Bumpers (pairs)

Features
Bed rail bumper set designed to fit over existing side rails on profiling or hospital beds. Provides padding along the length of the rails to reduce impact injury and to minimise the risk of limb entrapment or entanglement. The set includes upper and lower padded sections connected by a mesh panel, allowing visibility while maintaining coverage. Suitable for use with standard hospital beds with 200 cm side rails.

Technical specification

- Length: 2000 mm

- Compatibility: Standard hospital/profiling beds with 2000 mm side rails

- Configuration: Top and bottom padded sections connected by mesh panel

- Fixing method: Hook and loop fastening

- Quantity supplied: 2 bumpers (for one side of bed)

- Mobility: Not applicable (bed accessory)

- Materials: Padded covers with mesh (net window) section

- Colour: Blue

- Warranty: Manufacturer standard

Safety features

- Reduces risk of injury from contact with bed rails

- Covers gaps in side rails to reduce risk of limb entrapment

- Mesh panel maintains visibility of the user

- Secure attachment using hook and loop fastenings

- Flexible design allows rails to be lowered without removal

- CE marked

- Wipe clean or disinfect

Website link: https://www.thorpemill.co.uk/product/trcsb-blu/

## BED352 – Bed Lever for Solite Profiling Beds

Features
Supportive bed lever for Solite beds to assist with transfers and repositioning.

Technical specification

- Height 748mm (615mm above mattress platform when fitted)

- Width 270mm

- Product weight: Approx. 2.2kg

- Control: Manual fitting

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/beds-accessories/bed-accessories/standard-grab-handle

## BED355 – Dali Bed Grab Handle

Features
Grab handle for Dali beds to assist with repositioning and transfers.

Technical specification

- Dimensions: Fits standard Dali bed frame

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual fitting

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.lisclare.com/product/dali-eco/

## BED359 – Herida Avon Bariatric Mattress with Side Formers

Features
Bariatric static mattress with reinforced side formers for stability and support.

Technical specification

- Dimensions: 1980 x 1200 x 150mm

- User weight capacity: 317kg

- Mobility: Static

- Materials: High-density foam, side formers

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean, waterproof cover

Website link: https://www.winncare.uk/wp-content/uploads/2024/07/Avon-120.pdf

## BED360B– Dream Master Light

Features

The DreamMaster mattress elevator is a powered support system designed to raise the upper body in bed to assist with positioning, transfers, and independence in a domestic setting. It is compatible with a range of bed bases and is installed beneath the mattress. The system provides adjustable backrest elevation to support seated or semi-recumbent positioning. An optional powered knee-break function allows independent elevation of the lower limbs to assist with positioning and reduce migration in bed. Optional side rails can be fitted to provide additional support during repositioning. The unit is constructed from aluminium to provide structural support while maintaining a reduced overall weight.

Technical specification

- Maximum user weight: 254 kg (40 st)

- Width adjustment range: 76–152 cm (2’6”–5’)

- Backrest elevation range: 5°–85°

- Bed compatibility: Divan and flat/slatted bed bases

- Construction: Aluminium frame

- Knee-break (optional): Powered, independent adjustment via hand control

- Side rails (optional): Raise in conjunction with backrest

- Controls: Hand control (2 or 4 button depending on configuration)

Website link: DreamMaster Mattress Elevator (DM300) - SmitCare

## BED410 – 4 Section Profiling Bed with Auto-Regression

Features
Profiling bed with auto-regression to reduce shear and friction during movement.

Technical specification

- Dimensions: 965 x 2230mm

- User weight capacity: 191kg

- Safe working load: 226 kg

- Movement: 4-section profiling with auto-regression

- Control: Handset

- Mobility: Lockable castors

- Materials: Steel frame, wooden panels

- Warranty: 2 years

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering: Yes

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/solite-pro

## BED411 – 4 Section Low Profiling Bed with Auto-Regression

Features
Electric profiling bed with low height and auto-regression for pressure relief.

Technical specification

- Dimensions: 965 x 2230mm

- User weight capacity: 191kg

- Safe working load: 226kg

- Movement: 4-section profiling

- Control: Handset

- Mobility: Lockable castors

- Materials: Steel frame, wooden panels

- Warranty: 2 years

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering: Yes

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/solite-pro-low

## BED412 – Sidhil 4 Section Profiling Side Rails

Features
Side rails for Sidhil 4-section profiling beds, designed for safety and support.

Technical specification

- Dimensions: Full-length rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/standard-length-integral-side-rails

## BED412EX – Solite Pro Side Extended Side Rails

Features
Extended side rails for Solite Pro beds to provide additional coverage and safety.

Technical specification

- Dimensions: Extended length

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/solite-pro-extended-length-full-length-integral-side-rails

## BED413 – Mattress Infill with Straps

Features
Foam infill with securing straps to fill gaps in profiling beds and improve comfort.

Technical specification

- Dimensions: Compatible with standard profiling beds

- User weight capacity: Matches host mattress

- Safe working load: As per host bed

- Control: Manual strap fastening

- Mobility: Static

- Materials: Foam with waterproof cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/pressure-area-care/softrest-std-extension

## BED414EX – Solite Pro Mesh Bumper Extension Pad

Features
Extension pad for Solite Pro mesh bumpers to enhance safety and comfort.

Technical specification

- Dimensions: Compatible with Solite Pro bed rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Mobility: Static

- Materials: Mesh fabric, foam core

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or disinfect

Website link: https://drivemedical.co.uk/hospitals/extended-length-side-rail-pads-for-integral-extended-length-metal-side-rails

## BED441 – Dali Junior Short Bed

Features
Electric profiling bed designed for paediatric use with reduced length.

Technical specification

- Length 1980mm

- Width 1020mm

- Mattress platform 1840 x 900 mm

- Safe working load: 185 kg

- Movement: 4-section profiling

- Control: Handset

- Mobility: Lockable castors

- Materials: Steel frame, wooden panels

- Warranty: 2 years

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering: Yes

- Cleaning instructions: Wipe clean

Website link: https://www.lisclare.com/wp-content/uploads/Junior-bed-leaflet.pdf

## BED442 – Dali Junior Bed Side Rails

Features
Side rails for Dali Junior beds to prevent falls and provide safety for children.

Technical specification

- Dimensions: Full-length rails for junior bed

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.lisclare.com/wp-content/uploads/Junior-bed-leaflet.pdf

## BED442EB – Dali Junior Bed End Bumpers (90cm)

Features
Protective bumpers for the ends of Dali Junior beds to reduce injury risk.

Technical specification

- Dimensions: 90 cm length

- User weight capacity: N/A

- Safe working load: N/A

- Mobility: Static

- Materials: Foam core with wipe-clean cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or disinfect

Website link: https://www.lisclare.com/wp-content/uploads/Junior-bed-leaflet.pdf

## BED442MB – Dali Junior Side Rail Bumpers 180cm Mesh

Features
Mesh bumpers for Dali Junior bed side rails to reduce injury risk.

Technical specification

- Dimensions: 180 cm length

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Mobility: Static

- Materials: Mesh fabric, foam core

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or disinfect

Website link: https://www.lisclare.com/wp-content/uploads/Junior-bed-leaflet.pdf

## BED485 – 2 In 1 Bed Rail – Mk 2

Features
Dual-function bed rail for fall prevention and support during transfers.

Technical specification

- Base length: 625mm (24.5")

- Base width: 335mm (13")

- Base depth: 15mm (0.5")

- Handle maximum height from base of bed: 545mm (21.5")

- Product weight: 3kg (6.5lb)

- Maximum user weight: 160kg (25 stone)

- Control: Manual adjustment

- Mobility: Static (fits to bed frame)

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://completecareshop.co.uk/products/2-in-1-bed-rail

## BED520 – 4 Section Profiling Bed – Low

Features
Low-height profiling bed with electric adjustment for comfort and care.

Technical specification

- Dimensions: 1020 x 2240mm

- Height adjustment 210 – 610mm

- Safe working load: 180kg

- Movement: 4-section profiling

- Control: Handset

- Mobility: Lockable castors

- Materials: Steel frame, wooden panels

- Warranty: 2 years

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering: Yes

- Cleaning instructions: Wipe clean

Website link: https://www.invacare.co.uk/beds/beds/invacare-medley-ergo-medical-bed

## BED526– Casa Med Bariatric Low Profiling Bed

Features

The Casa Med Bariatric Low is a fully profiling bed designed to support bariatric users in care environments. It provides adjustable positioning to assist with pressure management, transfers, and clinical care. The bed includes electric adjustment of the backrest, leg rest, and knee break to facilitate positioning. A dual-action headrest and knee break function is incorporated to help reduce shear during profiling. The platform is height adjustable to support safe transfers and carer access. The frame is fixed and designed to provide stability under higher load capacity. The bed is supplied in a 120 cm width and is available with either plastic or wooden headboards. Integral side rails are included with the beech headboard version. The system uses electric actuators with a lockable handset for controlled operation. The bed can be assembled without tools and includes features to support transport and storage.

Technical specification

- Total weight: 95 kg (14.9 st)

- Outer dimensions (W × L): 132 × 213 cm

- Mattress support platform: 120 × 213 cm

- Height adjustment range: 40–80 cm

- Maximum safe working load: 317 kg (50 st)

- Maximum user weight: 286 kg (45 st)

- Castors: 100 mm diameter, 4 braking castors

- Profiling functions: Electric backrest, leg rest, knee break (2 motors)

- Sections: 4-section mattress support

- Control system: Electric handset (lockable)

- Construction: Steel frame

- Side rails: Integral (beech version) or optional

- Warranty: 3 years (parts), 10 years (frame components)

Website link: Casa Med Bariatric Profiling Bed Low Version

## BED526BL – Grab Rail for bed526 Bariatric Profiling Bed

Features
Support rail for bariatric profiling bed model 526, aiding transfers and repositioning.

Technical specification

- Dimensions: Compatible with bed526

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual fitting

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/hospitals/beds-accessories/bed-accessories/bariatric-grab-handle

## BED526INF - Bariatric Bed Mattress Infill

Mattress infill is added on top of the extension bracket to cover the additional metal frame.

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## BED552 – High Risk Repose Overlay

Features
The Repose Mattress Overlay is intended for use on a standard single bed and provides comfort alongside pressure redistribution. It is suitable for use in both acute and community care settings.

The overlay is supplied with an integrated pump that allows it to be inflated quickly. The pump automatically inflates the mattress to the appropriate pressure for use.

Technical specification

- Contents:
 1 × Repose Mattress Overlay and 1 × Large Pump

- Inflated dimensions:
 L 1800 mm × W 785 mm × H 50 mm

Indications for use
Repose products are designed to support pressure redistribution for individuals at risk of developing pressure ulcers, including those assessed as very high risk. They may also be used for individuals with existing pressure-related tissue damage; clinical supervision is recommended where damage is severe.

Limitations for use

Repose is not suitable for:

- Individuals weighing over 222 kg (Weight limit: 222 kg when used on a high-specification foam mattress; 139 kg when used on a non-high-specification foam mattress.)

- Individuals with unstable spinal fractures

- Situations where body shape prevents full support from the mattress overlay or cushion

Repose may be used with bed and trolley side rails, provided this is consistent with the manufacturer’s instructions and supported by a risk assessment.

Precautions

If any deterioration in skin condition is observed, seek clinical advice and discontinue use if recommended. Avoid contact with heat sources and sharp objects. When deflated, keep out of reach of infants, young children, and pets.

Website link: Repose Mattress Overlay | Frontier Medical Group

- BED623 – Quattro Plus Mattress Replacement Single

Features

Quattro Plus is a 1-in-4 alternating pressure mattress designed for the prevention and management of pressure ulcers. The system combines an active air cell cycle with pressure redistribution features to support tissue perfusion and reduce shear. The control unit has been redesigned to improve usability and infection control. It has a smooth, wipe-clean surface with fewer joints and crevices. Controls and an LCD display are positioned on the top panel for visibility. An integrated NFC tag allows quick access to device information, and servicing data can be accessed via the system interface. Internal components have been updated to simplify maintenance. The CPR function enables rapid deflation. The 1-in-4 alternating cycle maintains support across three inflated cells (≈75% body support) while one cell deflates to redistribute pressure. This may reduce peak interface pressures and improve patient comfort by limiting surface movement. TISSUEgard technology allows partial immersion and envelopment, helping to reduce interface pressure and shear. Ortho-Differential Support provides increased firmness at the mattress edges to assist transfers, with a softer central region for pressure redistribution.

Technical specification

Power Unit

- Dimensions (W × H × D): 346 × 258 × 156 mm

- Weight: 4.0 kg

- Cycle time: 16 minutes

Mattress

- Dimensions (L × W × D): 1950 × 880 × 180 mm

- Weight: 9.4 kg

- Cell type: 30 PU air cells

- Inner construction: Polyurethane (PU) air cells

- Cover: PU-coated multi-stretch nylon

- Maximum user weight: 200 kg (31 stone)

- Minimum user weight: None

Website link: Quattro® Plus | DHG

## BED705 – Herida Caithness Mattress

Features
High-specification mattress for medium to high-risk pressure care.

Technical specification

- Dimensions: 2000 x 880 x 150mm

- User weight capacity: Up to 200 kg

- Materials: Castellated foam, waterproof cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

- BED758 - Mercury Advanced Mattress Replacement System

The updated Dyna-Form Mercury Advance support surface uses sustainable foam while maintaining clinically proven performance in reducing pressure ulcer incidence. It also supports efficient patient care and lowers environmental impact.

Specification

- Product Code MAS12

- Dimensions 198cm x 88cm x 15cm

- Risk Category	Very High Risk

- Weight Limit 254kg capacity.

- Warranty 2 Years

Website Link: https://www.directhealthcaregroup.com/products/dyna-form-mercury-advance/

- BED758C - Mercury Advance Mattress with Community Pump

The updated Dyna-Form Mercury Advance support surface uses sustainable foam while maintaining clinically proven performance in reducing pressure ulcer incidence. It also supports efficient patient care and lowers environmental impact.
Specification

- Product Code MAS12

- Dimensions 198cm x 88cm x 15cm

- Risk Category	Very High Risk

- Weight Limit 254kg capacity.

- Warranty 2 Years

Website Link: https://www.directhealthcaregroup.com/products/dyna-form-mercury-advance/

- BED760 - Repose Wedge Positioner

Repose pressure relieving inflatable Wedge for the prevention and treatment of pressure ulcers.
Specification

- Inflated Dimensions: W 690mm x L 570mm x H 90mm

Website link: https://www.frontier-group.co.uk/product/wedge

## BED793 – Crash Mats

Features
Safety mats placed beside beds to reduce injury risk from falls.

Technical specification

- Dimensions: Approx. 180 × 60 × 5 cm

- Product weight: Approx. 3–4 kg

- User weight capacity: N/A

- Safe working load: N/A

- Mobility: Portable

- Materials: High-density foam with wipe-clean cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or disinfect

Website link: Fall Out Mat FOM - Thorpe Mill

## BED795 – Parnell Premier Bed Rail

Features
Premium bed rail for fall prevention and support during transfers.

Technical specification

- Hand rail height 82cm

- User weight capacity: 115KG

- Control: Manual fitting

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.parnellproducts.com/product/parnell-premier-bed-rail/

## BED795MED - Floor Mounted Bedrail

The Premier Bed Rail is a support aid designed to assist users when getting into and out of bed. It provides a stable handhold positioned at the bedside to support transfers. The unit is free-standing and fits under the mattress, making it suitable for use with a range of bed types, including standard and adjustable beds. It can be assembled without tools and stored flat when not in use.

Safety Features:

- Stable floor-standing frame

- Secure handhold for transfers

- Compatible with adjustable and fixed beds

- Tool-free assembly to reduce fitting error

Technical Specification:

- Hand rail height: 820 mm

- Hand rail width: 280 mm

- Floor frame length: 820 mm

- Overall height: 920 mm

- Minimum clearance under bed: 30 mm

- Maximum clearance under bed: 320 mm

- Maximum user weight: 160 kg (25 stone)

Website Link: Medequip Products: Premier Bed Rail - BED795

## BED865 – Dali Bed Short

Features
Short version of the Dali profiling bed, suitable for smaller users or limited spaces.

Technical specification

- Length: 1950mm

- Width: 1010mm

- User weight capacity: 165 kg

- Safe working load: 185 kg

- Movement: 4-section profiling

- Control: Handset

- Mobility: Lockable castors

- Materials: Steel frame, wooden panels

- Warranty: 1 year

Safety features

- Safety standards and compliance: CE marked

- Emergency lowering: Yes

- Cleaning instructions: Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

- BED865SR - Side Rail for BED865 Dali Short

Side Rail for BED865 Dali Short.

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## BED866 – Dali Short Mattress – Oversize

Features
Oversized mattress for Dali short beds, offering enhanced comfort and support.

Technical specification

- Dimensions: 1860 x 930 x 158mm

- Max User Weight 175kg

- Product weight: Approx. 9kg

- Mobility: Static

- Materials: Foam core with waterproof cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or disinfect

Website link: https://www.lisclare.com/product/rmax/

## BED867 – RMAX Static Mattress 180×93×15.8

Features
High-specification static foam mattress for very high-risk users.

Technical specification

- Dimensions: 180 × 93 × 15.8 cm

- User weight capacity: 175KG

- Safe working load: 220 kg

- Mobility: Static

- Materials: Multi-layered foam, waterproof cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.lisclare.com/product/rmax/

## BED913 – Standard Bed Rails for Profiling Bed

Features
Standard side rails for profiling beds to prevent falls and support safe transfers.

Technical specification

- Dimensions: Full-length rails

- User weight capacity: Matches host bed

- Safe working load: As per host bed

- Control: Manual locking

- Mobility: Static

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/grange-side-rail

## BED922 – Bed Side Wedges

Features
Foam wedges used to reduce risk of falls from bed without restrictive rails.

Technical specification

- Dimensions 900mm × 200mm

- User weight capacity: N/A

- Safe working load: N/A

- Mobility: Static

- Materials: Foam with wipe-clean cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or disinfect

Website link: https://www.thorpemill.co.uk/product/side-wedges-sw/

## BED925 – Bed Wedge (Large)

Features
Large foam wedge for postural support or elevation in bed.

Technical specification

- Dimensions: Approx. 600 × 300 × 150 mm

- Product weight: Approx. 2 kg

- User weight capacity: N/A

- Safe working load: N/A

- Mobility: Static

- Materials: Foam with waterproof cover

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or disinfect

Website link: https://www.directhealthcaregroup.com/products/dyna-form-wedge/

# Seating accessories

- SEA304A - Pair of Care Chair Armrest for 400mm (16") seat width

Pair of Care Chair Armrest for 400mm (16") seat width
Accessory – legacy / compatibility item only

- SEA304B - Pair of Care Chair Armrest for 450mm (18") seat width

Pair of Care Chair Armrest for 450mm (18") seat width
Accessory – legacy / compatibility item only

- SEA304C - Pair of Care Chair Armrest for 500mm (20") seat width

Pair of Care Chair Armrest for 500mm (20") seat width
Accessory – legacy / compatibility item only

- SEA304D - Pair of Care Chair Armrest for 550mm (22") seat width

Pair of Care Chair Armrest for 550mm (22") seat width
Accessory – legacy / compatibility item only

## 142.               SEA304FP – Accora Care Chair Removable Footplate

Features
Detachable footplate for Accora Care Chairs to support lower limbs and aid transfers.

Technical specification

- Dimensions: Adjustable height and angle

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Control: Manual adjustment

- Mobility: Static

- Materials: Metal frame, padded footrest

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: N/A

## 143.               SEA304HA2 – 51mm (2″) Depth Adjustment Kit for Care Chair

Features
Depth adjustment kit for Accora Care Chairs to improve postural support.

Technical specification

- Dimensions: Reduces Approx 51 mm from seat depth

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Control: Manual fitting

- Mobility: Static

- Materials: Upholstered foam, metal brackets

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://accora.care/specialist-seating/chair-accessories/depth-adjustment-kit

## SEA304LE – Accora Care Chair Leg Extension

Features
Leg extension accessory for Accora Care Chairs to increase seat height.

Technical specification

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Control: Manual fitting

- Mobility: Static

- Materials: Metal and plastic

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## SEA304M - Medium Accora Care Chair WITH 450mm armrests attached (4)

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## SEA306HA2 – 2” Height Adjustment Kit for Comfort Chairs

Features
Height adjustment kit to raise seat height by 2 inches for Configura Comfort Chairs.

Technical specification

- Dimensions: Adds 2″ (approx. 51 mm) to seat height

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Control: Manual fitting

- Mobility: Static

- Materials: Metal and plastic components

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://accora.care/specialist-seating/chair-accessories/height-adjustment-kit

## SEA306L – Configura Comfort Chair, Large

Features
Large-sized Configura Comfort Chair with adjustable posture and pressure relief features.

Technical specification

- Width 850mm

- Height 1115mm

- User weight capacity: Up to 160 kg

- Movement: Tilt-in-space, recline

- Control: Handset

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://accora.care/specialist-seating/chairs/configura-comfort

## SEA306M – Configura Comfort Chair, Medium

Features
Medium-sized Configura Comfort Chair with adjustable posture and pressure relief features.

Technical specification

- Width 800mm

- Height 1115mm

- User weight capacity: Up to 160 kg

- Movement: Tilt-in-space, recline

- Control: Handset

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean upholstery

Website link: https://accora.care/specialist-seating/chairs/configura-comfort

## SEA306S – Configura Comfort Chair, Small

Features
Small-sized Configura Comfort Chair for users requiring narrower seating dimensions.

Technical specification

- Width 750mm

- Height 1080mm

- User weight capacity: Up to 160 kg

- Movement: Tilt-in-space, recline

- Control: Handset

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://accora.care/specialist-seating/chairs/configura-comfort

## SEA315A – Accora Advance Chair, with Butterfly Back Rest

Features
Advanced riser recliner chair with butterfly backrest for enhanced comfort and support.

Technical specification

- Dimensions: Customisable seat width and depth

- User weight capacity: Up to 160 kg

- Movement: Tilt-in-space, recline, rise

- Control: Handset

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://accora.care/specialist-seating/chairs/configura-advance

## SEA315CB – Cocoon Backrest For Accora Chairs

Features
Padded cocoon-style backrest for enhanced comfort and lateral support in Accora chairs.

Technical specification

- Fits Advance, Comfort and Lite models.

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Mobility: Static

- Materials: Foam with wipe-clean cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: Cocoon Backrest | Riser Recliner Chair Accessories | Accora

## SEA630 – Chair – High Back

Features
High-back chair for general seating support, suitable for home or care environments.

Technical specification

- Seat Height 42-55cm

- Seat Depth 44.5-52cm

- Seat Width 49.5cm

- User weight capacity: 165kg

- Control: Manual

- Mobility: Static

- Materials: Wooden frame, upholstered seat and back

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://drivemedical.co.uk/community/oakham

## SEA773 – Active Medium-High Risk Cushion

Features
Pressure-relieving cushion for users at medium to high risk of pressure ulcers.

Technical specification

- Dimensions: 46 x 51 x 15cm

- User weight capacity: 108kg

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean cover

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## SEA638– Bariatric High Back Chair Ht Adjustable with Wings

Features

The Bariatric High Back Chair Ht Adjustable with Wings is a seating solution designed for plus-sized users, with an extra-wide frame and adjustable dimensions to support appropriate positioning. The chair allows adjustment of overall height, seat depth, and backrest height to accommodate individual user requirements. The seat and backrest are generously padded, with additional thickness to support pressure distribution. Upholstery is vinyl to facilitate cleaning, and the armrests are formed from moulded polyurethane (PU) to provide a durable, wipe-clean surface. Integrated lumbar support is incorporated into the backrest and varies in position relative to the selected backrest height. Adjustments are made using e-clips for seat height and wing nuts for seat depth and backrest height. The frame is constructed from welded steel and finished with a thermoplastic coating to resist wear and corrosion. Anti-slip feet are fitted to provide stability on floor surfaces. All materials and foams comply with UK furniture and furnishings fire safety regulations.

Technical specification

- Overall height: 108 cm

- Overall width: 70 cm

- Overall depth: 65 cm

- Seat width: 61 cm

- Seat depth: 45 cm

- Backrest width: 61 cm

- Product weight: 16.4 kg

- Maximum user weight: 254 kg (40 st)

Website link: Barkby Bariatric (with Wings)

## SEA980AR24 – Recliner Chair, Bariatric, 24″ Armrests Pair S

Features
Armrest set for 24″ bariatric recliner chairs, providing lateral support and comfort.

Technical specification

- Dimensions: 24″ width

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: N/A

## SEA980AR26 – Recliner Chair, Bariatric, 26″ Armrests Pair

Features
Armrest set for 26″ bariatric recliner chairs, providing lateral support and comfort.

Technical specification

- Dimensions: 26″ width

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: N/A

## SEA988LD – Lateral Supports for Accora Chairs – Pair

Features
Lateral support pads for Accora chairs to improve trunk stability and posture.

Technical specification

- Dimensions: Adjustable width

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Control: Manual adjustment

- Mobility: Static

- Materials: Foam padding, metal brackets

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: Lateral Support Wedges | Riser Recliner Accessories | Accora

## SEA984 – Bariatric Tilt-in-Space, 16″ Chair (with Independent Legrest)

Features
Heavy-duty tilt-in-space chair with independent legrest for bariatric users.

Technical specification

- Dimensions: Adjustable

- User weight capacity: 255kg

- Movement: Tilt-in-space, recline, legrest elevation

- Mobility: Static or wheeled

- Materials: Reinforced steel frame, upholstered seat

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://accora.care/specialist-seating/chairs/configura-bariatric

## SEA988DA – 2 Configura Chair Depth Adjustment Kit

Features
Depth adjustment kit for Configura chairs, allowing seat depth to be increased for user comfort.

Technical specification

- Dimensions: Reduces 50 mm depth

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Control: Manual fitting

- Mobility: Static

- Materials: Upholstered foam, metal brackets

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://accora.care/specialist-seating/chair-accessories/depth-adjustment-kit

## SEA1000A – Configura® Lite – Version 2 – Size Small

Features
Small version of the Configura Lite 2 chair for users requiring narrower seating.

Technical specification

- Width 750mm

- Height 1080mm

- User weight capacity: Up to 160 kg

- Movement: Single motor recline and rise

- Control: Handset

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://accora.care/specialist-seating/chairs/configura-lite-2

## SEA1000B – Configura Lite – Version 2 – Size Medium

Features
Lightweight riser recliner chair with medium seat size and pressure relief features.

Technical specification

- Width 800mm

- Height 1115mm

- User weight capacity: Up to 160 kg

- Movement: Single motor recline and rise

- Control: Handset

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://accora.care/specialist-seating/chairs/configura-lite-2

## SEA1000C – Height Adjustment Kit for Accora Lite Version 2 Chairs

Features
Accessory kit to raise the seat height of Accora Lite Version 2 chairs.

Technical specification

- Dimensions: Adds approx. 50 mm to seat height

- User weight capacity: Matches host chair

- Safe working load: As per host chair

- Control: Manual fitting

- Mobility: Static

- Materials: Metal and plastic

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://accora.care/specialist-seating/chair-accessories/height-adjustment-kit

## SEA1000L – Configura Lite 2 Large Single Motor Riser Recliner Chair

Features
Large version of the Configura Lite 2 chair with single motor for recline and rise.

Technical specification

- Width 850mm

- Height 1115mm

- User weight capacity: Up to 160 kg

- Movement: Single motor recline and rise

- Control: Handset

- Mobility: Static

- Materials: Upholstered foam, metal frame

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://accora.care/specialist-seating/chairs/configura-lite-2

# Cushions and pressure relief

## SEA100 – Contur Chair Overlay – RE-ISSUE ONLY

Features
Pressure-relieving overlay for use with standard chairs, designed for re-issue.

Technical specification

- Dimensions: Fits standard chair seat

- Product weight: Approx. 1.5 kg

- User weight capacity: 139 kg

- Safe working load: 139

- Mobility: Static

- Materials: Foam with waterproof cover

- Warranty: 1 year (re-issue)

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.frontier-group.co.uk/product/contur

## SEA107 – High Risk Foam Cushion 4 Height

Features
The Durham Cushion (DUR-115A) is a seat cushion for users who are at risk of pressure damage. It is made from two layers of foam. The top layer is memory foam that shapes to the body, and the base layer is firmer foam that provides support. It is used on chairs and other seating where a person is seated for extended periods.

Technical Specification

- Size: 43 x 43 x 10cm

- Maximum user weight: 150 kg

- Construction:

  - Top layer: Memory foam

  - Base layer: High-density foam

- Foam density:

  - Memory foam: 50–55 kg/m³

  - Base foam: 38–40 kg/m³

- Risk category: Very high risk

- Cover: Coated fabric cover

- Fire standard: BS7177 Crib 5

- Warranty: 2 years

Safety Features

- Spreads body weight to reduce pressure on seated areas

- Firm base helps keep the user stable

- Maximum user weight stated to avoid overloading

- Fire-tested materials

- Cover helps limit fluid entry

- Lightweight for safe handling and repositioning

- CE marked

- Wipe clean

Website link: https://www.winncare.uk/products/durham/

## SEA111 – High – Very High Risk Gel Cushion 3 Height

Features
The Berkshire Cushion (BERK-117) is a seat cushion designed for users at high risk of pressure damage. It is made from a combination of gel and foam to help spread weight while seated. The gel layer helps reduce pressure on contact areas, and the foam base provides support. It is intended for use on chairs where a person is seated for long periods.

Technical Specification

- Cushion dimensions: 440 × 440 × 75 mm

- Maximum user weight: 165 kg (26 stone)

- Cushion weight: Approximately 3 kg

- Construction:

  - Gel layer: Polyurethane / polyacrylamide

  - Base: High-density foam

- Foam density: 85 kg/m³ (base)

- Cover: Coated fabric cover

- Fire standard: BS7176

- Warranty: 2 years

Safety Features

- Gel and foam layers help spread body weight to reduce pressure

- Firm base helps maintain a stable sitting position

- Maximum user weight stated to avoid overloading

- Fire-tested materials

- Cover helps reduce fluid entry

- Designed for safe use during extended periods of sitting

- CE marked

- Wipe clean

Website link: https://www.winncare.uk/products/berkshire/

## SEA113 – V. High Risk Active Cushion

Features
Dynamic cushion for users at very high risk of pressure ulcers.

Technical specification

- Dimensions: 46 x 46 x 10cm

- User weight capacity: 150kg

- Mobility: Pump

- Materials: Air cell or gel system with waterproof cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

- Optional low-pressure alarm

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## SEA114 – High Risk Foam Cushion 3 Height

Features
Foam cushion for users at high risk of pressure ulcers.

Technical specification

- Dimensions: 43.18 x 43.18 x 7.62cm

- User weight capacity: Up to 150kg

- Mobility: Static

- Materials: High-density foam with waterproof cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: https://www.winncare.uk/products/durham/

## SEA139 – Herida Norfolk Bariatric Cushion

Features
Pressure-relieving cushion for bariatric users, designed for high-risk pressure care.

Technical specification

- Dimensions: 51 x 51 x 10cm

- User weight capacity: Up to 317kg

- Mobility: Static

- Materials: High-density foam with waterproof cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: https://www.winncare.uk/products/norfolk/

## BED553 – Heel Protectors – Repose

Features
Repose Foot Protectors are designed to reduce the risk of heel pressure damage by offloading the heel. A substantial proportion of pressure ulcers develop on the heels due to pressure, friction, and shear.

While support surfaces can help reduce pressure, complete offloading of the heel requires a dedicated device. Clinical studies have demonstrated the effectiveness of heel protectors across a range of care settings.

Technical specification

- Contents: One pair of Repose Standard Foot Protectors supplied with a small inflation pump

- Inflated Size: 380 mm (length) × 200 mm (width) × 230 mm (height)

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or disinfect

Website link: Repose Foot Protector | Frontier Medical Group

- EA601B - Ola Gel Cushion 17 x 17 2.5"

The Ola Gel cushion combines CME foam with a polyurethane gel layer to provide pressure redistribution and support, particularly protecting the ischial tuberosities. Its multi-layer construction enhances durability while maintaining consistent clinical performance.
Technical Specification:

- Product code: SM098G

- Dimensions: L43.2 cm x W43.2 cm x H9 cm

- Safe Working Load: 178 kg (28 st)

- Cushion Weight: 2 kg

Website link: https://www.winncare.uk/products/ola-gel/

## SEA107INT– Integrity Korat Cushion 17x17x4" (30)

Features

The Integrity Korat™ visco cushion is a pressure-redistributing seating cushion intended for individuals at very high risk of skin breakdown. It is suitable for use in hospital, homecare, and residential care environments.The cushion incorporates a viscoelastic foam upper layer that responds to body heat and immersion, allowing the surface to conform to the user and redistribute pressure. This may support comfort and reduce the likelihood of user intolerance. A high-density foam base provides structural support and reduces the risk of bottoming out. The outer cover is constructed from a multi-stretch, vapour-permeable, polyurethane-coated fabric to support pressure management and infection control. Welded seams reduce fluid ingress and facilitate cleaning, and a non-slip base helps maintain positioning during use. The product has been clinically evaluated for use by patients and healthcare professionals.

Technical specification

- Risk category: Very high risk

- Maximum user weight: 165 kg (26 st)

- Dimensions: 17x17x4"

- Alternative depth: 10 cm

- Foam type: Viscoelastic upper layer with high-density foam base (very severe category)

- Cover material: Multi-stretch, vapour-permeable, polyurethane-coated fabric

- Seams: Welded (high-frequency technology) to reduce fluid ingress

- Base: Non-slip

- Fire safety: BS7176 Crib 5 (composite foam and cover)

Website link: Integrity Korat Cushion | Integrity Mobility Solutions

## SEA1100– Integrity Korat Enviro Cushion (30)

Features

The Korat Enviro cushion is a pressure-redistributing overlay cushion designed for use on chairs and wheelchairs. It is intended for individuals requiring support with pressure management and microclimate control, particularly where moisture and skin integrity are clinical considerations. The cushion core is manufactured from a washable polymer material that can be cleaned and air dried, allowing repeated decontamination. The outer cover is removable and machine washable. The cover is constructed from a multi-stretch, vapour-permeable fabric to support infection control and maintain pressure-relieving properties. Welded seams reduce fluid ingress and facilitate cleaning, and a non-slip base assists with stability during use.The cushion functions as a reactive support surface, redistributing pressure in response to user weight and movement. Its internal structure allows air circulation through the core, which may assist in reducing heat and moisture at the skin interface, supporting the management of conditions such as moisture lesions and incontinence-associated dermatitis. The design supports comfort and immersion, while maintaining structural support to reduce the risk of bottoming out. The product has been clinically evaluated for use in care environments.

Technical specification

- Product type: Overlay cushion (for use on existing seating)

- Maximum user weight: 110 kg

- Core material: Washable polymer

- Cover: Multi-stretch, vapour-permeable, removable and machine washable

- Seams: Welded to reduce fluid ingress

- Base: Non-slip

- Function: Reactive pressure redistribution with airflow through cushion core

- Cleaning: Cushion core washable; cover machine washable

Website link: Integrity Korat Enviro | Integrity Mobility Solutions

## SEA111WIN– Selmed Gel Cushion

Features

The Selmed Gel cushion is constructed with multiple layers, combining visco-elastic foam, CME foam, and a polyurethane gel membrane.

The visco-elastic (memory) foam is temperature-sensitive and conforms to the user’s shape, distributing pressure across the seated surface. A gel layer beneath the foam helps reduce localized pressure, particularly around the ischial tuberosities.

A CME foam base provides structural support and contributes to the durability of the cushion.

Technical specification

- Standard cushion dimensions: 43.2 × 43.2 × 7.5 cm / 10 cm (17" × 17" × 3" / 4")

- Risk category: High to very high risk

- Fatigue classification: Very severe

- Maximum user weight: 180 kg (28 stone)

- Cushion weight: 1.65 kg

- Foam density: CME base – 50 kg/m³

- Cover material: Polyurethane-coated, multi-stretch nylon

- Seams: Welded

- Fire safety: Meets BS7177 Crib 5 composite standards

- Warranty: 2 years

Website link: Selmed Gel | Winncare

## SEA114INT– Integrity Korat Cushion 17x17x3" (30)

Features

The Integrity Korat™ visco cushion is a pressure-redistributing seating cushion intended for individuals at very high risk of skin breakdown. It is suitable for use in hospital, homecare, and residential care environments.The cushion incorporates a viscoelastic foam upper layer that responds to body heat and immersion, allowing the surface to conform to the user and redistribute pressure. This may support comfort and reduce the likelihood of user intolerance. A high-density foam base provides structural support and reduces the risk of bottoming out. The outer cover is constructed from a multi-stretch, vapour-permeable, polyurethane-coated fabric to support pressure management and infection control. Welded seams reduce fluid ingress and facilitate cleaning, and a non-slip base helps maintain positioning during use. The product has been clinically evaluated for use by patients and healthcare professionals.

Technical specification

- Risk category: Very high risk

- Maximum user weight: 165 kg (26 st)

- Dimensions: 17x17x3"

- Alternative depth: 10 cm

- Foam type: Viscoelastic upper layer with high-density foam base (very severe category)

- Cover material: Multi-stretch, vapour-permeable, polyurethane-coated fabric

- Seams: Welded (high-frequency technology) to reduce fluid ingress

- Base: Non-slip

- Fire safety: BS7176 Crib 5 (composite foam and cover)

Website link: Integrity Korat Cushion | Integrity Mobility Solutions

## SEA128A– DHG Dyna Tek Intelligent Air Cushion 17x17

Features

The Dyna-Tek® Intelligent Air cushion is a pressure-redistributing wheelchair cushion intended for users at very high risk of skin breakdown. It utilises a combination of air and foam cells with an internal valve system that redistributes air in response to user weight and movement, allowing automatic adjustment without manual inflation. The cushion incorporates a castellated and contoured surface to support pressure distribution and postural positioning. A layered air-over-foam construction provides impact damping and maintains support in the event of air loss. It is lightweight and suitable for single or multi-user environments. Removable cover options support infection control and routine cleaning.

Technical specification

- Risk category: Very high risk

- Maximum user weight: 152 kg (24 st)

- Dimensions: 43 × 43 × 7.5 cm (17" × 17" × 3")

- Cushion weight: 2 kg

- Foam density: 38–40 kg/m³

- Fatigue class: V

- Fire safety: BS 7176

- Cover options: Dartex PU, reverse Dartex PU, mesh spacer fabric

- Infection control: Removable, launderable cover; compatible with hypochlorite solutions

- Low profile option: 50 mm (2") available

- Warranty: 2 years

Website link: Dyna-Tek® Intelligent Air Cushion | DHG

## SEA315AP - Powered Accora Advance Dynamic Chair - With Butterfly Backrest (2)

Features
Powered specialist seating chair based on the Accora Configura Advance platform, supplied with a butterfly backrest to support postural management and comfort for higher dependency users. The chair provides tilt-in-space and recline functions to support pressure management, positioning, and assisted care in community and residential environments.

Technical specification

- Chair type: Powered dynamic specialist chair (Configura Advance platform) with butterfly backrest

- Seat width adjustment range: 350–550 mm (14–22 in)

- Seat depth adjustment range: 400–550 mm (16–22 in)

- Seat height adjustment range: 330–530 mm (13–21 in)

- Tilt-in-space range: 10°–37° (electric configuration)

- Backrest recline range: 90°–130° (lockable, reduced-shear recline)

- Overall chair width: 700 mm (28 in)

- Overall chair depth: 1075 mm (42 in)

- Overall chair height: 1280 mm (50 in)

- Overall depth with legs elevated: 1550 mm (61 in)

- Maximum user weight: 160 kg (25 st)

- Upholstery: Vinyl with vapour-permeable fabric to all contact areas

- Warranty: 2 years

- Controls (powered functions): Handset control (configuration dependent)

Safety features

- Maximum user weight clearly defined: 160 kg (25 st)

- Reduced-shear, lockable backrest recline intended to maximise pelvic stability

- Tilt-in-space positioning to support pressure management and postural stability

- Pressure management features: pressure-reducing fabric and continuous overlay for thigh support

- Cleaning: wipe-clean vinyl surfaces; vapour-permeable contact fabric as specified by manufacturer

- Use environment: indoor, supervised use in line with manufacturer instructions (Class I medical device platform)

Website link: https://accora.care/specialist-seating/chairs/configura-advance

## SEA6153– Integrity Manx Cushion 17"x17"x3" (30)

Features

The Integrity Manx Cushion is a pressure-redistributing seating cushion intended for individuals at high risk of skin breakdown. It is designed for use across a range of care environments and combines pressure management properties with compatibility for routine clinical use.The cushion is constructed from a single piece of castellated foam, which supports redistribution of peak pressures and contributes to user comfort. Its lower profile facilitates use with higher seating surfaces and may assist with transfers. The outer cover is manufactured from a multi-stretch, vapour-permeable polyurethane-coated fabric, supporting infection control measures while maintaining pressure-relieving performance. The sewn cover incorporates a zip with a protective flap to reduce fluid ingress and allow for cleaning.The product has been clinically evaluated for use by both patients and healthcare professionals. The cover is designed to withstand repeated cleaning, including high-temperature washing where required.

Technical specification

- Product code: 1001-15540 (10 cm depth variant)

- Risk category: High risk

- Maximum user weight: 150 kg (24 st)

- Dimensions: 43 cm (W) × 43 cm (L) × 7.5 cm (D)

- Alternative depth: 10 cm

- Foam type: Castellated foam (severe category)

- Cover material: Multi-stretch, vapour-permeable, polyurethane-coated fabric

- Fire safety: BS7176 Crib 5 (composite foam and cover)

Website link: Integrity Manx Cushion | Integrity Mobility Solutions

## SEA999B– Equazone Seat - 3 section Medium

Features

The Equazone Seat is a pressure-relieving seating insert designed to support individuals at risk of skin breakdown. It incorporates a self-inflating air and foam hybrid system that automatically adjusts to an appropriate pressure level without the need for manual inflation.

The insert consists of three moulded thermoplastic polyurethane (TPU) air sections, treated with antimicrobial properties, to provide lateral stability. An integrated foam layer functions as a support base in the event of air loss. The contoured design is intended to redistribute pressure, including offloading the coccyx area, while a widened cross-section supports air circulation to assist with heat management.

The construction includes welded seams to reduce fluid ingress and improve durability. The product is reusable and designed for routine use in seating applications.

Technical specification

- Maximum user weight: 254 kg (40 st)

- Overall height: 50 mm (2")

- Back section size: 43 × 71 cm

- Seat section size: 43 × 43 cm

- Leg section size: 43 × 43 cm

Website link: Equazone Seat – Helping Hand

# Hoists, slings and manual handling

## MOB1360 – SIBA Fitted Base Sheet – Single Bed – 90cm (W) × 200cm (L)

Features
Fitted base sheet for single beds, designed for use with in-bed turning systems.

Technical specification

- Dimensions: 90 × 200 cm

- Safe working load: 400KG

- Mobility: Static

- Materials: Satin and cotton blend

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Machine washable

Website link: https://www.hospitaldirect.co.uk/product/satin-in-bed-assistance-siba-sheets-combination-packs/

- MOB1386 - Satin In-Bed Assistance - Fitted Base Sheet - 120cm (W) x 200cm (L) (20)

A fitted in-bed sheet designed to help users move and turn more easily while lying in bed. It uses a central sliding panel that reduces friction, allowing smoother repositioning for both independent users and carers.

The sheet is designed to stay on the bed long-term and includes positioning markers to help with correct placement. It can be used for independent movement or assisted care and is suitable for a wide range of care settings.

Technical Specification:

- Size: 120 cm × 200 cm

- Mattress fit: Small double / queen size

- Material: Satin sliding panel with cotton edges

- Wash temperature: Up to 90°C

- Drying: Low-temperature tumble dry

- Design: Two-way sliding surface system

- Fit system: Adjustable drawstring edges for secure fitting

- Positioning aids: Central and directional markers

- Compatibility: Works with standard and dynamic mattresses (air mattress vents included)

- Use types: Independent use or assisted care (single carer)

- Safety features: Cotton edges help prevent slipping off bed

Website Link: Satin 'SIBA® Sheets' - 2 Way Base Sheets - Fully Fitted | Hospital Direct

## MOB00101A – Universal Poly Padded Sling Smal MANUAL HANDLING

Features

The Prism Universal Sling is used for general patient transfers and moving and handling tasks. It is designed to be easy to apply while providing appropriate support during transfers and can be used in either a divided leg or cross-over configuration depending on clinical requirements. The sling is available with or without head support and in a range of materials selected to suit different care needs. A polyester/parasilk blend offers strength and support for general use. Polyester mesh is breathable and allows water drainage, making it suitable for bathing or hydrotherapy environments. Parasilk is a low-friction material that can assist with fitting and removal, particularly where patient movement is limited, but it is not suitable for wet use as it retains water. Spacer fabric is soft and breathable, conforming to body shape and often used where a sling is required for extended periods. A patient-specific non-woven polyester option is intended for short-term use only and is not suitable for laundering, typically used where infection control or limited-duration use is required.

Technical specification

- Sling width: 825 mm

- User width range (minimum): 525 mm

- User width range (maximum): 610 mm

- Sling back: 490 mm

- User back range (minimum): 450 mm

- User back range (maximum): 530 mm

- Maximum weight capacity: 6-10 stone (38-64kg)

Clinical features

- Manufactured in the UK from UK-sourced materials

- Side straps for even weight distribution

- Quilted leg sections for added comfort (excluding spacer and patient-specific fabrics)

- No binding on outer leg section for added comfort (excluding spacer and patient-specific fabrics)

- Generous commode aperture for toileting

- Available in five fabric types and with or without head support

Website link: Prism Universal Sling

## MOB00101AH – Herida Universal Poly Padded Sling – Small

Features
Universal Poly Padded Sling

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use,

- Solid polyester is knitted with a smooth feel.

Website link: https://www.winncare.uk/products/universal-sling/

## MOB00101B– Universal Poly Padded Sling Medium

Features

The Prism Universal Sling is used for general patient transfers and moving and handling tasks. It is designed to be easy to apply while providing appropriate support during transfers and can be used in either a divided leg or cross-over configuration depending on clinical requirements. The sling is available with or without head support and in a range of materials selected to suit different care needs. A polyester/parasilk blend offers strength and support for general use. Polyester mesh is breathable and allows water drainage, making it suitable for bathing or hydrotherapy environments. Parasilk is a low-friction material that can assist with fitting and removal, particularly where patient movement is limited, but it is not suitable for wet use as it retains water. Spacer fabric is soft and breathable, conforming to body shape and often used where a sling is required for extended periods. A patient-specific non-woven polyester option is intended for short-term use only and is not suitable for laundering, typically used where infection control or limited-duration use is required.

Technical specification

- Sling width: 990 mm

- User width range (minimum): 630 mm

- User width range (maximum): 732 mm

- Sling back: 550 mm

- User back range (minimum): 500 mm

- User back range (maximum): 600 mm

- Maximum weight capacity: 8-17 stone (51-108kg)

Clinical features

- Manufactured in the UK from UK-sourced materials

- Side straps for even weight distribution

- Quilted leg sections for added comfort (excluding spacer and patient-specific fabrics)

- No binding on outer leg section for added comfort (excluding spacer and patient-specific fabrics)

- Generous commode aperture for toileting

- Available in five fabric types and with or without head support

Website link: Prism Universal Sling

## MOB00101BH – Herida Universal Poly Padded Sling – Medium

Features

- Universal Poly Padded Sling

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use,

- Solid polyester is knitted with a smooth feel.

Website link: https://www.winncare.uk/products/universal-sling/

## MOB00101CH – Herida Universal Poly Padded Sling – Large

Features

- Universal Poly Padded Sling

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use,

- Solid polyester is knitted with a smooth feel

Website link: https://www.winncare.uk/products/universal-sling/

## MOB00101D– Universal Poly Padded Sling X - Large

Features

The Prism Universal Sling is used for general patient transfers and moving and handling tasks. It is designed to be easy to apply while providing appropriate support during transfers and can be used in either a divided leg or cross-over configuration depending on clinical requirements. The sling is available with or without head support and in a range of materials selected to suit different care needs. A polyester/parasilk blend offers strength and support for general use. Polyester mesh is breathable and allows water drainage, making it suitable for bathing or hydrotherapy environments. Parasilk is a low-friction material that can assist with fitting and removal, particularly where patient movement is limited, but it is not suitable for wet use as it retains water. Spacer fabric is soft and breathable, conforming to body shape and often used where a sling is required for extended periods. A patient-specific non-woven polyester option is intended for short-term use only and is not suitable for laundering, typically used where infection control or limited-duration use is required.

Technical specification

- Sling width: 1150 mm

- User width range (minimum): 731 mm

- User width range (maximum): 850 mm

- Sling back: 700 mm

- User back range (minimum): 650 mm

- User back range (maximum): 750 mm

- Maximum weight capacity: 25-30 stone (159-191kg)

Clinical features

- Manufactured in the UK from UK-sourced materials

- Side straps for even weight distribution

- Quilted leg sections for added comfort (excluding spacer and patient-specific fabrics)

- No binding on outer leg section for added comfort (excluding spacer and patient-specific fabrics)

- Generous commode aperture for toileting

- Available in five fabric types and with or without head support

Website link: Prism Universal Sling

## MOB00101DH – Herida Universal Poly Padded Sling – XLarge

Features

- Universal Poly Padded Sling

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use,

- Solid polyester is knitted with a smooth feel.

Website link: https://www.winncare.uk/products/universal-sling/

## MOB00101EH – Herida Universal Poly Padded Sling – XXLarge

Features

- Universal Poly Padded Sling

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use,

- Solid polyester is knitted with a smooth feel.

Website link: https://www.winncare.uk/products/universal-sling/

## MOB00102AH – Herida Hammock Sling – Small

Features
A reliable, classic universal sling design, which has been refined for both standard transfers as a primary use, whilst having the ability to use in bathing applications.

This easy to use and popular product is manufactured in the UK by Herida Healthcare's specialist teams. Its design ensures a perfect fit to the user's body, whilst providing total support and stability, including patients who may have severe disabilities.

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use, the solid polyester is knitted with a smooth 
feel.

Website link: https://www.winncare.uk/products/the-hammock-sling-ouse/

## MOB00102BH – Herida Hammock Sling – Medium

Features
A reliable, classic universal sling design, which has been refined for both standard transfers as a primary use, whilst having the ability to use in bathing applications.

This easy to use and popular product is manufactured in the UK by Herida Healthcare's specialist teams. Its design ensures a perfect fit to the user's body, whilst providing total support and stability, including patients who may have severe disabilities.

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use, the solid polyester is knitted with a smooth 
feel.

Website link: https://www.winncare.uk/products/the-hammock-sling-ouse/

## MOB00102CH – Herida Hammock Sling – Large

Features
A reliable, classic universal sling design, which has been refined for both standard transfers as a primary use, whilst having the ability to use in bathing applications.

This easy to use and popular product is manufactured in the UK by Herida Healthcare's specialist teams. Its design ensures a perfect fit to the user's body, whilst providing total support and stability, including patients who may have severe disabilities.

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use, the solid polyester is knitted with a smooth 
feel.

Website link: https://www.winncare.uk/products/the-hammock-sling-ouse/

## MOB00102C– Hammock Sling - Poly/Slip Fit – Large

The Prism Hammock Sling is designed to provide increased support to the sacral region for users who require a higher level of postural assistance during transfers. It may be considered where a standard universal sling is not suitable, for example in cases of reduced body tone, following professional assessment.

It can be applied in either a divided leg or cross-over configuration, which may also support use for individuals with amputations following a professional risk assessment.

The sling includes a commode aperture to support toileting requirements.

It is manufactured in the UK from UK-sourced materials. The sling features quilted leg sections and no binding on the outer leg section for increased comfort (excluding spacer and patient-specific fabrics).

The sling is available in five fabric types:

Polyester / Parasilk is a smooth-knitted polyester providing strength and support for general use. Polyester mesh is a breathable and durable material used mainly for bathing or pool use. Parasilk is a low-friction material that assists with fitting and removal and is suitable for users with limited movement during application, but it is not suitable for bathing as it retains water when hoisted. Spacer is a super soft, breathable fabric that moulds to the user and is suitable for extended use. Patient-specific fabric is a non-woven polyester intended for short-term use only and is not suitable for washing, typically used for infection prevention or limited-duration use.

Technical specification

- Sling width: 1050 mm

- User width range (minimum): 668 mm

- User width range (maximum): 776 mm

- Sling back: 700 mm

- User back range (minimum): 650 mm

- User back range (maximum): 750 mm

- Maximum weight capacity: 17-25 stone (108-159kg)

Website link: Prism Hammock Sling

## MOB00102DH – Herida Hammock Sling – X-Large

Features
A reliable, classic universal sling design, which has been refined for both standard transfers as a primary use, whilst having the ability to use in bathing applications.

This easy to use and popular product is manufactured in the UK by Herida Healthcare's specialist teams. Its design ensures a perfect fit to the user's body, whilst providing total support and stability, including patients who may have severe disabilities.

- Manufactured to BSEN10535 standards

- Purple Polyester

- Designed for multi-functional use, the solid polyester is knitted with a smooth 
feel.

Website link: https://www.winncare.uk/products/the-hammock-sling-ouse/

## MOB0036A – Body Support Wedge – Standard Size

Features
Support wedge for positioning and comfort in bed or seating.

Technical specification

- Dimensions: Standard size

- Product weight: Approx. 1.5 kg

- User weight capacity: N/A

- Safe working load: N/A

- Size:  530 x 280 x 240mm

- Mobility: Static

- Materials: Foam with waterproof cover

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: Sit to Stand, Transfer and Lifting, Positioning and Support Equipment

- MOB346 – Mini Lift 160C

Features:
Compact electric standing hoist for safe patient transfers.

Technical Specification:

- Vertical reach: Approx. 800–1600 mm

- Product weight: Approx. 40 kg

- User weight capacity: Up to 160 kg

- Safe working load: 160 kg

- Recommended user height: 1400-2000 mm

- Lifting speed without load: 35 mm/s

- Control: Handset

- Mobility: Wheeled base with brakes

- Materials: Powder-coated steel

- Warranty: Manufacturer standard

Safety Features:

- CE marked

- Emergency lowering

- Wipe clean surfaces

Website link: https://www.directhealthcaregroup.com/products/minilift160classic/

## MOB397 – Sara Stedy Enhanced Transfer Aid

Features
Manual standing transfer aid designed to support sit-to-stand transitions for semi-weight-bearing users.

Technical specification

- Movement parameters:

  - Lift speed: Manual

  - Rotation capability: Swivel seat and pivoting base

  - Transfer path: Pivot and roll

  - Motorised vs. manual: Manual

- User weight capacity: Up to 182 kg

- Safe working load: 182 kg

- Control: Manual

- Mobility: Wheeled base with brakes

- Materials: Aluminium and coated steel

- Warranty: Manufacturer standard

Safety features

- CE marked

- Braked castors

- Wipe clean surfaces

Website link: https://www.arjo.com/int/products/patient-handling/standing-and-raising-aid/sara-stedy/

## MOB398L – Large Mackworth Willow Sling

Features
Full-body sling for general transfers, suitable for large users.

Technical specification

- Dimensions: Large size

- Product weight: Approx. 1.5 kg

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester mesh or padded fabric

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://prismmedical.co.uk/product/mackworth-willow-luxury-toilet-sling

## MOB398M – Medium Mackworth Willow Sling

Features:
Full-body sling for general transfers, suitable for medium users.

Technical Specification:

- Dimensions: Medium size

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester mesh or padded fabric

- Warranty: Manufacturer standard

Safety Features:

- CE marked

- Machine washable

Website link: https://prismmedical.co.uk/product/mackworth-willow-luxury-toilet-sling

## MOB398S – Small Mackworth Willow Sling

Features
Full-body sling for general transfers, suitable for small users.

Technical specification

- Dimensions: Small size

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester mesh or padded fabric

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://prismmedical.co.uk/product/mackworth-willow-luxury-toilet-sling

## MOB398XL – X Large Mackworth Willow Sling

Features
Full-body sling for general transfers, suitable for bariatric users.

Technical specification

- Dimensions: X-Large size

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester mesh or padded fabric

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://prismmedical.co.uk/product/mackworth-willow-luxury-toilet-sling

## MOB475AH – Herida Comfort Recline Sling – Small

Features

- Comfort Recline Sling

- Manufactured to BSEN10535 standards

- Black Spacer Fabric

- Padded, breathable, stretch material, padded for comfort during transfers.

- Has a pressure reducing material.

- Coloured coded straps and binding.

Website link: https://prismmedical.co.uk/product/prism-comfort-recline-sling

- MOB475B - Comfort Recline Sling – Medium

The Prism Comfort Recline Sling is a full-body, in-situ sling designed for prolonged use, offering comfort, support, and safety for amputees and users requiring extended periods in a sling.
Technical Specification

- Max User weight 108kg

- Material Spacer Fabric

Website link: https://prismmedical.co.uk/product/configuration/prism-comfort-recline-sling

## MOB475BH – Herida Comfort Recline Sling – Medium

Features

- Comfort Recline Sling

- Manufactured to BSEN10535 standards

- Black Spacer Fabric

- Padded, breathable, stretch material, padded for comfort during transfers.

- Has a pressure reducing material.

- Coloured coded straps and binding.

Website link: https://prismmedical.co.uk/product/prism-comfort-recline-sling

## MOB475CH – Herida Comfort Recline Sling – Large

Features

- Comfort Recline Sling

- Manufactured to BSEN10535 standards

- Black Spacer Fabric

- Padded, breathable, stretch material, padded for comfort during transfers.

- Has a pressure reducing material.

- Coloured coded straps and binding.

Website link: https://prismmedical.co.uk/product/prism-comfort-recline-sling

## MOB475DH – Herida Comfort Recline Sling – XL

Features

- Comfort Recline Sling

- Manufactured to BSEN10535 standards

- Black Spacer Fabric

- Padded, breathable, stretch material, padded for comfort during transfers.

- Has a pressure reducing material.

- Coloured coded straps and binding.

Website link: https://prismmedical.co.uk/product/prism-comfort-recline-sling

- MOB676B - Hospital Direct Flat Sliding Sheet with Handles 200x100cm

MANUAL HANDLING

A flat slide sheet with integrated handles designed to assist with repositioning and lateral transfers of individuals in bed or on a trolley. The sheet reduces friction to enable controlled movement during manual handling.

Technical Specification

- Type: Flat slide sheet with handles

- Material: Low-friction synthetic fabric (Slideezi®)

- Construction: Single-layer flat sheet with sewn handles

- Dimensions: 100 cm × 200 cm

- Surface: Dual-sided (low-friction side and soft contact side)

- Additional feature: Fluid-resistant barrier

- Weight capacity: Suitable for all patient weights (subject to risk assessment)

- Reusable: Yes

- Washing: 40°C to 74°C

- Drying: Low-temperature tumble dry

- Sizes available: Multiple size options

Safety Information

- For use by trained personnel following manual handling procedures

- Carry out a risk assessment prior to use

- Do not use if fabric or handles are damaged or worn

- Ensure handles are used correctly to maintain control during movement

- Monitor for risk of skin shear or discomfort during repositioning

- Do not use bleach or harsh cleaning agents

- Not intended for lifting; repositioning and transfer use only

Website Link: Flat Slide Sheets with Handles – Washable Slideezi® | Hospital Direct

- MOB685B - Slide Sheet - 1500mm x 700mm

A tubular roller slide sheet designed to assist with repositioning individuals in bed or on a trolley. It reduces friction to support lateral movement, turning, and repositioning during manual handling.

Technical Specification

- Type: Tubular roller slide sheet

- Material: Low-friction synthetic fabric (Slideezi®)

- Construction: Double-layer tubular design

- Dimensions: 70 cm × 150 cm

- Weight capacity: Suitable for all patient weights (subject to risk assessment)

- Reusable: Yes

- Washing: 40°C to 74°C

- Drying: Low-temperature tumble dry

- Sizes available: Multiple size options

Safety Information

- For use by trained personnel following manual handling procedures

- Carry out a risk assessment prior to use

- Do not use if the material is damaged or excessively worn

- Do not use bleach or harsh cleaning agents

- Ensure correct placement to minimise risk of skin shear or discomfort

- Not suitable for lifting; repositioning use only

Website Link: Tubular Roller Slide Sheets – Washable Slideezi® | Hospital Direct

- MOB707 – Oxford Midi 180 Electric Hoist

Features:
Electric mobile hoist for safe patient transfers in home and care environments.

Technical Specification:

- Vertical reach: Approx. 450–1800 mm

- Horizontal reach: Approx. 800 mm

- Turning radius: Approx. 1200 mm

- Arm extension range: Adjustable spreader bar

- Lift speed: Approx. 35 mm/sec

- Rotation capability: Manual sling rotation

- Tilt or recline angles: N/A

- Transfer path: Vertical lift and wheeled base

- Motorised vs. manual: Motorised lift, manual base

- Product weight: Approx. 42 kg

- User weight capacity: Up to 180 kg

- Safe working load: 180 kg

- Control: Handset

- Mobility: Wheeled base with brakes

- Materials: Powder-coated steel

Warranty: Manufacturer standard

Dimensions

- Length: 1225 mm

- Maximum Overall Height: 1840 mm

- Minimum Overall Height: 1240 mm

Spreader Bar

- Maximum Height: 1660 mm

- Minimum Height: 525 mm

- Height at Maximum Reach: 1060 mm

- Reach at Maximum Height: 420 mm

- Reach at Minimum Height: 440 mm

Performance and Manoeuvrability

- Maximum Reach: 656 mm

- Turning Radius: 1235 mm

Base Dimensions

- Legs Open – External Width: 1170 mm

- Legs Open – Internal Width: 1070 mm

- Legs Closed – External Width: 600 mm

- Legs Closed – Internal Width: 540 mm

Clearances and Castors

- Overall Height of Legs: 100 mm

- Ground Clearance: 25 mm

- Front Twin Castors: 75 mm

- Rear Braked Castors: 100 mm

Safety Features:

- CE marked

- Emergency lowering

- Wipe clean surfaces

Website link: https://www.joerns.co.uk/product/oxford-midi-180/

- MOB708 – Major Electric Hoist

Features
Heavy-duty electric hoist for safe patient transfers.

Technical specification

- Reach parameters:

  - Spreader Bar Reach at Maximum Height 610mm

  - Spreader Bar Reach at Minimum Height 515mm

  - Turning radius: 1310 mm

  - Arm extension range: Adjustable spreader bar

- Movement parameters:

  - Rotation capability: Manual sling rotation

  - Tilt or recline angles: N/A

  - Transfer path: Vertical lift and wheeled base

  - Motorised vs. manual: Motorised lift, manual base

- User weight capacity: Up to 200 kg

- Control: Handset

- Mobility: Wheeled base with brakes

- Materials: Reinforced steel

- Warranty: Manufacturer standard

Safety features

- CE marked

- Emergency lowering

- Wipe clean

Website link: https://oxfordhealthcare.co.uk/oxford-major-200-hoist/

- MOB728 – Patient Turner – Multi-function, Pivoting Transfer Platform

Features

The ReTurn7500i is a sit-to-stand and transfer aid designed to support assisted transfers between bed, wheelchair, and toilet, as well as repositioning within a seated surface. It provides a stable support frame for weight transfer and standing practice without manual lifting. It is also used as a rehabilitation aid for repeated sit-to-stand practice and controlled transfers.

Technical specification

- Product: Sit-to-stand and transfer platform

- Model: ReTurn7500i

- Product code: 7500i

- Height: 1150 mm

- Baseplate length: 685 mm

- Baseplate width: 570 mm

- Baseplate height: 40 mm

- Product weight: 16.5 kg

- Safe working load: 150 kg

- Unit: Single device

- Materials: Not specified

- Warranty: Manufacturer standard

Safety features

- Supports controlled sit-to-stand transfers

- Reduces need for manual lifting during transfers

- Provides stable base support during use

- Assists with repositioning in seated positions

- Compatible with ReTurn belt system

- CE marked

- Braked wheels

- Wipe clean

Website link: https://www.directhealthcaregroup.com/products/return7500i/

## MOB728BL – Large Belt – for Standing Transfer Aid

Features
Support belt for use with standing transfer aids, size large.

Technical specification

- Dimensions: Large size, adjustable

- Product weight: Approx. 0.8 kg

- User weight capacity: Matches host aid

- Safe working load: As per host system

- Control: Manual fastening

- Mobility: Used with standing aid

- Materials: Nylon, Velcro

- SWL: 205 kg

- Measurement: 125-145 cm

- Handles: 4

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://www.directhealthcaregroup.com/int/products/returnbelt/

## MOB728BM – Medium Belt – for Standing Transfer Aid

Features
Support belt for use with standing transfer aids, size medium.

Technical specification

- Dimensions: Medium size, adjustable

- Product weight: Approx. 0.7 kg

- User weight capacity: Matches host aid

- Safe working load: 205 kg

- Control: Manual fastening

- Mobility: Used with standing aid

- Materials: Nylon, Velcro

- Handles: 4

- Measurement: 105-125 cm

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://www.directhealthcaregroup.com/int/products/returnbelt/

## MOB728BS – Small Belt – for Standing Transfer Aid

Features
Support belt for use with standing transfer aids, size small.

Technical specification

- Dimensions: Small size, adjustable

- Product weight: Approx. 0.6 kg

- User weight capacity: Matches host aid

- Safe working load: 205 kg

- Handles: 4

- Measurement: 90-105 cm

- Control: Manual fastening

- Mobility: Used with standing aid

- Materials: Nylon, Velcro

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://www.directhealthcaregroup.com/products/returnbelt/

- MOB761E – ETAC Turner Pro MANUAL HANDLING

Features
Manual transfer aid for pivoting seated users between surfaces.

Technical specification

- Reach parameters:

  - Vertical reach: Adjustable handle height (780–1200 mm)

  - Horizontal reach: N/A

  - Turning radius: Approx. 800 mm

  - Arm extension range: Fixed handles

- Movement parameters:

  - Lift speed: Manual

  - Rotation capability: 360° swivel base

  - Tilt or recline angles: N/A

  - Transfer path: Pivot and roll

  - Motorised vs. manual: Manual

- Product weight: Approx. 8 kg

- User weight capacity: Up to 200 kg

- Safe working load: 200 kg

- Control: Manual

- Mobility: Wheeled base

- Materials: Aluminium, steel, non-slip footplate

- Warranty: Manufacturer standard

Safety features

- CE marked

- Braked wheels

- Wipe clean

Website link: Etac Turner Pro

- MOB773 – Mini Lift - Standing Hoist

(Referenced as Thorax Sling with Seat Support – Small)

Features:
Supportive sling for standing transfers, designed for use with compatible hoists.

Technical Specification:

- Size: Small

- Product weight: Approx. 1.1 kg

- Safe Working Load (SWL): 125 kg / 275 lbs

- User Length (min–max): 1400–2000 mm / 55.1–78.7 in

- Material: Powder-coated steel

Dimensions

- Height: 995 mm / 39.2 in

- Length: 900 mm / 35.4 in

- Width of Base (outer): 600–890 mm / 23.6–35.0 in

- Width of Base (inner): 445–745 mm / 17.5–29.3 in

Weight

- Total Weight: 39 kg / 86.0 lbs

Functionality

- Base Widening: Electrical

- Height of Footplate (for user): 50 mm / 2.0 in

- Castor Diameter: 80 mm / 3.1 in

- Lifting Speed (without load): 35 mm (1.4 in) per second

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester, padded sections

- Warranty: Manufacturer standard

Safety Features:

- CE marked

- Machine washable

Website link: https://www.directhealthcaregroup.com/app/uploads/hc_low_tech_datasheet_uk_minilift125_1506-6.pdf

## MOB773A – Thorax Sling with Seat Support – Small

Features
Same as above, sized for small users.

Technical specification

- Size: Small

- Product weight: Approx. 1.1 kg

- Recommended User weight: 30-60kg

- Safe working load: 300kg

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester, padded sections

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Machine washable

Website link: https://www.directhealthcaregroup.com/int/products/thoraxsling/

## MOB773B – Thorax Sling with Seat Support – Medium

Features
Same as above, sized for medium users.

Technical specification

- Size: Medium

- Product weight: Approx. 1.3 kg

- Recommended User weight: 50-80kg

- Safe working load: 300kg

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester, padded sections

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Machine washable

Website link: https://www.directhealthcaregroup.com/int/products/thoraxsling/

## MOB773D – Thorax Sling with Seat Support – Large

Features
Supportive sling for standing transfers, designed for use with compatible hoists.

Technical specification

- Size: Large

- Product weight: Approx. 1.5 kg

- Recommended User weight: 70-110kg

- Safe working load: 300kg

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester, padded sections

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Machine washable

Website link: https://www.directhealthcaregroup.com/int/products/thoraxsling/

## MOB778 – Mini Lift Sling - Small

Features:
Supportive sling for use with Mini Lift hoists, size small.

Technical Specification:

- Dimensions: Small size

- Product weight: Approx. 1.1 kg

- Recommended user weight: 30-60 kg

- Lift vest width: 94 cm

- SWL: 300 kg

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester, padded sections

- Warranty: Manufacturer standard

Safety Features:

- CE marked

- Machine washable

Website link: https://www.directhealthcaregroup.com/int/products/thoraxsling/

## MOB779 – Mini Lift Sling - Medium

Features:
Supportive sling for use with Mini Lift hoists, size medium.

Technical Specification:

- Dimensions: Medium size

- Product weight: Approx. 1.3 kg

- Recommended user weight: 50-80 kg

- Lift vest width: 104 cm

- SWL: 300 kg

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester, padded sections

- Warranty: Manufacturer standard

Safety Features:

- CE marked

- Machine washable

Website link: https://www.directhealthcaregroup.com/int/products/thoraxsling/

## MOB789 – Mini Lift Sling - Large

Features:
Supportive sling for use with Mini Lift hoists, size large.

Technical Specification:

- Dimensions: Large size

- Product weight: Approx. 1.5 kg

- Recommended user weight: 70-110 kg

- Lift vest width: 124 cm

- SWL: 300 kg

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester, padded sections

- Warranty: Manufacturer standard

Safety Features:

- CE marked

- Machine washable

Website link: https://www.directhealthcaregroup.com/int/products/thoraxsling/

## MOB938A – Motion Transfer Aid

Features:
Manual transfer aid for pivoting seated users between surfaces.

Technical Specification:

- Height: 117 cm

- Width: 61 cm

- Product Weight: 15.5 kg

- Maximum User Weight Capacity: 150 kg (24 st)

- Compact design providing stable support for standing, transferring, and toileting

- Ergonomic handlebar with multiple grip positions for both user and carer, including integrated hooks for compatibility with various sling types

- Padded leg support with tool-free, single-handed adjustment

- Low-profile platform for easy mounting and dismounting, with smooth-rolling castors for stability and effortless manoeuvrability across different surfaces

- Single-action braking system for enhanced safety and control

- Main bar angle: 75°

Safety Features:

- CE marked

- Braked wheels

- Wipe clean

Website link: https://drivemedical.co.uk/care-home/motion-transfer-aid

- MOB944 - Casa lift 180 Mobile Hoist

A mobile patient lifting hoist designed to assist with the transfer and repositioning of individuals with reduced mobility. Suitable for use in domestic and care environments, including transfers from floor level, beds, and chairs.

Technical Specification

- Type: Mobile patient hoist

- Maximum load: 180 kg

- Frame: Lightweight metal construction

- Mobility: Four castors

- Leg adjustment: Variable width for access around furniture

- Lifting range: Floor to elevated transfer height

- Control: Handheld wired handset

- Power: Electric actuator with rechargeable battery

- Safety controls: Emergency stop and emergency lowering functions

- Alert system: Audible low battery warning

- Portability: Dismantlable for transport and storage

- Components: Interchangeable parts

Safety Information

- For use by trained caregivers following manual handling procedures

- Do not exceed maximum load (180 kg)

- Ensure hoist is on a stable, level surface during operation

- Check all components and connections before use

- Use appropriate compatible sling (not included)

- Keep clear of moving parts during lifting and lowering

- Use emergency stop and lowering functions only when required

- Recharge battery regularly and respond to low battery alerts

- Do not use if any mechanical or electrical faults are present

Website Link: Casa Lift Hoist (Lifts up to 180kg)

## MOB947 – Hoist Electric 150

Features
Electric mobile hoist for safe patient transfers, reissue model.

Technical specification

- Dimensions: Standard lifting range

- User weight capacity: Up to 150 kg

- Movement: Electric lift

- Control: Handset

- Mobility: Wheeled base with brakes

- Materials: Powder-coated steel

- Warranty: 1 year (reissue)

Safety features

- CE marked

- Emergency lowering

- Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## MOB992J – Multifit - Size 11 Sling

Features:
Universal sling with padded support for general transfers, size 11.

Technical Specification:

- Dimensions: Size 11

- Product weight: Approx. 1.6 kg

- User weight capacity: Up to 250 kg

- Safe working load: 284 kg

- Control: Manual positioning

- Mobility: Used with hoist

- Materials: Polyester with padded sections

- Warranty: Manufacturer standard

Safety Features:

- CE marked

- Machine washable

Website link: https://oxfordhealthcare.co.uk/product/oxford-multifit-sling/

## MOB1019A – Small Chest Sling (Maple)

Features
Chest sling for upper body support during standing transfers, size small.

Technical specification

- Dimensions: Small size

- Control: Manual positioning

- Mobility: Used with hoist or standing aid

- Materials: Padded polyester

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://prismmedical.co.uk/product/configuration/mackworth-maple-stand-aid-sling-with-back-support

## MOB1019B – Medium Chest Sling (Maple)

Features
Chest sling for upper body support during standing transfers, size medium.

Technical specification

- Dimensions: Medium size

- Safe working load: 180 kg

- Control: Manual positioning

- Mobility: Used with hoist or standing aid

- Materials: Padded polyester

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://prismmedical.co.uk/product/configuration/mackworth-maple-stand-aid-sling-with-back-support

## MOB1019C – Large Chest Sling (Maple)

Features
Chest sling for upper body support during standing transfers, size large.

Technical specification

- Dimensions: Large size

- Control: Manual positioning

- Mobility: Used with hoist or standing aid

- Materials: Padded polyester

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://prismmedical.co.uk/product/configuration/mackworth-maple-stand-aid-sling-with-back-support

## MOB1019D – X Large Chest Sling (Maple)

Features
Chest sling for upper body support during standing transfers, size X-Large.

Technical specification

- Dimensions: X-Large size

- Control: Manual positioning

- Mobility: Used with hoist or standing aid

- Materials: Padded polyester

- Warranty: Manufacturer standard

Safety features

- CE marked

- Machine washable

Website link: https://prismmedical.co.uk/product/configuration/mackworth-maple-stand-aid-sling-with-back-support

## MOB1020A – Mackworth M160 Hoist

Features
Electric mobile hoist for safe transfers, reissue model.

Technical specification

- Dimensions: Standard lifting range

- User weight capacity: Up to 160 kg

- Movement: Electric lift

- Control: Handset

- Mobility: Wheeled base with brakes

- Materials: Powder-coated steel

- Warranty: 1 year (reissue)

Safety features

- CE marked

- Emergency lowering

- Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## MOB1021– Mackworth 180 hoist

Features

The Mackworth M180 Mobile Hoist is designed to support the transfer of individuals with reduced mobility in care home, hospital, and homecare environments. It may be used to assist with a range of transfers, including from bed, chair, or floor, within its specified lifting range. The hoist has a safe working load of 180 kg and uses an electric actuator system with wired hand controls for raising and lowering the carry bar. Manual leg opening allows adjustment of the base width to position the hoist around furniture and the user. The unit can be operated by a single caregiver.

Safety features include emergency lowering (electric and manual) and an emergency stop function. A low battery indicator is also included. The hoist is fitted with swivel castors and an ergonomic handle to support manoeuvrability. The frame is constructed from powder-coated steel. Assembly can be completed without tools. A folding mast option is available to support storage and transport. An optional quick release carry bar system allows for carry bar changes or the addition of a weighing scale unit.

Technical specification

- Safe working load: 180 kg (28 st 4 lb)

- Unit weight (unboxed): 39 kg

- Base weight: 18 kg

- Mast and boom weight: 21 kg

- Carry bar weight (optional): 2 kg

- Height: 1,340–1,970 mm

- Length: 1,310 mm

- Internal width: 540–1,030 mm

- External width: 680–1,120 mm

- Ground clearance: 25 mm

- Under bed clearance: 112 mm

- Turning circle: 1,010 mm

Additional specifications

- Material: Powder-coated steel, vinyl PVC

- Castors: 100 mm

- Hand control: Wired electric handset

- Charging time: 8 hours

- Lifts per charge: 30

- Sling connection: Looped

Website link: Prism Medical | Mackworth M180 Mobile Hoist | Patient Fall Prevention - Adapt Life

- MOB1333 - TUBULAR ROLLER SLIDE SHEETS - WASHABLE SLIDEEZI 120cm x 200cm

A tubular roller slide sheet used to reposition patients in beds or on trolleys. It allows turning, swivelling and transferring.

Technical Specification

- Type: Tubular roller slide sheet

- Dimensions: 120 cm × 200 cm

- Material: Slideezi® fabric

- Design: Tubular

- Reusable: Yes

- Washing: 40°C to 74°C

- Drying: Low-temperature tumble dry

- Sizes available: Multiple size options

Safety Information

- Do not use bleach

- Ensure correct use during repositioning

- Do not use if damaged

Website Link: Tubular Roller Slide Sheets – Washable Slideezi® | Hospital Direct

## MOB1618B – Guldmann Expandable, 2.5m Free Standing Gantry

Features

The free-standing rail system is designed for temporary installations and can be assembled without permanent fixing. It allows adjustment of both height and width to suit the environment and has a maximum lifting capacity of 250 kg. It is compatible with the GH1 hoist system.

Technical specification

- Free-standing rail system (2.5 m)

- Minimum dimensions (H × W): 2020 × 2720 mm

- Maximum dimensions when extended (H × W): 2820 × 3890 mm

- Maximum lifting capacity: 255 kg

Weight (excluding hoist):

- Complete rail system: 52.0 kg

- Each vertical pole: 13.5 kg

- Top brackets and rail: 25.0 kg

Website link: Guldmann Gantry System | Simply Moving and Sleeping | Specialists in Disability Equipment

## MOB1619A – Guldmann 205Kg CT and Gantry Hoist (Hoist and Spreader Bar Only)

Features
Ceiling track hoist system with spreader bar for safe transfers.

Technical specification

- Safe working load: Up to 205 kg

- Control: Handset

- Mobility: Fixed track

- Materials: Aluminium and steel

- Warranty: Manufacturer standard

Safety features

- CE marked

- Emergency lowering

- Wipe clean

Website link: https://media.guldmann.com/cdn/5FPGsqC/datasheet-gh1-gb.pdf

## MOB1619B – Guldmann 255Kg CT and Gantry Hoist (Hoist and Spreader Bar Only)

Features
Heavy-duty ceiling track hoist system for bariatric users.

Technical specification

- Safe working load: 255 kg

- Control: Handset

- Mobility: Fixed track

- Materials: Aluminium and steel

- Warranty: Manufacturer standard

Safety features

- CE marked

- Emergency lowering

- Wipe clean

Website link: https://media.guldmann.com/cdn/5FPGsqC/datasheet-gh1-gb.pdf

- MOB1636 - Superfine 4-way in-bed system BASE sheet 90Wx200L

A fitted in-bed movement sheet designed to help users reposition more easily while lying down. It uses a smooth sliding surface in multiple directions to reduce friction, making turning, sliding, and adjusting position easier for both independent users and carers. The sheet is intended to stay on the bed and be used as part of a sliding system for everyday care and movement support.

Specifications

- Size: 90 cm × 200 cm

- Mattress fit: Single bed

Website Link: Superfine® In-bed System – Lisclare

- MOB1637 - Superfine 4-way in-bed system TOP sheet 190Wx140L

A sliding top sheet used as part of an in-bed movement system to help users reposition more easily. It works with a base sheet to reduce friction, allowing smoother turning and movement in bed for both users and carers. It is placed on top of the user and used to assist safe repositioning without needing to lift the person.

Specifications

- Size: 190 cm × 140 cm

- Material: Superfine low-friction fabric

Website Link: Superfine® In-bed System – Lisclare

- MOB1638 - Bariatric Superfine 4-way in-bed system top and base sheet pack

A bariatric in-bed movement system designed to help with safe repositioning in bed. It uses a low-friction base and top sheet that work together to reduce effort when turning or adjusting a person’s position.

The system is designed for larger users and provides smoother movement for both independent use and assisted care.

Specifications

- Product code: 2623002

- Base sheet size: 120 cm × 200 cm

- Top sheet size: 220 cm × 140 cm

- System type: 4-way sliding in-bed system

- Material: Superfine low-friction fabric

- Use: Bariatric patient handling support

Website Link: Superfine® In-bed System – Lisclare

- MOB1639 - Flat Slide Sheet without handles 100x200

Reusable low-friction sheet used to assist with repositioning and transfers of patients in bed. Designed to reduce friction between the patient and bed surface to support safer manual handling.

Safety Information

- Latex free

- Intended for use as part of a manual handling system

- Requires appropriate risk assessment before use

- Low-profile seams to reduce pressure points

Technical Specification

- Size: 100 cm × 200 cm

- Material: Silicone-coated on both sides

- Construction: Reusable, reinforced textile

- Cleaning: Suitable for repeated hospital laundering

- Storage: Integrated loop hanger

- Labelling: GS1 compliant patient label

- Pack quantity: 10 per unit of issue

Website Link: Washable Slide Sheet - GBUK

- MOB1640 - Pair Flat Slide Sheets without Handles 140x200

A reusable pair of flat slide sheets used to assist with repositioning and transferring patients in bed. Designed to reduce friction and support-controlled movement during manual handling tasks such as turning or sliding a patient. Used in pairs as part of a patient handling system. Not intended for lifting.

Safety Information

- Not for lifting use

- Latex free

- Must be used following a manual handling risk assessment

- Should not be left loose where they may create a slip hazard

- Regular inspection required; remove from use if damaged or worn

Technical Specification

- Product code: PM-FS3

- Size: 140 cm × 200 cm

- Configuration: Supplied as a pair

- Material: Silicone-coated nylon (70 denier)

- Surface: Low-friction, double-sided coating

- Handles: None

- Cleaning: Machine washable (standard healthcare laundering cycles)

- Disinfection: Suitable for thermal disinfection cycles (per guidelines)

- Drying: Air dry only (do not tumble dry)

- Durability: Approx. service life up to 3 years with correct care

- Storage: Flat storage recommended; keep dry and clean between uses

Website Link: Prism Flat Sheets

## MOB2001A – SIBA Satin In-Bed Turner 4Way Draw Sheet With Handles 200 × 140cm

Features
Satin draw sheet with handles for repositioning and turning users in bed.

Technical specification

- Dimensions: 200 × 140 cm

- Safe working load: 400 kg

- Mobility: Manual repositioning

- Materials: Satin polyester with reinforced handles

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Machine washable

Website link: https://www.hospitaldirect.co.uk/product/satin-siba-sheets-4-way-top-draw-sheet/

- OB934 – PAIR of Washable Flat Sheets 90 × 200 cm

Flat slide sheet designed to assist with repositioning, turning, and lateral transfers of patients in bed or on trolleys. The sheet has a low-friction surface to reduce resistance during movement and a soft upper surface for patient comfort. Suitable for use in clinical and community settings and can remain in situ where appropriate.

Technical specification

- Dimensions: 90 cm x 200 cm

- User weight capacity: Not specified (dependent on safe use and handling guidelines)

- Safe working load: Not specified

- Mobility: Not applicable (manual handling aid)

- Materials: Low-friction technical fabric with soft upper surface

- Reusable: Yes

- Washing instructions: 40–74°C

- Drying: Low temperature tumble dry

- Infection control: Do not use bleach

- Warranty: Manufacturer standard

Safety features

- Reduces friction during repositioning to minimise manual handling risk

- Supports safer patient handling when used in line with moving and handling guidance

- Can remain under patient to reduce repeated handling

- Washable for infection control compliance

- Suitable for use across healthcare environments including wards, theatres, and community care

- CE marked

Website link: https://www.hospitaldirect.co.uk/product/flat-sheet-without-handles-washable-slideezi/

- OXF-PRESENCE-S - Oxford Presence Hoist

A mobile hoist used for lifting and transferring patients between surfaces. It supports a range of transfer tasks and can be used in different environments. The hoist includes powered leg positioning for access around furniture and a control system to monitor operation.

Technical Specification

- Product: Oxford Presence Mobile Hoist

- Safe working load: 227 kg

- Lifting range: 430 mm to 1910 mm

- Overall dimensions (max): 1150 × 1460 × 2065 mm (W × D × H)

- Overall dimensions (min): 670 × 1400 × 1460 mm (W × D × H)

- Leg positioning: Powered

- Control system: Smart Monitor

- Attachment options: Interchangeable spreader bar, cradle and weigh-scale

- Turning radius: 1630 mm

- Castors: Front twin castors and rear braked castors

Safety Information

- Do not exceed safe working load (227 kg)

- Ensure correct use during lifting and transfer

- Check components before use

- Do not use if damaged

- Use safety systems provided during operation

Website Link: Joerns | Oxford Presence Mobile Hoist | Powered Leg Adjustment | 227kg - Adapt Life

- PEAMOB041 - Slide Sheet Small

A tubular roller slide sheet designed to assist with repositioning individuals in bed or on a trolley. It reduces friction to enable lateral movement, turning, and repositioning during manual handling tasks.

Technical Specification

- Type: Tubular roller slide sheet

- Material: Low-friction synthetic fabric (Slideezi®)

- Construction: Double-layer tubular design

- Dimensions: 70 cm × 72 cm

- Weight capacity: Suitable for all patient weights (subject to risk assessment)

- Reusable: Yes

- Washing: 40°C to 74°C

- Drying: Low-temperature tumble dry

- Sizes available: Multiple size options

Safety Information

- For use by trained personnel following manual handling guidelines

- Perform a risk assessment before use

- Do not use if fabric is damaged or worn

- Do not use with bleach or harsh chemicals

- Ensure correct positioning to avoid patient discomfort or skin shear

- Not intended for lifting; for repositioning only

Website Link: Tubular Roller Slide Sheets – Washable Slideezi® | Hospital Direct

# Paediatric

## PPEABAT050 - Otter bath chair - Small

A paediatric bathing chair designed to support children with limited mobility during bathing or showering. The seat and backrest can be adjusted in angle to support positioning during use. The chair includes a removable water-permeable cover and a plastic frame designed for wet environments. It is fitted with safety straps and adjustable head, and body supports to help maintain positioning. The unit can be used in bathtubs or showers and can be folded for storage when not in use. Optional bases are available for different bathing setups.

Technical specification

- Max user load: 30 kg

- Weight (with accessories): 5.1 kg

- Seat width: 41 cm

- Seat depth: 30 cm

- Seat height: 14–26 cm

- Seat angle: 90°

- Backrest angle: 0°–90°

- Backrest height: 40 cm

- Backrest width: 41 cm

- Max body size: 90 cm

Folded dimensions:

- Height: 15 cm

- Length: 90 cm

- Width: 42.5 cm

Safety information

- Do not exceed 30 kg maximum load

- Use only under supervision

- Ensure straps and supports are secured before use

- Check all adjustments are locked before use

- Use on stable bathing or shower surfaces

Website Link: Drive DeVilbiss children's bathing system Otter - safe bathing fun for children, size S

- PEABAT051 - Otter Bath Chair - Medium

A paediatric bathing chair designed to support children with limited mobility during bathing or showering. The seat and backrest can be adjusted in angle to support positioning during use. The chair includes a removable water-permeable cover and a plastic frame designed for wet environments. It is fitted with safety straps and adjustable head, and body supports to help maintain positioning. The unit can be used in bathtubs or showers and can be folded for storage when not in use. Optional bases are available for different bathing setups.

Technical specification

- Max user load: 60 kg

- Weight (with accessories): 5.2 kg

- Seat width: 41 cm

- Seat depth: 35 cm

- Seat height: 14–26 cm

- Seat angle: 90°

- Backrest angle: 0°–90°

- Backrest height: 56 cm

- Backrest width: 41 cm

- Max body size: 80–130 cm

Folded dimensions:

- Height: 15 cm

- Length: 105 cm

- Width: 42.5 cm

Safety information

- Do not exceed 60kg maximum load

- Use only under supervision

- Ensure straps and supports are secured before use

- Check all adjustments are locked before use

- Use on stable bathing or shower surfaces

Website Link: Drive DeVilbiss children's bathing system Otter - safe bathing fun for children, size M

- PEABAT082 - Penguin Bath Seat R82

A bathing seat designed for use in bathtubs to support children during washing. It provides postural support for the head and trunk while allowing movement of the arms and legs. The seat is designed to be positioned inside compatible bathtubs.

Features

- Head and trunk support during bathing

- Open design allowing movement of arms and legs

- Adjustable abduction block for positioning

- Secured in bathtub using suction cups

- One size design

Technical specification

- Item number: 87702

- Length: 800 mm

- Height: 260 mm

- Internal width: 330 mm

Website Link: Penguin seat

- PEAMOB216B – Crocodile Size 1

Features
The R82 Crocodile Size 1 is a paediatric posterior walking aid designed to support early-stage independent mobility in children with physical limitations. It provides a stable framework that promotes upright posture and controlled gait development. The lightweight and foldable structure allows for routine use in indoor environments and on smooth external surfaces. Adjustable components enable configuration to suit individual user requirements, supporting progressive development and functional mobility.

Technical Specification

- Height to handles: 420–710 mm

- Height to handles (with starter kit): 380–540 mm

- Overall length: 720 mm

- Overall width: 630 mm

- Folded dimensions (L×W×H): 720 × 620 × 300 mm

- Device weight: 6.4 kg

- Maximum user height: 1150 mm

- Maximum user mass: 30 kg

- Maximum rated load: 38.4 kg

- Turning diameter: 1600 mm

- Wheel size:

  - Large castor: Ø200 mm

  - Small castor: Ø140 mm

- Width between handles: 190–350 mm

Safety Features

- Anti-reverse wheel stops to prevent unintended backward movement

- Adjustable rear wheel resistance for controlled gait speed

- Optional anti-tip devices to improve stability

- Stable frame geometry to support upright positioning

- Hand brake compatibility for additional control

- Foldable locking mechanism for secure transport and storage

- CE marked

- Braked rear wheels

- Wipe clean

Website link: R82 Crocodile

- PEAMOB216C – Crocodile Size 2

Features
The R82 Crocodile Size 2 is a mid-range paediatric walking aid intended for children requiring continued mobility support as they grow. It facilitates gait training through a posterior support design that encourages natural walking patterns. The frame incorporates adjustable height and handle positioning, allowing adaptation to the user’s physical development. It is suitable for indoor use and level outdoor environments, offering consistent manoeuvrability and stability.

Technical Specification

- Height to handles: 560–850 mm

- Overall length: 840 mm

- Overall width: 690 mm

- Folded dimensions (L×W×H): 860 × 690 × 270 mm

- Device weight: 6.8 kg

- Maximum user height: 1500 mm

- Maximum user mass: 45 kg

- Maximum rated load: 53.8 kg

- Turning diameter: 1900 mm

- Wheel size:

  - Large castor: Ø200 mm

  - Small castor: Ø140 mm

- Width between handles: 240–420 mm

Safety Features

- Anti-reverse wheel stops to limit unintended rearward rolling

- Adjustable rolling resistance for speed control

- Compatibility with fixed or swing-away anti-tip systems

- Wide base frame for improved lateral stability

- Optional braking systems for user or attendant control

- Durable construction designed for repeated daily use

- CE marked

- Braked rear wheels

- Wipe clean

Website link: R82 Crocodile

- PEAMOB216D – Crocodile Size 3

Features
The R82 Crocodile Size 3 is a larger paediatric walking aid designed for older children requiring higher levels of support and load capacity. It maintains a posterior walking configuration to encourage upright posture and functional gait patterns. The increased frame dimensions and handle range accommodate taller users, while maintaining manoeuvrability and structural stability. It is intended for both indoor use and smooth outdoor environments.

Technical Specification

- Height to handles: 740–1025 mm

- Overall length: 870 mm

- Overall width: 690 mm

- Folded dimensions (L×W×H): 1000 × 690 × 290 mm

- Device weight: 11.3 kg

- Maximum user height: 1800 mm

- Maximum user mass: 80 kg

- Maximum rated load: 93.3 kg

- Turning diameter: 2000 mm

- Wheel size:

  - Large castor: Ø200 mm

  - Small castor: Ø140 mm

- Width between handles: 330–480 mm

Safety Features

- Anti-reverse wheel stop mechanism to prevent backward roll

- Adjustable resistance on rear wheels for controlled movement

- Anti-tip options to reduce risk of rearward instability

- Increased frame size for improved weight distribution and balance

- Brake system compatibility for enhanced control

- Secure folding mechanism for safe handling during storage and transport

- CE marked

- Braked rear wheels

- Wipe clean

Website link: R82 Crocodile

- PEAMOB217 – Hip Supports for Crocodile – Size 1 – Small

Features
The R82 Crocodile Back and Side Support (Small) is a positioning accessory designed to help support the lower back and pelvis when used with compatible Crocodile walking frames. It provides gentle support from behind and the sides to help keep the user sitting in a more stable and upright position. The width can be adjusted to suit the user for a closer and more comfortable fit.

Technical Specification

- Compatible with: R82 Crocodile Size 0, 1, 2, and 3

- Height: 90 mm

- Width adjustment range: 120–230 mm

- Type: Back and side support

- Size: 1 (Small)

Safety Features

- Helps support the user’s lower back and pelvis

- Reduces leaning to one side while seated

- Adjustable width allows a better individual fit

- Supports a more stable sitting position during use

- Smooth padded design helps reduce discomfort

- Designed to stay securely in position during activity

- CE marked

- Wipe clean

Website link: Back and side support

## PEAMOB217A – Hip Supports for Crocodile – Size 2 – Large

Features
The R82 Crocodile Back and Side Support (Large) is designed to provide additional support for the lower back and pelvis when using compatible Crocodile walking frames. It gives gentle support from behind and the sides to help keep the user sitting upright and stable. The width can be adjusted to match the user’s body size for improved comfort and positioning.

Technical Specification

- Compatible with: R82 Crocodile Size 1, 2, and 3

- Height: 90 mm

- Width adjustment range: 180–290 mm

- Type: Back and side support

- Size: 2 (Large)

Safety Features

- Supports the lower back and pelvis to improve sitting position

- Helps reduce sideways leaning

- Adjustable width for a more secure fit

- Promotes a stable seated posture during use

- Padded surface helps reduce pressure and discomfort

- Designed to remain securely attached during movement

- CE marked

- Wipe clean

Website link: Back and side support

## PEAMOB217B – Hip/Back Supports for Crocodile – Size 3

Features
The R82 Back Support (Size 3) is a padded support designed to help keep a user sitting upright when used with a compatible Crocodile walking frame. It fits around the lower back and hips to give gentle support from behind and slightly from the sides. It is adjustable in width so it can be set to fit the user more closely and comfortably during use.

Technical Specification

- Compatible with: R82 Crocodile Size 3

- Back support height: 7 in (approx. 178 mm)

- Width adjustment: 9.5–11.5 in (approx. 241–292 mm)

- Type: Padded back support

- Adjustment: Width adjustable

- Dimensions: Size 3

- Product weight: Approx. 1.3 kg

- User weight capacity: Matches host aid

- Safe working load: As per host system

- Control: Manual attachment

- Mobility: Static

- Materials: Foam padding, Velcro straps

- Warranty: Manufacturer standard

Safety features

- Helps keep the user sitting upright

- Provides gentle support to reduce leaning to one side

- Curved shape helps hold the lower back and hips in position

- Adjustable width helps achieve a secure and comfortable fit

- Padded surface helps reduce pressure and discomfort

- Designed to stay securely in place during use

- CE marked

- Wipe clean

Website link: https://www.r82.org/products/crocodile/

## PEAMOB218A - Crocodile Walker Seat - Size 2

A paediatric gait trainer designed to support children with reduced mobility during walking. The frame provides postural support and assists with balance and movement. Size 2 is intended for larger children within the Crocodile range. The frame is height adjustable, foldable, and can be adapted with accessories for individual positioning needs. It is suitable for indoor use and outdoor use on even surfaces.

Technical specification

- Item number: 86823

- Mass device: 6.8 kg

- Maximum user mass: 45 kg

- Maximum rated load: 53.8 kg

- Maximum user height: 1500 m

- Height from floor to handles: 560–850 mm

- Width between handles: 240–420 mm

- Overall width: 690 mm

- Length: 840 mm

- Turning diameter: 1900 mm

Folded dimensions:

- Height: 270 mm

- Length: 860 mm

- Width: 690 mm

Wheel type: Small castors 28 mm / 140 mm, large castors 30 mm / 200 mm

Stability:

- Downhill: 15°

- Uphill: 7°

- Sideways: 4.5°

Safety information

Do not exceed 45 kg maximum user mass

Website Link: R82 Crocodile

## PEAMOB218B - Crocodile Walker Seat - Size 3

A paediatric gait trainer designed to support children with reduced mobility during standing and walking. The frame provides postural support and assists with balance and controlled movement. Size 3 is intended for older or larger children within the Crocodile range. The frame is height adjustable, foldable, and can be adapted with positioning accessories. Suitable for indoor use and outdoor use on even surfaces.

Technical specification

- Item number: 86863

- Mass device: 11.3 kg

- Maximum user mass: 80 kg

- Maximum rated load: 93.3 kg

- Maximum user height: 1800 mm

- Height from floor to handles: 740–1025 mm

- Width between handles: 330–480 m

- Overall width: 690 mm

- Length: 870 mm

- Turning diameter: 2000 mm

Folded dimensions:

- Height: 290 mm

- Length: 1000 mm

- Width: 690 mm

Wheel type: Small castors 28 mm / 140 mm, large castors 30 mm / 200 mm

Stability:

- Downhill: 15°

- Uphill: 7°

- Sideways: 4.5°

Safety information

Do not exceed 80 kg maximum user mass

Website Link: R82 Crocodile

## PEAMOB219 – Forearm Supports Size 1

Features
The R82 Crocodile Forearm Support with Handgrip (Size 1) is an accessory designed to support the user’s arms while using a compatible Crocodile walking frame. It helps position the forearms and hands to improve stability and control during movement. The support includes a forearm platform and a handgrip for holding during use.

Technical specification

- Dimensions: Size 1

- Product weight: Approx. 1 kg per pair

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual adjustment

- Mobility: Static

- Length: 150 mm

- Width: 80 mm

- Materials: Metal frame with padded support

- Warranty: Manufacturer standard

Safety features

- Supports the forearms to improve upper body stability

- Handgrip provides a secure place to hold

- Helps keep arms in a consistent position during use

- Reduces strain on hands and wrists

- Secure attachment to compatible walking frame

- Smooth surfaces to reduce risk of discomfort

- CE marked

- Wipe clean

Website link: Forearm support with handgrip

## PEAMOB219A – Forearm Supports Size 2

Features
The R82 Crocodile Forearm Support with Handgrip (Size 2) is an accessory designed to support the user’s arms while using a compatible Crocodile walking frame. It helps position the forearms and hands to improve stability and control during movement. The support includes a forearm platform and a handgrip for holding during use.

Technical specification

- Dimensions: Size 2

- Product weight: Approx. 1.2 kg per pair

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual adjustment

- Mobility: Static

- Length: 180 mm

- Width: 100 mm

- Materials: Metal frame with padded support

- Warranty: Manufacturer standard

Safety features

- Supports the forearms to help maintain stability

- Handgrip allows a secure hold during use

- Helps keep arms positioned correctly

- Reduces pressure on hands and wrists

- Attaches securely to the walking frame

- Smooth finish to reduce discomfort

- CE marked

- Wipe clean

Website link: Forearm support with handgrip

## PEAMOB219B – Forearm Supports Size 3

Features
The R82 Crocodile Forearm Support with Handgrip (Size 3) is an accessory designed to support the user’s arms while using a compatible Crocodile walking frame. It helps position the forearms and hands to improve stability and control during movement. The support includes a forearm platform and a handgrip for holding during use.

Technical specification

- Dimensions: Size 3

- Product weight: Approx. 1.3 kg per pair

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual adjustment

- Mobility: Static

- Materials: Metal frame with padded support

- Warranty: Manufacturer standard

- Length: 180 mm

- Width: 100 mm

Safety features

- CE marked

- Wipe clean

Website link: Forearm support with handgrip

## PEASEA002 - Jenx Corner Seat with straps

The Corner Seat Size 1 is a paediatric positioning seat designed to support early sitting in children approximately 6 months to 2 years. It provides postural stability in long sitting by promoting midline alignment of the trunk and pelvis. The corner design offers lateral support, while the adjustable backrest allows modification of height to suit the child’s growth. An abduction block helps maintain appropriate hip positioning. The seat can be used with or without upper back support depending on clinical need and can be paired with compatible accessories for additional positioning or activity support.

Technical Specification

- Approx. Age Range: 6 months – 2 years

- Maximum User Weight: 25 kg

- Back Pad Height (seat to top): 315 – 425 mm

- Seat Depth: 350 mm

- Base Size (Footprint): 500 – 500 mm

Safety

- Ensure the child is always supervised while in use.

- Positioning straps and supports must be correctly fitted and adjusted, allowing minimal clearance (approximately one finger width).

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## PEASEA003 - Corner Seat Size 2

The Corner Seat Size 2 is a paediatric positioning seat designed to support sitting in children approximately 1 to 6 years. It promotes postural stability in long sitting by maintaining midline alignment of the trunk and pelvis. The corner configuration provides consistent lateral support, while the adjustable backrest allows height modification to accommodate growth. An abduction block assists with lower limb positioning and pelvic alignment. The seat can be configured with or without upper back support depending on clinical requirements and can be used with compatible accessories for additional support and function.

Technical Specification

- Approx. Age Range: 1 – 6 years

- Maximum User Weight: 35 kg

- Back Pad Height (seat to top): 440 – 620 mm

- Seat Depth: 370 mm

- Base Size (Footprint): 540 – 565 mm

Safety

- The child must be supervised at all times during use.

- All positioning straps and supports should be correctly fitted and adjusted, allowing minimal clearance (approximately one finger width).

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## PEASEA004 – Activity Table Size 1

Features
The Jenx Nursery Table Size 1 is a paediatric support surface designed for therapeutic, educational, and daily activity use. It provides a stable platform for children requiring postural support during seated or standing activities. The table is height adjustable and allows angular positioning to facilitate engagement, positioning, and accessibility. It is suitable for use in clinical, educational, and home environments when paired with compatible seating systems.

Technical Specification

- Table size (L × W): 510 × 605 mm

- Table height (topside): 250 – 345 mm

- Table angle (from horizontal): 40° recline to 40° forward tilt

- Maximum load weight: 20 kg

- Distance between table legs (width): 570 mm

Safety Features

- Stable base frame to reduce risk of tipping during use

- Adjustable height to support correct positioning and minimise strain

- Controlled tilt mechanism for safe angular adjustment

- Load rating defined to prevent overloading

- Wide leg spacing to improve stability and accommodate seating systems

- Smooth surface edges to reduce risk of contact injury

- CE marked

- Wipe clean

Website link: https://jenx.com/nursery-table/

## PEASEA004B – Activity Table Size 2

Features
The Jenx Nursery Table Size 2 is a larger paediatric table designed to support children requiring additional space and higher positioning ranges. It provides a structured surface for therapy, learning, and daily activities, with adjustable height and tilt to accommodate user needs. The design supports integration with postural seating systems and enables consistent positioning for functional activities.

Technical Specification

- Table size (L × W): 510 × 705 mm

- Table height (topside): 290 – 440 mm

- Table angle (from horizontal): 40° recline to 40° forward tilt

- Maximum load weight: 20 kg

- Distance between table legs (width): 670 mm

Safety Features

- Wide support base to enhance stability during use

- Adjustable height range to maintain safe and appropriate positioning

- Secure tilt adjustment mechanism to control table angle

- Defined maximum load capacity to reduce structural risk

- Increased leg spacing for stability and compatibility with larger seating systems

- Rounded edges to minimise risk of injury during contact

- CE marked

- Wipe clean

Website link: https://jenx.com/nursery-table/

## PEASEA125 – Leckey Activity System – Vida (previously Leckey)

Features

The Squiggles Early Activity System is a modular early intervention positioning set designed for children from birth to 36 months. It provides configurable support for a range of therapeutic positions including supine, prone, side lying, sitting, and four-point kneeling. The system uses a floor mat with interchangeable supports such as wedges, rolls, and straps to assist with head, trunk, and limb positioning during activity and therapy. It is intended for use in home and clinical environments as part of early postural management and developmental support programmes.

Technical specification

- Maximum user weight: 15 kg (2 st 5 lb)

- Maximum user height: 1000 mm (39.4")

- Floor mat: 1760 × 950 mm

- Sitting support: 190 × 300 mm (seat width 200–250 mm range)

- Small roll: 60 × 600 mm

- Medium roll: 80 × 700 mm

- Trunk wedge: 270 × 208 mm, 7.5° angle

- Positioning straps: 80 × 660 mm

- Includes mat, supports, straps, rolls, and carry bag

Safety

- Must not be used without supervision

- Must be set up by a trained professional

- Use only on a flat, stable surface with the supplied mat

- Do not modify components or use non-approved parts

- Inspect regularly for wear or damage

- For indoor use only

- Keep away from heat sources and open flame

- Clean and dry before use; follow manufacturer cleaning guidance

Website link: Early Activity System | Leckey

- PEASEA140 - Heathfield seat with tray size 1 and Skis

The Heathfield Chair Size 1 is a paediatric seating system designed for children from approximately 2 years who require low to moderate postural support in sitting. It provides a stable and supportive seating position suitable for use in home, school, or clinical environments. The chair features a simple, durable wooden frame with adjustable arm components to accommodate growth and positioning needs. When configured with a tray, it supports functional activities such as feeding, play, and learning. The addition of skis increases base stability and reduces the risk of tipping, particularly during active use. The system can be adapted with accessories to meet individual postural requirements.

Technical Specification

- Size: 1

- Maximum User Weight: 25 kg

- Seat Height: 180 – 230 mm

- Seat Width: 240 mm

- Seat Depth: 180 – 240 mm

- Back Height: 250 mm

- Product Weight: 5 kg

Safety

- Ensure the child is supervised at all times while seated.

- Confirm the chair is stable on a flat, even surface; skis should be correctly fitted to enhance stability.

- All accessories (including tray and supports) must be securely attached before use.

- Positioning aids and straps, where used, should be correctly adjusted with minimal clearance.

- Do not exceed the maximum user weight.

Website Link: The Heathfield chair for mild postural support - Smirthwaite

- PEASEA141 - Heathfield seat with tray size 2 and Skis

The Heathfield Chair Size 2 is a paediatric seating system designed for children who require low to moderate postural support in sitting. It provides a stable and supportive seating position suitable for use in home, school, or clinical environments. The chair features adjustable seating dimensions to accommodate growth and positioning needs. When used with a tray, it supports functional activities such as feeding, play, and learning. The addition of skis increases the base of support, improving overall stability during use.

Technical Specification

- Size: 2

- Seat Height: 200 – 300 mm

- Seat Width: 280 mm

- Seat Depth: 220 – 310 mm

- Back Height: 280 mm

- Maximum User Weight: 45 kg

- Product Weight: 6 kg

Safety

- Ensure the child is supervised at all times while seated.

- Confirm the chair is positioned on a stable, level surface; skis must be securely fitted.

- All accessories (including tray and supports) should be correctly attached before use.

- Any positioning straps or supports must be appropriately adjusted.

- Do not exceed the stated maximum user weight.

Website Link: The Heathfield chair for mild postural support - Smirthwaite

## PEASEA194 – Medium Wombat Head Support Size 1, 2, 3 and Xpanda Size 1, 2, 3

Features
Adjustable head support for Wombat and X:Panda seating systems, medium size.

Technical specification

- Dimensions: Medium size

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual adjustment

- Mobility: Static

- Materials: Foam, vinyl cover, metal frame

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/anatomic-head-support-ee54313b/

## PEASEA195A – Wombat Hip Support for Size 1 and 2

Features
The R82 Wombat Hip Supports (Size 1 + 2) are accessories used to make the seat narrower and help keep the user sitting in a stable and upright position. They are fitted to compatible Wombat chairs and help keep the hips in place during everyday activities such as sitting, learning, or playing.

Technical Specification

- Compatible with: R82 Wombat Solo and Wombat Living (Sizes 1 and 2)

- Height: 80 mm

- Width: 190 mm

- Function: Reduces seat width and supports hip position

Safety Features

- Helps keep hips in a stable position while seated

- Reduces side-to-side movement in the chair

- Attaches securely to the chair

- Small size allows easier transfers in and out of the seat

- Smooth surfaces to reduce risk of discomfort

- Can be positioned to suit the user

- CE marked

- Wipe clean

Website link: Hip supports

## PEASEA195B – Wombat Hip Support for Size 3

Features
The R82 Wombat Hip Supports (Size 3) are designed to reduce seat width and help keep larger users stable while seated. They support the hips to improve sitting position and are used with compatible Wombat chairs in both daily and care settings.

Technical Specification

- Compatible with: R82 Wombat Solo and Wombat Living (Size 3)

- Height: 100 mm

- Width: 250 mm

- Function: Reduces seat width and supports hip position

Safety Features

- Helps keep the user steady in the chair

- Limits side-to-side movement

- Securely attaches to the seating system

- Larger size provides support for bigger users

- Smooth finish to reduce discomfort

- Adjustable position to suit individual needs

- CE marked

- Wipe clean

Website link: Hip supports

## PEASEA196 – Wombat Knee Abduction Pommel for Size 1, 2, 3

Features
Knee abduction pommel designed to reduce hip adduction and assist with maintaining lower limb alignment during sitting. Helps prevent forward sliding in the chair and supports a stable seated position. Suitable for paediatric users requiring additional positioning support. Compatible with R82 x:panda seating systems.

Technical specification

- Size: Small

- Item number: 8910408-10

- Compatible with: R82 x:panda

- Height: 170 mm

- Length: 170 mm

- Width: 70 mm

- User weight capacity: Dependent on host seating system

- Safe working load: As per chair specification

- Control: Manual fitting

- Mobility: Static

- Materials: Foam, vinyl cover

Safety features

- Maintains lower limb positioning

- Reduces risk of forward sliding

- Provides padded support for pressure distribution

- Secure attachment to seating system

- CE marked

- Wipe clean

Website link: https://www.r82.org/products/wombat-living/

## PEASEA199B – Wombat 1, 2 and 3 Fixed Side Supports – Standard Fixing

Features

The R82 Wombat Fixed Side Supports are seating accessories designed to provide additional side support for users seated in compatible Wombat chairs. They help keep the upper body in a stable and upright position during daily activities such as eating, learning, or playing. The supports are fixed in place and are suitable for use in both home and care environments.

Technical Specification

- Compatible with: R82 Wombat Solo and Wombat Living

- Depth: 110 mm

- Height: 150 mm

- Size: One size

Safety Features

- Provides side support to help maintain an upright sitting position

- Reduces side-to-side movement while seated

- Fixed design ensures consistent positioning

- Secure attachment to compatible seating systems

- Smooth surfaces to reduce risk of discomfort

- Designed for stable use during everyday activities

- CE marked

- Wipe clean

Website link: Side supports, fixed

## PEASEA199C – Swing Away Side Supports for Wombat 1, 2, 3

Features
Side supports for Wombat seating systems that swing away for easy transfers.

Technical specification

- Dimensions: Compatible with Wombat sizes 1–3

- Product weight: Approx. 1.5 kg per pair

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual swing mechanism

- Mobility: Static

- Materials: Foam padding, metal brackets

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: https://www.r82.org/products/wombat-living/

## PEASEA208 – X:Panda Swing Away Side Supports

Features
Side supports for X:Panda seating systems that swing away for easier transfers.

Technical specification

- Dimensions: Compatible with all X:Panda sizes

- Product weight: Approx. 1.5 kg per pair

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual swing mechanism

- Mobility: Static

- Materials: Foam padding, metal brackets

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: https://www.r82.org/products/xpanda/

## PEASEA213D – Hip Supports for X-Panda Size 4

Features
Adjustable hip supports for X:Panda Size 4 seating systems to enhance postural alignment.

Technical specification

- Dimensions: Size 4

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual adjustment

- Materials: Padded foam, metal brackets

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/#accessories-anchor

## PEASEA142 – X:PANDA Size 1

Features
Modular seating system for children with postural needs, adjustable in multiple dimensions.

Technical specification

- Dimensions: Size 1 seat base

- User weight capacity: Up to 30 kg

- Safe working load: 38.5kg

- Control: Manual adjustment

- Mobility: Static or mobile base options

- Materials: Aluminium frame, padded upholstery

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/

## PEASEA143 – X:Panda Size 2

Features
Same as Size 1, with larger dimensions for growing children.

Technical specification

- Dimensions: Size 2 seat base

- Product weight: Approx. 12–14 kg

- User weight capacity: 30kg

- Safe working load: 40.5kg

- Control: Manual adjustment

- Mobility: Static or mobile base options

- Materials: Aluminium frame, padded upholstery

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/

## PEASEA144A – X:Panda Size 3 with Hydraulic Hi-Low Base

Features
Postural seating system with hydraulic height-adjustable base for ease of transfers and therapy.

Technical specification

- Dimensions: Size 3 seat with Hi-Low base

- User weight capacity: Up to 60 kg

- Safe working load: 73 kg

- Control: Hydraulic height adjustment

- Mobility: Mobile base with lockable castors

- Materials: Aluminium, steel, padded upholstery

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/

## PEASEA145 – X:Panda Size 4 with Strong Base

Features
Largest size in the X:Panda range with reinforced base for older children or young adults.

Technical specification

- Dimensions: Size 4 seat with strong base

- User weight capacity: Up to 80 kg

- Safe working load: 95 kg

- Control: Manual adjustment

- Mobility: Static or mobile base

- Materials: Reinforced aluminium, padded upholstery

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean upholstery

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/

## PEASEA191A – Wombat Size 1

Features
The R82 Wombat Living Size 1 is a highly adaptable activity chair designed for children, offering long-term support through built-in growth and extensive adjustability. The chair features a lightweight yet robust frame with height adjustment and tilt-in-space functionality to promote active sitting and ease of transfers. It is designed to support good posture, reduce pressure points, and provide comfort throughout the day. With configurable seating, adjustable supports, and a wide range of accessories, the chair can be tailored to meet individual clinical and functional needs in both home and educational environments.

Technical specification

- Size: 1

- Seat width: 300 mm

- Seat depth: 180 – 310 mm

- Backrest height: 280 – 340 mm (standard) / 360 – 430 mm (high back)

- Seat height range: approx. 360 – 650 mm (depending on castors)

- Overall width: 550 mm

- Overall length: 600 mm

- Product weight: 15.5 kg

- User weight capacity: Up to 45 kg

- Safe working load: 45 kg

- Tilt-in-space: approx. -15° to +20°

- Backrest angle: 90° to 118°

- Control: Manual or optional powered height adjustment

- Mobility: Mobile (castors with brakes)

- Materials: Aluminium/steel frame with foam cushions

- Warranty: Manufacturer standard

Safety features

- Lockable castors for stability during transfers

- Tilt and posture support to reduce pressure risk

- Adjustable supports for positioning and safety

- Optional pelvic, hip, and side supports

- Wipe-clean, durable surfaces

- CE marked

- Wipe clean upholstery

Website link: R82-Wombat-Living.pdf

## PEASEA192A – Wombat Size 2

Features
The R82 Wombat Living Size 2 is an adjustable activity chair designed for growing children and young people, combining ergonomic support with flexibility. It offers height adjustment, tilt-in-space positioning, and configurable seating to encourage active sitting and independence. The chair supports correct posture while allowing for movement and interaction, making it suitable for daily activities such as learning, play, and mealtimes. Its modular design allows for a wide range of accessories and adjustments to meet changing needs over time.

Technical specification

- Size: 2

- Seat width: 370 mm

- Seat depth: 250 – 380 mm

- Backrest height: 310 – 380 mm

- Seat height range: approx. 360 – 650 mm

- Overall width: 550 mm

- Overall length: 600 mm

- Product weight: 15.5 kg

- User weight capacity: Up to 45 kg

- Safe working load: 45 kg

- Tilt-in-space: approx. -15° to +20°

- Backrest angle: 90° to 118°

- Control: Manual or optional powered height adjustment

- Mobility: Mobile (castors with brakes)

- Materials: Aluminium/steel frame with foam cushions

- Warranty: Manufacturer standard

Safety features

- Lockable castors for secure positioning

- Postural support to maintain safe seating alignment

- Adjustable components to reduce risk of pressure points

- Compatible with multiple positioning supports

- Easy-clean surfaces for hygiene

- CE marked

- Wipe clean upholstery

Website link: R82-Wombat-Living.pdf

## PEASEA193A – Wombat Size 3

Features
The R82 Wombat Living Size 3 is a larger activity chair designed for older children and teenagers, providing advanced postural support and long-term adaptability. It features a robust adjustable frame with tilt-in-space and height adjustment options to facilitate safe transfers and active positioning. The chair promotes independence and comfort during daily activities, while its modular system allows for tailored configuration with a wide range of accessories to meet complex needs. Suitable for home, school, and care environments.

Technical specification

- Size: 3

- Seat width: 420 mm

- Seat depth: 320 – 500 mm

- Backrest height: 390 – 500 mm

- Seat height range: approx. 390 – 700 mm

- Overall width: 610 mm

- Overall length: 720 mm

- Product weight: 22 kg

- User weight capacity: Up to 70 kg

- Safe working load: 70 kg

- Tilt-in-space: approx. -10° to +22°

- Backrest angle: 90° to 133°

- Control: Manual or optional powered height adjustment

- Mobility: Mobile (castors with brakes)

- Materials: Aluminium/steel frame with foam cushions

- Warranty: Manufacturer standard

Safety features

- Lockable castors for safe transfers

- Advanced postural positioning to reduce strain and pressure

- Adjustable supports for trunk, pelvis, and limbs

- Stable frame for higher user weights

- Wipe-clean and durable materials

- CE marked

- Wipe clean upholstery

Website link: R82-Wombat-Living.pdf

- PEASEA193CH – Wombat Size 3 Charger

Features
Battery charger for Wombat Size 3 seating system with powered features.

Technical specification

- Dimensions: Standard charger unit

- Product weight: Approx. 0.5 kg

- User weight capacity: N/A

- Safe working load: N/A

- Control: Plug-in power supply

- Mobility: Portable

- Materials: Plastic housing, electrical components

- Warranty: Manufacturer standard

Safety features

- CE marked

- Overcharge protection

- Wipe clean

Website link: https://www.r82.org/products/wombat-living/

## PEASEA198A – Wombat and Xpanda Tray for Size 1

Features
Tray accessory for Wombat and Xpanda seating systems, providing a stable surface for activities.

Technical specification

- Dimensions: Size 1 tray

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual attachment

- Mobility: Static

- Materials: Plastic or composite

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/tray-shape-5-with-edge/

## PEASEA198B – Wombat and Xpanda Tray for Size 2 and 3

Features
Tray accessory for Wombat and Xpanda seating systems, sized for Size 2 and 3.

Technical specification

- Dimensions: Size 2 and 3 tray

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual attachment

- Mobility: Static

- Materials: Plastic or composite

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/tray-shape-5-with-edge/

## PEASEA198C – Wombat and Xpanda Tray for Size 4

Features
Tray accessory for Wombat and Xpanda seating systems, sized for Size 4.

Technical specification

- Dimensions: Size 4 tray

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual attachment

- Mobility: Static

- Materials: Plastic or composite

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/tray-shape-6-with-edge/#variants-anchor

## PEASEA200A – Wombat and Xpanda Neoprene Waistcoat Vest for Size 1

Features
Neoprene vest for postural support and safety in Wombat and Xpanda seating systems.

Technical specification

- Dimensions: Size 1

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual fastening

- Mobility: Static

- Materials: Neoprene, Velcro straps

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or hand wash

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/torso-vest/

## PEASEA200B – Wombat and Xpanda Neoprene Waistcoat Vest for Size 2

Features
Same as above, sized for Size 2

Technical specification

- Dimensions: Size 2

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual fastening

- Mobility: Static

- Materials: Neoprene, Velcro straps

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or hand wash

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/torso-vest/

## PEASEA200C – Wombat and Xpanda Neoprene Waistcoat Vest for Size 3

Features 
Same as above, sized for Size 3 users.

Technical specification

- Dimensions: Size 3

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual fastening

- Mobility: Static

- Materials: Neoprene, Velcro straps

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or hand wash

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/torso-vest/

## PEASEA204 – Xpanda Adjustable Arm Support for size 1,2,3

Features

Adjustable upper arm supports provide additional upper limb support. They assist with comfortable positioning and proximity of the arms. The supports are adjustable in both height and width.

Technical specification

- Compatible with: R82 x:panda, R82 Kudu

- Height: 170 mm

- Width: 140 mm

Website link: Adjustable upper arm supports

## PEASEA206 – Xpanda Neoprene Sandals Size 1

Features
Adjustable neoprene sandals for use with X:Panda seating systems, providing foot support.

Technical specification

- Dimensions: Size 1

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Velcro fastening

- Mobility: Static

- Materials: Neoprene, Velcro

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean or hand wash

Website link: https://www.etac.com/en-us/us/products/pediatrics/accessories/neoprene-sandals/

## PEASEA207C – Xpanda Width Growth Pack, BLACK, Size 3 (Large)

Features
Growth pack to increase seat width for X:Panda Size 3 seating systems.

Technical specification

- Dimensions: Size 3 width extension

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual fitting

- Mobility: Static

- Materials: Metal brackets, padded inserts

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/

## PEASEA207D – Xpanda Width Growth Pack, Black, Size 4 (Med)

Features
Same as above, sized for X:Panda Size 4.

Technical specification

- Dimensions: Size 4 width extension

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual fitting

- Mobility: Static

- Materials: Metal brackets, padded inserts

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/

## PEASEA207B – Xpanda Width Growth Pack, BLUE, Size 2 (Large)

Features
Width growth pack for X:Panda Size 2 seating system in blue.

Technical specification

- Dimensions: Size 2 width extension

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual fitting

- Mobility: Static

- Materials: Metal brackets, padded inserts

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/

## PEASEA207A – Xpanda Width Growth Pack, RED, Size 1 (Large)

Features
Width growth pack for X:Panda Size 1 seating system in red.

Technical specification

- Dimensions: Size 1 width extension

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual fitting

- Mobility: Static

- Materials: Metal brackets, padded inserts

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/r82-xpanda/

## PEASEA211A – Ankle Strap for Wombat – Size 1

Features
Adjustable ankle strap for Wombat seating systems, providing lower limb support and positioning.

Technical specification

- Dimensions: Size 1

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual buckle fastening

- Mobility: Static

- Materials: Neoprene, Velcro, plastic buckle

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or hand wash

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/ankle-straps/

## PEASEA211B – Ankle Strap for Wombat – Size 2

Features
Same as above, sized for Wombat Size 2.

Technical specification

- Dimensions: Size 2

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual buckle fastening

- Mobility: Static

- Materials: Neoprene, Velcro, plastic buckle

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or hand wash

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/ankle-straps/

## PEASEA211C – Ankle Strap Size 3, With Buckle

Features
Ankle strap with buckle for Wombat Size 3 seating systems, designed for secure lower limb positioning.

Technical specification

- Dimensions: Size 3

- User weight capacity: Matches host seating

- Safe working load: As per host system

- Control: Manual buckle fastening

- Mobility: Static

- Materials: Neoprene, Velcro, plastic buckle

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or hand wash

Website link: https://www.etac.com/en-gb/uk/products/paediatrics/accessories/ankle-straps/

# Respiratory therapy

## ELEC0015A – Adult Nebuliser Pack – Durable Medix Year Kit

Features
Complete adult nebuliser kit for year-round use, compatible with Medix systems.

Technical specification

- Dimensions: Standard adult kit

- Intended use: Replacement consumables for nebuliser therapy (12-month supply)

- Patient type: Adult

- Compatibility: Suitable for Medix nebulisers including AC2000, AC3000, AC4000, Turboneb, Econoneb, World Traveller

- Not compatible with Actineb, MicroAir, AC1000

Contents:

- 4 × adult face masks

- 4 × nebuliser chambers (cups)

- 4 × Drive Line Tubes

- 1 × mouthpiece with T

- 4 × inlet filters

- 1 × outlet filter

- Total pack weight: approximately 495 g

Safety features

- CE marked

- Washable components

Website link: https://www.hce-uk.com/product/clement-clarke-medix-year-pack-adult/

- ELEC0019B - Vacuaide 7325 Portable Suction Unit

The VacuAide 7325 is a portable suction unit designed to remove secretions from the upper airway. It is intended for use in clinical and home care environments where suction support is required.

The device is compact and battery operated, allowing use without continuous mains power. It provides adjustable suction levels and is designed for short-term, intermittent use. The unit can be used on mains power or its internal battery and is suitable for transport between care settings.

Safety Features:

- Adjustable suction control

- Battery operation for use away from mains power

- Internal battery backup for continuity of use

- Compact housing to reduce handling strain

- Built-in tube holder to reduce disconnection risk

Technical Specification:

- Model: VacuAide 7325 (Item 7325P-I)

- Weight: 1.72 kg (with accessories)

- Dimensions: 178 mm (W) x 175 mm (H) x 180 mm (D)

- Vacuum range: 50–550 mmHg

- Airflow: up to 27 L/min

- Battery runtime: up to 45 minutes (mains independent use)

- Charging time: approx. 180 minutes

- Power supply: 240 V / 60 Hz

- Operating temperature: up to 40°C

- Storage temperature: -40°C to 70°C

- Humidity: up to 95% relative humidity

Website Link: Drive DeVilbiss VacuAide 7325 portable secretion suction pump with rechargeable battery and EU plug

- ELEC0019T– Vacuaide tubing

Patient Tubing (6 ft) for VacuAide Suction Units. Single-use patient tubing for connection to VacuAide suction units. Suitable for use with Drive DeVilbiss suction units and collection canisters.

Technical specification

- Length: 6 ft

- Use: Single patient use

- Compatibility: Drive DeVilbiss suction units and collection canisters

Website link: 6FT Patient Tubing

## ELEC0036AM – Adult Mask and Chamber Kit for ELEC0036ELITE

Features
The Salter Labs® 8900 Small Volume Jet Nebulizer is a respiratory therapy device designed for the delivery of aerosolised medication in clinical and home environments. It utilises jet nebulisation to convert liquid medication into a fine mist suitable for inhalation. The device is configured with a mask interface and safety tubing, supporting consistent drug delivery while accommodating patients who may require treatment in varied positions. Its design enables efficient medication use and integration into standard respiratory care routines, including compatibility with ventilator circuits.

Technical Specification

- Configuration: Nebuliser with mask and 2.1 m (7 ft) safety tubing

- Patient type: Adult

- Nebulisation capacity: 3 cc medication volume

- Delivery time: ≤ 7 minutes (at 7 LPM flow rate)

- Operating flow rate: 7 litres per minute (LPM)

- Operation: Jet nebulisation system

- Orientation: Operable in horizontal and vertical positions

- Pack quantity: 50 units

- Compatibility: Suitable for use in ventilator circuits

Safety Features

- Anti-drool “T” design to reduce risk of contamination from patient saliva

- Leak-resistant screw-on cap to maintain a sealed medication chamber

- Stable base design to allow the unit to stand securely during use

- Orientation-independent function to ensure consistent delivery when tilted

- Removable components to support effective cleaning and infection control

- Controlled aerosol output to minimise medication waste and ensure dose consistency

- CE marked

- Washable components

Website link: https://www.henleysmed.com/products/drug-nebulizers

## ELEC0036ELITE – Nebuliser with Mouthpiece

Features
Portable nebuliser system for respiratory therapy, includes mouthpiece.

Technical specification

- Dimensions: Compact unit

- User weight capacity: N/A

- Safe working load: N/A

- Control: On/off switch

- Mobility: Portable

- Materials: Plastic housing, tubing, mouthpiece

- Warranty: Manufacturer standard

Safety features

- CE marked

- Overheat protection

- Wipe clean

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## ELEC0036ELITEF – Nebuliser Filters for ELEC0036ELITE

Features
Pack of replacement filters for the ELEC0036ELITE nebuliser.

Technical specification

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual replacement

- Mobility: Portable

- Materials: Filter media

- Warranty: Manufacturer standard

Safety features

- CE marked

- Replace regularly

- Store in dry conditions

Legacy item – retained within the catalogue solely to support compatibility, recycling, or reissue. New supply is not required unless explicitly agreed.

## ELEC0036M – Mouth Piece and Chamber Kit for ELEC0036ELITE

Features
Jet nebuliser system designed for delivery of aerosolised medication in respiratory care. Suitable for use in clinical and home environments. The device converts liquid medication into a fine mist for inhalation and can be used in both upright and angled positions. Compatible with standard oxygen or air flow sources and can be used within ventilator circuits where required. Supplied as a kit with mouthpiece, reservoir tubing, and safety tubing.

Technical specification

- Application: Aerosol medication delivery

- Medication capacity: 3 ml (approx.)

- Flow rate: 7 L/min (typical operating flow)

- Nebulisation time: ≤ 7 minutes (for 3 ml at 7 L/min)

- Orientation: Operates in vertical and angled positions

- Kit contents:

  - Nebuliser chamber with tee

  - Mouthpiece

  - 150 mm (6″) reservoir tube

  - 2.1 m (7 ft) safety tubing

- Size: Adult / Paediatric use

- Pack quantity: 50 units

- Materials: Medical-grade plastic components

- Reuse: Single patient use or as per local infection control policy

- Warranty: Manufacturer standard

Safety features

- Anti-drool “T” design to reduce contamination from saliva

- Secure screw cap to minimise leakage during use

- Stable base design to allow upright positioning

- Compatible with standard respiratory circuits

- Designed for disassembly to allow cleaning in line with infection control procedures

- CE marked

- Washable components

Website link: https://www.henleysmed.com/products/drug-nebulizers

## ELEC0036PM – Paediatric Mask and Chamber Kit for ELEC0036ELITE

Features
Child-sized mask and chamber for use with the ELEC0036ELITE nebuliser.

Technical specification

- Dimensions: Paediatric size

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual use

- Mobility: Portable

- Materials: Medical-grade plastic

- Warranty: Manufacturer standard

Safety features

- CE marked

- Washable components

Website link: N/A

## 301. ELEC0037F – Inlet Filter

Features
Replacement inlet filter for compatible nebulisers or suction units.

Technical specification

- Dimensions: Standard fit

- Product weight: < 0.1 kg

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual replacement

- Mobility: Portable

- Materials: Filter media

- Warranty: Manufacturer standard

Safety features

- CE marked

- Replace regularly

- Store in dry conditions

Website link: InnoSpire Deluxe Air Inlet Filters (4 pack) HH1311/00 | Philips

## ELEC0037FHO – Expiratory Filter Holder with 10 Filters

Features
Filter holder with 10 disposable filters for use with compatible respiratory devices.

Technical specification

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual use

- Mobility: Portable

- Materials: Plastic housing, paper filters

- Warranty: Manufacturer standard

Safety features

- CE marked

- Replaceable filters

- Wipe clean

Website link: https://www.philips.co.uk/c-p/HH1350_00/innospire-deluxe-compressor-nebuliser-system

## ELEC0037FP – Expiratory Filter Pads – 50 Pack

Features
Pack of 50 replacement filter pads for use with compatible respiratory filter holders.

Technical specification

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual replacement

- Mobility: Portable

- Materials: Filter paper

- Warranty: Manufacturer standard

Safety features

- CE marked

- Disposable

- Store in dry conditions

Website link: https://www.philips.co.uk/c-p/HH1350_00/innospire-deluxe-compressor-nebuliser-system

## ELEC0038 – Filters for ELEC035 – Pack of 12

Features
Pack of 12 replacement filters for the ELEC035 suction unit. The DeVilbiss bacterial filter cartridge is a single-use component used in compatible suction units to reduce contamination of the suction system during removal of fluids. It is fitted inline within the suction circuit and is intended to support infection control during clinical suction procedures. The filter is used with compatible DeVilbiss suction devices in hospital, community, or home care settings where airway or secretion suction is required.

Technical specification

- Dimensions: Compatible with ELEC035

- Device type: inline bacterial filter cartridge

- Compatibility: DeVilbiss 7305 series suction units and 7314 QSU

- Function: filtration of airborne and fluid contaminants within suction system

- Configuration: single-use cartridge

- Weight: approximately 0.012 kg

- Control: Manual replacement

- Mobility: Portable

- Materials: Filter media

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean housing

- For use only with compatible suction equipment

- Must be replaced according to manufacturer or local infection control policy

- Do not reuse single-use components

- Do not use if packaging is damaged or filter integrity is compromised

- Ensure correct fitting to avoid leakage or loss of suction efficiency

- Dispose of in accordance with clinical waste procedures

- Use only in accordance with trained clinical practice guidelines for suction equipment

Website link: drivemedical.de/en/products/productdetail/20582

- ELEC0051 - HospiNeb Nebuliser System with Tubing and Mask

The HospiNeb Nebuliser System is a compressor-based device used to deliver liquid medication in aerosol form for inhalation. It is designed for use in clinical environments and home care settings where respiratory treatment is required.

The system includes a compressor unit, nebuliser chamber, tubing, and adult and child masks. It is intended for repeated use and supports continuous operation during treatment. The unit is fitted with an air filtration system to support consistent performance during use.

Safety Features:

- Continuous operation for extended treatments

- Built-in air filtration system

- Stable compressor housing for clinical use

- Designed to prevent overheating during use

- Enclosed storage for tubing and accessories

Technical Specification:

- Power supply: 230V ~ 50Hz

- Power input: 210VA

- Air flow: approx. 14 L/min

- Maximum pressure: 3.5 ± 0.5 bar

- Sound level: approx. 57 dB at 1 metre

- Dimensions: 200 x 180 x 110 mm

- Weight: 1.8 kg

- Operation mode: Continuous use

- Maximum fill volume: 8 ml

- Compliance: EU Medical Device Regulation (EU 2017/745)

Website Link: FLAEM HospiNeb - Henleys Medical Supplies

- ELEC0051F - Kit 2 Filters Porex Professional (Type-xs-9226) (Pack of 2)

Pack of 2 Filters for HospiNeb Nebuliser
Website link: https://www.henleysmed.com/products/flaem-hospineb

- ELEC0051KA - HosiNeb Kit RF7 Plus Adult
Flaem nebuliser patient pack includes:

- The RF7+ plus high efficiency nebuliser

- Soft touch mask Adult

- One metre tube

Website link: https://www.henleysmed.com/products/flaem-hospineb

## ELEC0052– Medix AC2000 Mains Nebuliser

Features

Medix AC2000 Nebuliser (Clement Clarke). Mains-powered compressor nebuliser for delivery of prescribed medications, including those used in chronic and acute respiratory conditions. Suitable for nebulising a range of solutions, including higher viscosity medications.

The unit has a continuously rated motor for sustained use. The casing includes integrated storage for accessories and mains cable. A large on/off rocker switch is fitted for ease of operation. The device requires annual servicing.

Technical specification

- Power supply: 220–240 V, 50 Hz

- Weight: 2.45 kg

- Dimensions: 37.5 × 23.5 × 11.5 cm

- Sound level: ≈60 dBA

- Particle size: 3.4 µm

- Nominal flow rate: 7–9 L/min at 138 kPa

- Suitability: Suitable for tracheostomy use

- Service interval: Annual service required

- Guarantee: 3 years

- EAN: 5030818050206

Website link: Medix AC2000 Nebuliser - Evergreen Nebulizers

## ELEC0052Y– Medix AC2000 Year Pack

Features

Pack containing consumables required to support maintenance and use of Medix nebuliser systems over a 12-month period. Includes 4 nebuliser kits for the Medix nebuliser system. Each kit contains a Lifecare chamber, air tube, and a choice of adult mask, child mask, or mouthpiece. Kits are intended for use for up to 3 months per set. The air tube is 180 cm in length and is manufactured from latex-free medical grade plastic. It connects the compressor to the medication chamber. The tube should not be immersed in water as it is difficult to fully dry.

The face mask is manufactured from medical grade, latex-free plastic and covers the nose and mouth. The mouthpiece is also latex-free.

The pack also contains 4 inlet filters and 1 outlet filter for the nebuliser unit. Filters are used to ensure air intake remains free from dust. Inlet filters should be replaced every 3 months or when discolouration occurs. Outlet filters should be replaced every 12 months. Filters cannot be cleaned and must be replaced.

Technical specification

Nebuliser Kits (x4)

- Components: Lifecare chamber, air tube, mask or mouthpiece

- Use duration: Up to 3 months per kit

- Intended use: Single patient use

Air Tube

- Length: 180 cm

- Material: Latex-free medical grade plastic

- Cleaning: Do not immerse in water

Mask / Mouthpiece

- Material: Medical grade, latex-free plastic

Filters

- Inlet filters: 4 included

- Outlet filter: 1 included

- Inlet filter replacement: every 3 months or when discoloured

- Outlet filter replacement: every 12 months

- Cleaning: Cannot be cleaned, must be replaced

Compatibility

- Medix AC2000

- Medix AC4000

- Medix Econoneb

- Medix Turboneb

- Medix World Traveler

- Medix Actineb

Website link: Medix Nebuliser Year Pack - Evergreen Nebulizers

## ELEC035– Vacuaide QSU Suction Unit

Features

Portable suction pump for oral, nasal, and tracheal airway secretion management. Available in battery-powered and mains-powered versions. Designed for intermittent suction with adjustable vacuum control and pressure display via integrated manometer.

The unit operates with on/off control and a rotary vacuum adjustment system providing continuous (infinitely variable) regulation of suction pressure. Vacuum level is displayed via the built-in gauge. The device supports both portable use and fixed clinical or home use. A carrying bag is included for transport and storage.

Technical specification

- Model: VacuAide 7314

- Use: Oral, nasal, and tracheal suction

- Vacuum range: 50–550 mmHg

- Air flow (max): 27 L/min

- Operation: Mains or battery powered

- Battery runtime: 60 minutes

- Power supply: 240 V, 60 Hz

- Controls: On/off switch with rotary vacuum adjustment

- Pressure display: Integrated manometer (vacuum indicator)

Dimensions

- Width: 20.3 cm

- Height: 21.1 cm

- Depth: 22 cm

Weight

- 3 kg (with accessories)

- 3 kg (without accessories)

Environmental Limits

- Operating temperature: up to 40°C

- Storage temperature: -40 to 70°C

- Relative humidity: up to 95%

Additional

- Includes transport/operating bag

- Reimbursement code: 01.24.02.1019

Website link: Drive DeVilbiss portable secretion suction pump VacuAide QSU 7314, 800ml container, with rechargeable battery and EU plug

## VACUAIDE1 – Replacement Kit of Parts for ELEC035

Features
Replacement parts kit for the ELEC035 suction unit, including tubing and connectors.

Technical specification

- User weight capacity: N/A

- Safe working load: N/A

- Control: Manual replacement

- Mobility: Portable

- Materials: Medical-grade plastic

- Warranty: Manufacturer standard

Safety features

- CE marked

- Replace regularly

- Wipe clean

Website link:

https://shop.drivemedical.com/us/en/products/respiratory/suction-therapy/suction-accessories/suction-therapy-accessories/p/7305D-633?allowedPageSizes=18andallowedPageSizes=36andallowedPageSizes=54andsiteName=Drive+Medical+US+SiteandsiteUid=usanditemsPerPage=18andsiteName=Drive+Medical+US+SiteandsiteUid=us

## VACUAIDE4 – Elbow Connection for ELEC035

Features
Replacement elbow connector for use with the ELEC035 suction unit. 90-degree elbow connector designed for use with DeVilbiss 7305 series suction systems. Provides directional connection between suction tubing components to support airway secretion removal procedures. Used in clinical suction setups where flexible routing of tubing is required. Disposable Container Filter Cartridge Assembly for 800ml.

Technical specification

- Dimensions: Standard fit for ELEC035

- Product weight: < 0.1 kg

- Product type: 90° elbow connector

- Pack size: 12 units

- Compatibility: DeVilbiss 7305 series suction units

- Application: Suction tubing connection component

- Control: Manual attachment

- Mobility: Static

- Materials: Medical-grade plastic

- Warranty: Manufacturer standard

Safety features

- CE marked

- Wipe clean or disinfect

- Ensure secure connection before use to prevent air leakage.

- Check integrity before each clinical procedure.

Website link: https://shop.drivemedical.com/us/en/products/respiratory/suction-therapy/suction-accessories/suction-therapy-accessories/p/7305D-619

Mobility

## KIT757– Braked trolley

Features

Domestic mobility and support trolley for carrying items and assisting with seated transfers within the home. Features clip-on plastic trays with raised edges to reduce item displacement. Attendant brakes allow the trolley to be stabilised during use, including when standing or sitting. The handle height is adjustable to suit user requirements. The lower tray is positioned forward to support safe walking posture. The upper tray is positioned for use while seated. Four castors support movement across indoor surfaces. Supplied flat pack with tool-free assembly and disassembly for storage or transport.

Technical specification

- Overall height: 81–91 cm

- Width: 60 cm

- Depth: 53 cm

- Product weight: 7.6 kg

Trays

- Lower tray height: 43 cm

- Upper tray height: 68 cm

- Tray depth: 32 cm

- Tray width: 46 cm

- Material: Clip-on plastic trays with raised edges

Mobility

- Castors: 4 large wheels (15 cm / 5.9")

- Brakes: Attendant hand brakes

Load Capacity

- Safe working load: 15 kg

- Maximum user weight: 125 kg (20 st)

Adjustment / Assembly

- Handle height: Adjustable

- Assembly: Tool-free flat pack design

Warranty

- 1 year

Website link: Handy Trolley

## MOB081– Medium Wheeled Trough Frame

Features

Walking frames with forearm platforms are intended for users who bear weight through their forearms rather than their hands during mobility. The forearm platforms provide upper limb support, and the padded armrests assist with distributing weight between the arms and legs while walking.

The frame includes integrated forearm platforms and padded armrests. It has a turning width of 81 cm.

Technical specification

- Height: 92–99 cm (support), 103–110 cm (maximum height)

- Width: 60 cm

- Depth: 54 cm

- Product weight: 2.5 kg

- User weight capacity: 160 kg

- Warranty: 1 year

Website link: Domestic Wheeled Frame with Forearm Platform

## MOB098A– Big Foot Half Step - Version 2

Features

The Bigfoot Half Step is designed to reduce the height difference between a doorway and ground level, providing an intermediate step to support safer access. The wide platform allows space for both feet, supporting stability before stepping up or down.

The step is suitable for outdoor use and is constructed from plastic materials that are resistant to rust and temperature changes. The surface includes fixed grips to support slip resistance.

The height is adjustable from 9–11 cm or 14–16 cm with the supplied height spacer, allowing the step height to be adapted to the environment.

Optional support handles can be attached without permanent installation. One or two handles can be clipped into the step and removed as required. A floor fixing kit is included for use with the support handles.

Technical specification

- Width: 750 mm

- Depth: 440 mm

- Height: 90–110 mm or 140–160 mm (with spacer)

- Product weight: 5.5 kg

- Shipping weight: 6.1 kg

- Maximum user weight: 220 kg (35 stone / 485 lbs)

Website link: Bigfoot Half Step | Langham

## MOB228 – 4 Wheeled Walker – Large

Features
Alternative 4-wheeled walker with ergonomic handles, seat, and shopping bag.

Technical specification

- Dimensions: Adjustable handle height 875-995mm

- Seat Height 620mm

- User weight capacity: 165KG

- Control: Manual brakes

- Mobility: 4 wheels with locking brakes

- Materials: Aluminium frame, fabric bag

- Warranty: Manufacturer standard

Safety features

- CE marked

- Locking brakes

- Wipe clean

Website link: https://www.performancehealth.co.uk/days-100-series-lightweight-rollators

## MOB259B - Drive - Height Adjustable 4 Wheeled Walker

A walking aid designed to support balance and stability during walking. It has a seat for resting and a frame that can be folded for storage or transport. The handles and seat height can be adjusted to suit the user.

It is intended for use on flat, stable surfaces indoors and outdoors. A storage bag is fitted under the seat for carrying small personal items

Technical specification

- Model: R8BLHA

- Weight: 7 kg

- Maximum user weight: 135 kg

- Handle height: 79–99 cm

- Seat height: 48–58 cm

- Width: 64 cm

- Depth: 69–75 cm

- Seat size: 34 cm wide × 31 cm deep

- Wheels: 19 cm solid wheels

- Colour: Blue

- Folds for storage

Safety information

- Use on flat, even ground

- Ensure brakes are applied before sitting

- Do not exceed weight limit

- Check height adjustments are locked before use

- Do not use on steep slopes or uneven ground

- Check regularly for wear or damage

Website Link: Adjustable Seat Height Rollator

## MOB293 - TA180 Transfer Aid v2.

A manual transfer aid used to support assisted standing and short-distance transfers between seating, bed, and toilet. It is designed for individuals who can bear some weight but require support due to reduced strength or balance. The device has a frame with padded support areas and a foot tray. The legs can be opened to allow positioning around chairs or other furniture. The rear support pads help maintain stability once the user is in a standing position.

Technical specification

- Safe working load: 185 kg

- Device weight: approx. 38–40 kg

- Overall height: 1,001–1,021 mm

- Internal width: 440–518 mm (varies by configuration)

- External width: 680–1,048 mm (legs open)

- Ground clearance: 78 mm

- Under-bed clearance: 128 mm

- Turning circle: 1,010 mm

- Castor size: 100 mm

- Frame material: Powder-coated steel

- Padding: PU foam with vinyl covering

- Foot tray: Removable

Safety information

- For assisted use only; not intended for independent transfer without support

- Ensure the user can weight-bear as assessed by a healthcare professional

- Do not exceed maximum weight limit

- Ensure brakes are applied during transfer and positioning

- Check all pads and fixings are secure before each use

- Use only on flat, stable surfaces

- Regular inspection required for wear, stability, and wheel function

- CE marked and complies with EU MDR 2017/745

Website Link: Freeway TA180

## MOB532D - GBUK Prime Curved Transfer Board

A seated transfer board used to assist with transfers. It is suitable for patients with upper strength and supports independent movement.

Technical Specification

- Type: Seated transfer board

- Model: BB-Prime

- Dimensions: 26 cm × 62 cm

- Safe working load: 200 kg

- Latex free: Yes

- Reusable: Yes

- Cleaning: Easy to wipe clean

- Construction: Hard-wearing board

- Weight: Lightweight

- Shape: Curved

- Edges: Tapered

- Labelling: GS1 compliant patient label

Safety Information

- Suitable for patients with upper strength

- Do not exceed safe working load (200 kg)

- Ensure correct placement before transfer

- Do not use if damaged

Website Link: Seated Transfer Board - GBUK

## MOB565SM – Coopers Three Wheel Walker with Bag

Features
Tri-wheeled walker for indoor and outdoor use, includes storage bag.

Technical specification

- Adjustable handle height 800-927mm

- User weight capacity: Up to 130 kg

- Control: Manual brakes

- Mobility: 3 wheels with locking brakes

- Materials: Aluminium frame, vinyl bag

- Warranty: Manufacturer standard

Safety features

- CE marked

- Locking brakes

- Wipe clean

Website link: https://www.sunrisemedical.co.uk/mobility-aids/coopers/mobility-aids/three-wheel-walker-with-bag

## MOB582 - Walking Frame c/w Wheels 35"- 38"

A walking frame used to help support standing and walking. It is used in hospitals and care settings. The frame is made from lightweight metal and has a wide base for stability. Wheels help it move more easily.

Technical specification

- Product code: 7335C

- Maximum user weight: 160 kg

- Handle height: 885–965 mm

- Width: 660 mm

- Depth: 585 mm

- Weight: 2.0 kg

- Material: Aluminium

- Wheels: Yes (front)

- Base: Wide

- Ferrule: Z25

Safety information

- Use only on flat, even ground

- Do not exceed weight limit

- Check condition before each use

- Make sure height is set correctly before use

- Do not use on stairs or uneven surfaces

Website Link: COOPERS Hospital Walking Frame | Sunrise Medical

## MOB613 - Sliding Sheet Large (20)

A tubular roller slide sheet designed to assist with repositioning individuals in bed or on a trolley. It reduces friction to support lateral movement, turning, and repositioning during manual handling.

Technical Specification

- Type: Tubular roller slide sheet

- Material: Low-friction synthetic fabric (Slideezi®)

- Construction: Double-layer tubular design

- Dimensions: 100 cm × 120 cm

- Weight capacity: Suitable for all patient weights (subject to risk assessment)

- Reusable: Yes

- Washing: 40°C to 74°C

- Drying: Low-temperature tumble dry

- Sizes available: Multiple size options

Safety Information

- For use by trained personnel following manual handling procedures

- Carry out a risk assessment prior to use

- Do not use if the material is damaged or worn

- Do not use bleach or harsh cleaning agents

- Ensure correct placement to minimise risk of skin shear or discomfort

- Not intended for lifting; repositioning use only

Website Link: Tubular Roller Slide Sheets – Washable Slideezi® | Hospital Direct

## MOB643 - Walking Stick - Bariatric

A walking stick used to support balance when walking and standing. It is made from metal and has a curved handle. It can be adjusted in height and used in either hand.

Technical specification

- Model: 2507

- Height: 73–96 cm

- Maximum user weight: 318 kg

- Material: Steel

- Tip: Rubber, non-slip

Website Link: 2507 : Heavy Duty Adjustable Walking Stick - Roma Medical

## MOB646 – Elbow Crutch – Bariatric – Pair

Features
Heavy duty elbow crutches constructed from durable steel with a chromium plated finish for strength and longevity. The crutches are height adjustable to suit a range of users, with flexibility to adjust both the handle to ground height and the armband cuff position. Designed for stability and safety, they include a non-slip rubber tip and an ergonomic cuff with an internal diameter of 14 cm for secure support and user comfort.

Technical specification

- Product weight: 1.3 kg

- Handle to armband cuff: 14 – 24 cm

- Handle to ground height: 66 – 91 cm

- User weight capacity: Up to 318 kg

- Safe working load: 318 kg

- Control: Manual adjustment

- Mobility: Walking aid (mobile)

- Materials: Heavy duty steel with chromium plated finish

Safety features

- Non-slip rubber tip for grip and stability

- Adjustable height for safe positioning

- Ergonomic cuff support for user control

- CE marked

Website link: https://romamedical.co.uk/product/2125-heavy-duty-elbow-crutches/

## MOB647 - XL Folding Walking Frame - Bariatric (Drive)

The Bariatric Folding frame is a lightweight walking frame that is suitable for indoor or outdoor use. The legs have a ferrule tip, providing grip for comfort and security.
Specification
Height 82-100cm

Width 62cm

Depth	52.5cm

User Weight Capacity 228kg

Website link: https://drivemedical.co.uk/community/grand-xl-bariatric-walker

## MOB660HD - Wheels for Drive Bariatric Frame (MOB647)

A wheel conversion kit designed for use with compatible heavy-duty walkers. The set includes 5" dual rubber front wheels and rear glide caps with covers, allowing the walker to move more easily across indoor and outdoor surfaces. The wheels provide smoother movement over uneven ground, while the rear glides support controlled sliding contact with the floor. The height-adjustable design allows the walker to be set to the user’s requirements. Intended for use with compatible walker models to assist with mobility and reduce the need to lift the frame during use.

Technical specification

- Wheel size: 5" (12.7 cm)

- Wheel type: Dual rubber wheels

- Height adjustment: 8 positions

- Additional height (variant dependent): up to 4"

- Includes: Rear glide caps and glide covers

- Compatibility: Heavy-duty walkers (e.g. 10220-2, 10223-2)

- Colour: Silver vein frame with grey wheels

Website Link: Product Specifications

## MOB1503 – Door Line Multi Ramp

Features
The Doorline Multi (DM5) is a threshold bridging component designed to facilitate safe and stable passage over uPVC and similar door thresholds when used in conjunction with compatible portable ramps. It is intended to reduce the risk of impact or damage to both the threshold surface and mobility equipment by providing a controlled transition surface. The unit supports wheelchair and mobility aid users by improving alignment between the ramp and doorway, enabling consistent access across uneven or raised entry points.

Technical Specification (DM5)

- Maximum recommended height: 10 cm (3.9 in)

- Total length: 56 cm (22 in)

- Width: 70 cm (27.6 in)

- Maximum load capacity: 350 kg (55 stone)

- Unit weight: 3.7 kg (8.2 lbs)

- Mobility: Mobile

Safety Features

- Load-rated to 350 kg to accommodate wheelchairs, occupants, and attendants within specified limits

- Provides a gradual transition to reduce tipping risk at threshold edges

- Designed to minimise direct contact with fragile uPVC thresholds, reducing potential structural damage

- Wide surface area to support lateral stability during use

- Lightweight construction to allow correct positioning and repositioning without excessive manual handling strain

- Intended for use with compatible ramp systems to maintain alignment and safe gradient control

- CE marked

- Wipe clean

Website link: Doorline Multi - Enable Access

## MOB1615A – 2ft Metro Folding Suitcase Wheelchair Ramp

Features
Portable folding ramp for wheelchair access over small thresholds and steps.

Technical specification

- Dimensions: 2 ft length

- Product weight: Approx. 5 kg

- User weight capacity: Up to 300 kg

- Safe working load: 300 kg

- Control: Manual deployment

- Mobility: Portable

- Materials: Aluminium with anti-slip surface

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.enable-access.com/product/wheelchair-ramp-the-new-metro-portable-folding-aluminium-ramp/?attribute_length=60cm+%282%27%29

## MOB1615C – 4ft Metro Folding Suitcase Wheelchair Ramp

Features 
Portable folding ramp for wheelchair access over steps and thresholds.

Technical specification

- Dimensions: 4 ft length

- Product weight: 9.1 kg

- User weight capacity: Up to 300 kg

- Safe working load: 300 kg

- Control: Manual deployment

- Mobility: Portable

- Materials: Aluminium with anti-slip surface

- Warranty: Manufacturer standard

Safety features

- Safety standards and compliance: CE marked

- Cleaning instructions: Wipe clean

Website link: https://www.enable-access.com/product/wheelchair-ramp-the-new-metro-portable-folding-aluminium-ramp/?attribute_length=60cm+%282%27%29

## MOB7677 - Heavy duty 4 wheeled walker

A four-wheeled rolling walker designed to assist individuals with reduced mobility. The wider frame provides increased support and stability.

The walker includes a padded seat and backrest for resting, along with a storage basket for carrying personal items. It is fitted with 8" wheels suitable for indoor and outdoor use.

Handle height can be adjusted to suit the user. Loop-operated brakes are fitted to control movement and secure the walker when stationary. The frame can be folded for storage and transport.

Technical specification

- Maximum user weight: 180 kg

- Handle height: 890–990 mm

- Seat height: 560 mm

- Overall width: 735 mm

- Width between handles: 545 mm

- Seat dimensions: 460 mm (W) x 350 mm (D)

- Depth (folded): 500 mm

- Wheel size: 8"

- Product weight: 11.85 kg

- Colour: Gun metal grey

Safety information

- Do not exceed 180 kg maximum user weight

- Use on level surfaces

- Ensure brakes are applied before sitting

Website Link: COOPERS Heavy Duty Rolling Walker | Sunrise Medical

## MOB7771 - Trolley walker (with breaks)

A wheeled household trolley designed to assist with the transport of meals, drinks, and small items between rooms. The height of the handle can be adjusted to suit the user. The trolley is fitted with two removable trays for carrying items, which can be detached for cleaning. Handle-mounted brakes are included to help control movement and secure the trolley when stationary. The castors are designed for indoor use.

Technical specification

- Product code: VG798WB

- Overall dimensions: 840 x 530 x 530 mm0

- Handle height: 840–1020 mm

- Footprint: 530 x 530 mm

- Tray size: 463 x 345 mm

- Maximum user weight: 135 kg

- Maximum tray load: 15 kg

- Net weight: 8.5 kg

- Colour: Brown and cream

Safety information

- Do not exceed 135 kg user weight

- Do not exceed 15 kg per tray

- Use on level indoor surfaces

- Ensure brakes are applied when stationary

Website Link: Height Adjustable Kitchen Strolley Trolley with Brakes | Aidapt

## MOB7772 – 4 Wheeled Walker – Option 1

Features
The R8 aluminium lightweight rollator is designed to provide stability, comfort, and ease of movement for both indoor and outdoor use. It features a rustproof yet lightweight aluminium frame, making it durable while remaining easy to manoeuvre and transport. The rollator includes height-adjustable handles with an anatomical design for improved grip and user comfort. Looped cable brakes are easy to operate and can be locked for added safety. A curved, padded backrest and flip-up padded seat enhance comfort, while swivelling front wheels and puncture-proof tyres ensure smooth movement across a variety of surfaces. The rollator also folds easily for storage and transport and includes wipe-clean surfaces for hygiene.

Technical specification

- Overall height: 82 – 98 cm (approx.)

- Width: 64 cm

- Depth: 62 cm

- Seat height: 57 cm

- Seat depth: 31 cm

- Seat width: 36 cm

- Product weight: 7 kg

- User weight capacity: Up to 135 kg (21 stone)

- Safe working load: 135 kg

- Control: Manual (handle height adjustment and hand brakes)

- Mobility: Mobile walking aid (indoor and outdoor use)

- Materials: Aluminium frame

- Tyres: 18 cm (7") puncture-proof

- Colour: Red

Safety features

- Looped, lockable cable brakes for secure stopping

- Puncture-proof tyres for reliability across surfaces

- Stable aluminium frame for user support

- Ergonomic handles for controlled grip

- Easy folding mechanism for safe storage and transport

- CE marked

Website link: https://drivemedical.co.uk/care-home/walking-aids/rollators/aluminium-lightweight-rollator-outdoor

## MOB1048A - Heavy Duty Rollator

A heavy duty rollator designed to assist with walking and provide support during movement. It can be used indoors and outdoors and includes a padded seat and backrest for seated rest. The rollator also features a storage basket and a foldable frame for storage and transport.

Technical Specification

- Type: Rollator

- Brakes: Looped cable brakes

- Handle height: 88–100 cm (adjustable)

- Wheels: 20 cm solid puncture-proof tyres

- Seat: Padded, flip-up

- Backrest: Padded

- Storage: Basket included

- Frame: Foldable

- Use: Indoor and outdoor

- Colour options: Blue

Safety Information

- Ensure brakes are applied when stationary

- Adjust handle height before use

- Check wheels and brakes before use

- Do not use if damaged or unstable

Website Link: Heavy Duty Rollator

## MOB1320G - Aluminium 3 Wheel Walker with Bag and Basket - Graphite (16)

A lightweight tri-wheel walking frame designed to support mobility for users with reduced balance or strength. It is suitable for indoor and outdoor use and provides assistance during walking.

Technical Specification

- Product: Days Aluminium Lightweight Tri-Wheel Walker

- Maximum user weight: 125 kg

- Product weight: 7.7 kg

- Overall width: 68.5 cm

- Overall depth: 61 cm

- Handle height: 83–93 cm (adjustable)

- Wheel diameter: 19 cm

- Frame material: Aluminium

- Brakes: Loop lockable brakes

- Height settings: 5 positions (25 mm intervals)

- Storage: Foldable design

- Accessories: Available with or without basket and tray

Safety Information

- Do not exceed maximum user weight (125 kg)

- Ensure brakes are engaged when stationary

- Check frame, wheels, and brakes before use

- Ensure height is correctly adjusted before use

- Do not use if any part is damaged or unstable

Website Link: Days Lightweight Aluminium Tri Wheel Walker | Performance Health®

# Retail

A proportion of community equipment is currently supplied under a retail/prescription model, where practitioners specify an assessed functional need rather than a prescribed make or model. Under this arrangement, local retailers are responsible for selecting and supplying items they consider clinically appropriate and fit for purpose within broad product categories, such as bath boards, raised toilet seats and walking aids

As a result of this approach, detailed, Council approved technical specifications were not historically developed for individual retail items. Variation in product make, model and design was therefore an accepted feature of the retail model, provided that equipment met basic safety requirements and fulfilled the intended functional outcome.

It is intended that the current retail model will end on 31st December 2026. From that date, all items previously supplied through the retail model will be included within the scope of this contract and should be priced and delivered in accordance with the arrangements set out in the ITT and Costing Schedule. For the purposes of this procurement, retail items (referred to within the Costing Schedule as Ex-Retail equipment) are included to ensure pricing continuity, service volume modelling and transparency. To reflect historic practice while improving consistency and quality assurance, retail equipment is specified within this document at a category level, rather than as individual named products.

The category-level specifications set out the minimum technical, safety and functional requirements that all retail items supplied under the contract must meet or exceed.

Former retail items may be supplied as Close Technical Equivalents, provided they meet the minimum requirements set out in the relevant retail category specification as detailed below:

## Bath Boards and Bath Seats

Applicable Product Codes: BA01 – BA17

Equipment description

Bath boards and bath seats designed to support adults with reduced mobility to safely transfer into and use a standard domestic bath.

Intended use

To provide stable seated support across or within a bath for individuals unable to stand for prolonged periods or step fully into a bath.

Core technical requirements

- Manufactured from water-resistant, easy-clean materials

- Non-slip upper surface

- Adjustable fixing brackets suitable for standard bath widths

- Rounded edges and smooth surfaces to reduce injury risk

- Drainage apertures to prevent water pooling

Safe working load

- Minimum SWL: 150 kg (clearly marked)

Dimensions and adjustability

- Width and fixing mechanisms must allow secure fitting to standard UK domestic baths

- Height adjustable where applicable

Standards and compliance

- Must comply with relevant UKCA/CE requirements

- Designed in line with MHRA guidance for bathing aids

Installation and removal

- Must be installable and removable without permanent alteration to the bath

- No tools required or basic hand tools only

Training and use

- Simple user instructions to be provided

- Suitable for independent or assisted use following basic instruction

## Raised Toilet Seats and Toilet Frames

Applicable Product Codes: TA01 – TA16C

Equipment Description

Raised toilet seats and surrounding frames to increase seat height and/or provide arm support for toilet use.

Intended use

To support safe toileting for individuals with reduced hip, knee or lower limb mobility.

Core technical requirements

- Stable, non-slip base or secure toilet pan fixing

- Smooth, wipe-clean surfaces

- Armrests where supplied must provide firm support

Height options

- A range of height increases to meet assessed need

- Secure fixing mechanism appropriate to toilet type

Safe working load

- Minimum SWL: 150 kg

Standards and compliance

- UKCA/CE marked

- Suitable for use in wet environments

Installation

- Easy installation without structural alteration

- Clear fitting instructions supplied

## Walking Aids (Frames and Sticks)

Applicable Product Codes: MO13 – MO32

Equipment description

Mobility aids including walking sticks and walking frames.

Intended use

To provide balance and mobility support for ambulant adults.

Core technical requirements

- Lightweight but robust construction

- Height adjustable to suit individual user

- Non-slip ferrules or tips fitted

Standards and compliance

- UKCA/CE marked

- Designed in accordance with relevant walking aid standards

User safety

- Clearly labelled with height adjustment indicators

- Suitable for indoor and/or outdoor use as appropriate

## Hip Replacement Kit

Applicable Product Code: PL27

Equipment description

A pre-assembled kit of simple daily living aids supplied to individuals following hip replacement surgery, designed to support independence, reduce the need for bending, and promote safe adherence to post-operative precautions.

Intended use

To assist individuals recovering from hip replacement surgery to carry out essential daily activities safely, while minimising hip flexion and reducing the risk of dislocation or strain during the early recovery period.

Kit contents

Each Hip Replacement Kit must contain:

- Sock aid – to support independent donning of socks without bending

- Grabber / reacher – to assist retrieval of items from floor level or overhead

- Long handled shoehorn – to allow footwear to be put on safely without excessive hip flexion

Items must be suitable for adult use and provided as a single issued kit.

Core technical requirements

- Lightweight but durable construction

- Smooth, easy clean surfaces suitable for domestic use

- Components designed to be used one handed where possible

- No sharp edges or protrusions that could cause injury

- Clearly identifiable handles and grips

Materials

- Plastic, metal or composite materials suitable for repeated domestic use

- Latex free construction

- Easy clean and moisture-resistant finishes

Dimensions and ergonomics

- Long handled items must provide sufficient reach to avoid the need for bending below hip height

- Grips designed to support individuals with reduced dexterity or grip strength

Standards and compliance

- UKCA or CE marked as applicable

- Designed in line with general principles of assistive daily living equipment

- Suitable for use following clinical instruction or written guidance

Infection control and hygiene

- Supplied new at point of issue

- Suitable for routine domestic cleaning

- Materials compatible with mild household detergents

Training and instructions

- Clear written instructions to be provided for each item or for the kit as a whole

- Suitable for independent use following basic explanation by a practitioner